| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0097-v1 |
| --- | --- |

| Title: | EE2-3.5: On POC conditions of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ning Yan, Xiaoyu Xiu, Wei Chen, Xianglin Wang | Email: | yanning03@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-3.5, which proposes to apply the BDOF to the bi-prediction CUs where both reference pictures come from the same direction to the current picture.

The simulation results on top of ECM-14.0 are summarized as follows:

RA: { -xxx% Y, -xxx% U, -xxx% V, xxx% EncT, xxx% DecT }

LDB: { -0.11% Y, -0.04% U, -0.41% V, 100.5% EncT, 101.0% DecT }

Introduction

BDOF for no “true” bi-prediction CUs were proposed in JVET-AI0198 [1], and included in EE2 [2] as Test 3.5 for further investigation.

In Test 3.5, it is proposed to apply the BDOF to the bi-prediction CUs where both reference pictures come from the same direction to the current picture. More specifically, the existing BDOF design in ECM-14.0 is kept exactly unchanged except that the following condition is introduced to additionally enable the BDOF for the CUs:

- The bi-prediction CUs having two different reference pictures (i.e., different POCs) that are either before or after the current picture in display order.

Test Result

The proposed scheme was implemented on top EMC-14.0 and tested following the CTC [3]. The simulation results are summarized in the following table.

Table 1: Performance of EE2-3.5

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.02% | -0.07% | -0.01% | 100.4% | 100.6% | 100.0% | 100.0% |
| Class C | -0.01% | -0.01% | 0.01% | 99.8% | 99.6% | 99.6% | 99.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.01% | 0.12% | -0.02% | 99.3% | 99.3% | 99.9% | 99.9% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.06% | -0.30% | 100.8% | 100.9% | 100.0% | 100.0% |
| Class C | -0.06% | -0.06% | -0.38% | 99.6% | 101.1% | 99.1% | 98.8% |
| Class E | -0.26% | 0.03% | -0.65% | 101.2% | 100.9% | 99.9% | 99.9% |
| Overall | -0.11% | -0.04% | -0.41% | 100.5% | 101.0% | 99.7% | 99.6% |
| Class D | -0.06% | 0.77% | -0.13% | 99.3% | 100.6% | 99.9% | 99.9% |

Conclusion

In this contribution, the results of EE2 Test 3.5 on POC conditions for BDOF are reported. It is recommended to adopt the proposed method into the ECM.

Reference

- N. Yan, X. Xiu, W. Chen, X.Wang, “Non-EE2: On POC conditions of BDOF,” JVET-AI0198, July 2024.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AI2024, July 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, July 2024.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).