| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0099-v1 |
| --- | --- |

| Title: | EE1-3.1: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qipu Qin Cheolkon Jung | Email: | kippqin@163.com zhengzk@xidian.edu.cn |
| Source: | Xidian University |  |  |

_____________________________

# Abstract

JVET-AI0089 [1] proposed RA/LDB unified reference frame synthesis for VVC inter coding enhancement, named URFS. In this contribution, VTM-11.0_NNVC-9.0 (NN-tools on) is first used to generate a new training dataset. Then, URFS is retrained based on the training strategy proposed in JVET-AI0089 to obtain new RA and LDB models for tests. URFS is integrated in the latest VTM-11.0_nnvc-10.0 anchor. Compared with VTM-11.0_NNVC-10.0 (NN-tools on), the BD-rate performance of retrained URFS under RA and LDB configurations is as follows:

RA: {3.48%, 4.38%, 4.30%} BD-rate reductions for {Y, U, V} components.

LDB: {3.60%, 3.94%, 0.31%} BD-rate reductions for {Y, U, V} components.

- Introduction

In JVET-AC0114 [2] and JVET-AD0160 [3], a deep reference frame (DRF) generation method has been proposed to enhance the performance of VVC inter coding. JVET-AI0089 [1] proposed a unified reference frame synthesis for RA and LDB configurations in VVC using image-feature pyramids and progressive-recursive flow estimation, named URFS. For RA and LDB configurations, URFS adopts the same network architecture that is optimized end-to-end. URFS takes two previously decoded frames from the decoded picture buffer (DPB) as input and learns to interpolate (i.e., RA configuration) or extrapolate (i.e., LDB configuration) a reference frame. The reference frame is inserted into the reference pictures list (RPL) for the subsequent inter prediction. For training, an effective training strategy based on QP distance is introduced to maximize the performance of URFS.

In this contribution, the following three tests are performed and reported:

Test 3.1.1: Trained on Vimeo-90K triplet (compressed data by VTM-11.0-NNVC-4.0).

Test 3.1.2: Trained on Vimeo-90K triplet (compressed data by VTM-11.0-NNVC-9.0).

Test 3.1.3: Trained on BVI-DVC dataset (compressed data by VTM-11.0-NNVC-9.0); This test is not complete due to the time limit. To train URFS on BVI-DVC dataset, it is currently in the process of generating training data.

Compared with JVET-AI0089 [1], major changes in this contribution are summarized as:

- In JVET-AI0089 [1], VTM-11.0_NNVC-2.0 (NN-tools off) is used to generate a training dataset. However, this contribution utilizes VTM-11.0_NNVC-9.0 (NN-tools on) to generate a new training dataset.

- In JVET-AI0089 [1], URFS is integrated in an outdated version VTM-11.0_NNVC-4.0 (NN-tools off) anchor for tests. However, this contribution embeds the URFS model retrained on the new dataset into the latest version of VTM-11.0_NNVC-10.0 (NN-tools on) for tests according to CTC.

- Experimental Results

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 4090ti (1x24GB) |
|  | Framework | PyTorch v2.0.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch | 125 + 125 + 155 |
|  | Batch size | 10 |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 65h + 65h + 80h |
|  | Training data information | Vimeo90K-triplets (compressed by VTM-11.0_NNVC-9.0 (NN-tools on)) |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | Small QP: 7, 12, 17, 22, 27 Middle QP: 12, 17, 22, 27, 32 Large QP: 22, 27, 32, 37, 42 |
| Optional | Number of iterations | 385 x 7320iterations/epoch |
|  | Patch size | 256 x 256 |
|  | Learning rate | Form 1e-4 to 1e-5 |
|  | Learning rate update strategy |  |
|  | Optimizer | ADAMW |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | CPU only |
|  | Framework | PyTorch v2.0.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | RA: 1.9M LDB: 1.9M |
|  | Total Number of Parameters (All Models) | 2 x 1.9M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | RA: 7.9M LDB: 7.9M |
|  | Multiply Accumulate (kMAC/pixel) | RA: 727K LDB: 727K |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size | 1 |
|  | Patch size | 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information |  |

- Experimental results

Each sequence was test based on VTM-11.0_NNVC-10.0 (NN-tools on) according to the common test conditions (CTCs) defined in [4]. The evaluation was preformed in the RA and LDB configurations, respectively.

  - Results of Test 3.1.1

Table 3 BD-rate performance of the proposed URFS over VTM-11.0_NNVC-4.0 (NN-tools off)

under RA configuration

| Class | BD-rate Over VTM-11.0_nnvc-4.0 (Random access Main10) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | -3.93% | -9.65% | -9.66% | -4.44% | -11.62% | -11.07% | 3% |
| Class A2 | -5.77% | -13.61% | -8.84% | -6.42% | -14.56% | -9.38% | 3% |
| Class B | -4.47% | -11.17% | -10.94% | -5.04% | -12.70% | -13.23% | 3% |
| Class C | -6.77% | -14.39% | -14.45% | -7.99% | -14.85% | -16.08% | 5% |
| Class E | - | - | - | - | - | - | - |
| Overall | -5.24% | -12.21% | -11.20% | -5.98% | -13.43% | -12.79% | 4% |
| Class D | -8.91% | -14.67% | -15.77% | -7.97% | -15.76% | -15.69% | 6% |

Table 4 BD-rate performance of the proposed URFS over VTM-11.0_NNVC-4.0 (NN-tools off)

under LDB configuration

| Class | BD-rate Over VTM-11.0_nnvc-4.0 (Low delay B Main10) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | bit DIFF |
| Class A1 | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - |
| Class B | -5.33% | -12.54% | -8.27% | -5.89% | -13.92% | -18.97% | 5% |
| Class C | -6.68% | -19.00% | -15.51% | -8.07% | -19.86% | -17.54% | 6% |
| Class E | -6.30% | -11.99% | -7.02% | -6.89% | -11.96% | -13.14% | 4% |
| Overall | -6.02% | -14.55% | -10.37% | -6.87% | -15.41% | -17.04% | 5% |
| Class D | -7.07% | -12.72% | -9.45% | -7.19% | -7.31% | -6.37% | 6% |

  - Results of Test 3.1.2

Table 5 BD-rate performance of the proposed URFS over VTM-11.0_NNVC-10.0 (NN-tools on)

under RA configuration

Table 6 BD-rate performance of the proposed URFS over VTM-11.0_NNVC-10.0 (NN-tools on)

under LDB configuration

In Table 6, the BD-rate of the V channel in Classes B and E appears to be overfitted. It should be noted that the test models in Tables 5 and 6 are trained on the training datasets generated by VTM-11.0_NNVC-9.0 (NN-Tools ON), instead of VTM-11.0_NNVC-2.0 (NN-Tools OFF). The reasons for overfitting are as follows:

- Since VTM-11.0_NNVC-9.0 introduces NN-Tools by default, NN-Intra and LOP-3 filter also contribute to the BD-rate gains and significantly affects the chroma channels (U and V) rather than the luma channel (Y). Thus, the BD-rate gains in the U and V channels are relatively lower than in the Y channel, especially for the V channel.

- Unlike the RD configuration that performs video frame interpolation, the LDB configuration performs video frame extrapolation to generate a new reference frame. Thus, the LDB configuration meets lots of unpredictable cases caused by video frame extrapolation, and leads to relatively lower BD-rate gains than the RA configuration.

- Conclusions

This contribution mainly reports the results of two experiments. Test 3.1.1 uses the VTM-11.0_NNVC-2.0 (NN-tools off) to generate a dataset to train the URFS models (i.e., RA and LDB) and integrates them on VTM-11.0_NNVC-4.0 (NN-tools off) for tests. Test 3.1.2 uses the VTM-11.0_NNVC-9.0 (NN-tools on) to generate a dataset to retrain the URFS models (i.e., RA and LDB configurations) and integrates them on VTM-11.0_NNVC-10.0 (NN-tools on) for tests. Test 3.1.3 on BVI-DVC dataset is not complete due to time limit, which will be reported soon.

Following the cycle of JVET EE1, we will try to adopt the SADL tool to implement URFS.

- References

[1] Q. Qin and C. Jung. AHG11: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding. JVET-AI0089, 25th Meeting, Sapporo, JP, 12–19 July 2024.

[2] J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AC0114, 29th Meeting, by teleconference, 11–20 January 2023.

[3] J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AD0160, 30th Meeting, Antalya, TR, 21–28 April 2023.

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11–20 January 2023.

- Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).