| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0101-v1 |
| --- | --- |

| Title: | AHG9: Miscellaneous modifications for SEI messages in the TuC for future extensions of VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Tan Junghak Nam Jangwon Lee Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes miscellaneous modifications that are asserted to provide improvement to some aspects of SEI messages in the TuC for future extensions of VSEI. The proposed modifications are as follows:

(Related to image format metadata SEI message)

- Add signalling for byte-alignment before the syntax element ifm_data_payload_byte[ i ][ j ]

(Related to generative face video SEI message)

- Modify constraint on gfv_cnt and gfv_nn_present_flag

(Related to bitdepth range information SEI message)

- Define constraint on bri_nuh_layer_id_present_flag and bri_pic_partition_flag

(Related to display overlay information SEI message)

- Change doi_num_display_overlays_minus2 to doi_num_display_overlays_minus1

- Modify the condition to signal the presence of offset parameters and resampling information

Add signalling for byte-alignment before the syntax element ifm_data_payload_byte[ i ][ j ].

Problem statement

The current design of image format metadata (IFM) SEI message can specify syntax and semantics for image information according to image format type which is signalled with ifm_type_id. The image information payload is included in the IFM SEI message. However, it misses the signalling of byte-alignment to ensure that each payload starts at a byte-aligned position.

Proposed modifications

It is proposed to add signalling for byte-alignment before the syntax element ifm_data_payload_byte[ i ][ j ]. Text that are added are indicated by yellow highlight.

…

Image format metadata SEI message

| image_format_metadata( payloadSize ) { | Descriptor |
| --- | --- |
| ifm_cancel_flag | u(1) |
| if ( !ifm_cancel_flag ) { |  |
| ifm_persistence_flag | u(1) |
| ifm_num_metadata_payloads | ue(v) |
| for ( i=0; i<ifm_num_metadata_payloads; i++ ) { |  |
| ifm_type_id[ i ] | ue(v) |
| ifm_uri_present_flag[ i ] | u(1) |
| if( !ifm_uri_present_flag[ i ] ) { |  |
| ifm_payload_len_minus1[ i ] | ue(v) |
| while( !byte_aligned( ) ) |  |
| ifm_bit_equal_to_zero | f(1) |
| for( j = 0; j <= ifm_payload_len[ i ]; j++ ) |  |
| ifm_data_payload_byte[ i ][ j ] | b(8) |
| } |  |
| else { |  |
| while( !byte_aligned( ) ) |  |
| ifm_bit_equal_to_zero | f(1) |
| ifm_data_uri[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Modify constraint on gfv_cnt and gfv_nn_present_flag

Problem statement

The current design of generative face video (GFV) SEI message includes the signalling of translator neural network which may be present in the GFV SEI message itself, in other location specified by URI, or in other GFV SEI message with the same Id. The following is the semantic of gfv_nn_present_flag.

gfv_nn_present_flag equal to 1 indicates that a neural network that may be used as a TranslatorNN( ) is contained or indicated by the SEI message. gfv_nn_present_flag equal to 0 indicates that a neural network that may be used as a TranslatorNN( ) is not contained or indicated by the SEI message. When gfv_nn_present_flag is not present, it is inferred to be 0.

When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following constraint applies:

–	If gfv_cnt is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise (gfv_cnt is greater than 0), there shall be at least one GFV SEI message that is present in either the current picture unit or a preceding picture unit in output order in the current CLVS and has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of this SEI message, the following applies for deriving the applicable TranslatorNN:

–	If gfv_cnt is greater than 0 and there exists one or more preceding GFV SEI messages in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last preceding GFV SEI message in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise, the applicable TranslatorNN is defined by a GFV SEI message that is present in the last preceding picture unit puB in output order in the current CLVS that has the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1. When there are multiple such GFV SEI messages present in the picture unit puB that have the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last of such GFV SEI messages in decoding order.

Among constraints related to the syntax element gfv_nn_present_flag, in our assertion, the following constraint may be problematic.

“When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1.”

To our understanding, this only requires the first GFV SEI message in the first picture unit in a CLVS to have gfv_nn_present_flag equal to 1. This does not seem right since it does not ensure the mechanism for finding reference TranslatorNN would work when a GFV SEI message has gfv_nn_present_flag is equal to 0 since there is no guarantee that the first GFV SEI message of each particular gfv_id will have gfv_nn_present_flag equal to 1. In our opinion, instead of the above constraint, what is needed is a constraint that requires the first GFV SEI message of a particular gfv_id in a CLVS to have gfv_nn_present_flag to be equal to 1.

Proposed modifications

It is proposed to specify that when a GFV SEI message is the first GFV SEI message with a particular gfv_id in a CLVS in output order, the gfv_nn_present_flag shall be present and set to 1. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

…

When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1.

When a GFV SEI message is the first GFV SEI message with a particular gfv_id in a CLVS in output order, gfv_nn_present_flag shall be present and equal to 1.

…

Define constraint on bri_nuh_layer_id_present_flag and bri_pic_partition_flag

Problem statement

In the current design of bitdepth range information SEI message (BRI) SEI message, bit ranges are represented using separate layer pictures, subpictures, or constituent rectangles. However, bri_nuh_layer_id_present_flag and bri_pic_partition_flag can both be equal to 0, meaning that neither bri_nuh_layer_id nor bri_partition_id are defined, leaving no information to distinguish between multiple ranges.

Proposed modifications

It is proposed to define a constraint for the BRI SEI message to disallow both bri_nuh_layer_id_present_flag and bri_pic_partition_flag from being equal to 0. Text that are added are indicated by yellow highlight.

It is a requirement of bitstream conformance that at least one of bri_nuh_layer_id_present_flag or bri_pic_partition_flag shall be equal to 1 when bri_num_ranges_minus1 is greater than 0.

Change doi_num_display_overlays_minus2 to doi_num_display_overlays_minus1

Problem statement

In the current design of display overlays SEI message (DOI) SEI message, the number of display overlays is signaled by doi_num_display_overlays_minus2, which means the minimum number of display overlays is two. However, in our assertion, this design may be inefficient when there is only one item to display, as defining two overlays to represent an empty area is unnecessary. The size of the displayed picture can be expanded, and the expanded area can be left as empty space for future use.

Proposed modifications

It is proposed to change doi_num_display_overlays_minus2 to doi_num_display_overlays_minus1. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

| display_overlays_info( payloadSize ) { | Descriptor |
| --- | --- |
| doi_id | u(6) |
| doi_cancel_flag | u(1) |
| if( !doi_cancel_flag ) { |  |
| doi_persistence_flag | u(1) |
| doi_num_display_overlays_minus21 | ue(v) |
| doi_target_pic_size_present_flag | u(1) |
| if( doi_target_pic_size_present_flag ) |  |
| doi_target_pic_width_minus1 | u(16) |
| doi_target_pic_height_minus1 | u(16) |
| } |  |
| doi_nuh_layer_id_present_flag | u(1) |
| doi_pic_partition_flag | u(1) |
| if( doi_pic_partition_flag ) { |  |
| doi_partition_type_flag | u(1) |
| doi_partition_id_len_minus1 | u(4) |
| } |  |
| doi_offset_params_present_flag | u(1) |
| if( doi_offset_params_present_flag ) |  |
| doi_offset_param_length_minus1 | u(4) |
| doi_resampling_enabled_flag | u(1) |
| if( doi_resampling_enabled_flag ) |  |
| doi_size_param_length_minus1 | u(4) |
| for( i = 0; i < doi_num_display_overlays_minus21 + 21; i++ ) { |  |
| if ( doi_nuh_layer_id_present_flag ) |  |
| doi_nuh_layer_id[ i ] | u(6) |
| if ( doi_pic_partition_flag ) |  |
| doi_partition_id[ i ] | u(v) |
| doi_alpha_present_flag[ i ] | u(1) |
| if ( doi_alpha_present_flag[ i ] ) { |  |
| if (doi_nuh_layer_id_present_flag) |  |
| doi_alpha_nuh_layer_id[ i ] | u(6) |
| if (doi_pic_partition_flag) |  |
| doi_alpha_partition_id[ i ] | u(v) |
| } |  |
| if( i > 0 ) { |  |
| if (doi_offset_params_present_flag ) |  |
| doi_top_left_x[ i ] | u(v) |
| doi_top_left_y[ i ] | u(v) |
| } |  |
| if( doi_resampling_enabling_flag ) { |  |
| doi_width_minus1[ i ] | u(v) |
| doi_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

…

doi_num_display_overlays_minus21 plus 21 specifies the number of display overlays for which information is signalled in the SEI message. The value of doi_num_display_overlays_minus21 shall be in the range of 0 to 301, inclusive.

…

for( i = 0; i < doi_num_display_overlays_minus21 + 21; i++ )  {
	for( h = 0, y = doi_top_left_y[ i ]; y < DisplayOverlayHeight[ i ]; h++, y++ ) 
		for( w = 0, x = doi_top_left_x[ i ]; x < DisplayOverlayWidth[ i ]; w++, x++ )
			if( !doi_alpha_present_flag[ i ] )
				TargetPicture[ 0 ][ x ][ y ] = OverlayTexture[ i ][ 0 ][ w ][ h ]	
			else
				TargetPicture[ c ][ x ][ y ] = OverlayWithAlpha( TargetPicture[ 0 ][ x ][ y ], 
						OverlayTexture[ 0 ][ i ][ w ][ h ], OverlayAlpha[ i ][ w ][ h ] )

for( ( cIdx = 1; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ ++ )  {
		for( h = 0, y = doi_top_left_y[ i ]/SubHeightC; y < DisplayOverlayHeight[ i ]/ SubHeightC; h++, y++ ) 
			for( w = 0, x = doi_top_left_x[ i ]/SubWidthC; x < DisplayOverlayWidth[ i ]/ SubWidthC; w++, x++ )
				if( !doi_alpha_present_flag[ i ] )
					TargetPicture[ cIdx ][ x ][ y ] = OverlayTexture[ i ][ cIdx ][ w ][ h ]					
				else
					TargetPicture[ cIdx ][ x ][ y ] = OverlayWithAlpha( TargetPicture[ cIdx ][ x ][ y ], 
							OverlayTexture[ cIdx ][ i ][ w ][ h ], OverlayAlpha[ i ][ w ][ h ] )

Modify the condition to signal the presence of offset parameters and resampling information

Problem statement

The coded picture size of the display overlay with index 0 may not be equal to the target picture size, so the offset parameters and resampling information for the display overlay need to be signaled. In the current design of the DOI SEI message, the offset parameters can be signaled when the index of the display overlay is greater than zero, and the resampling information can also be signaled when the index of the display overlay is greater than zero.

Proposed modifications

It is proposed to modify to signal the presence of offset parameters and resampling information, along with related information, regardless of the number of display overlays. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

| display_overlays_info( payloadSize ) { | Descriptor |
| --- | --- |
| doi_id | u(6) |
| doi_cancel_flag | u(1) |
| if( !doi_cancel_flag ) { |  |
| doi_persistence_flag | u(1) |
| doi_num_display_overlays_minus2 | ue(v) |
| doi_target_pic_size_present_flag | u(1) |
| if( doi_target_pic_size_present_flag ) |  |
| doi_target_pic_width_minus1 | u(16) |
| doi_target_pic_height_minus1 | u(16) |
| } |  |
| doi_nuh_layer_id_present_flag | u(1) |
| doi_pic_partition_flag | u(1) |
| if( doi_pic_partition_flag ) { |  |
| doi_partition_type_flag | u(1) |
| doi_partition_id_len_minus1 | u(4) |
| } |  |
| doi_offset_params_present_flag | u(1) |
| if( doi_offset_params_present_flag ) |  |
| doi_offset_param_length_minus1 | u(4) |
| doi_resampling_enabled_flag | u(1) |
| if( doi_resampling_enabled_flag ) |  |
| doi_size_param_length_minus1 | u(4) |
| for( i = 0; i < doi_num_display_overlays_minus2 + ; i++ ) { |  |
| if ( doi_nuh_layer_id_present_flag ) |  |
| doi_nuh_layer_id[ i ] | u(6) |
| if ( doi_pic_partition_flag ) |  |
| doi_partition_id[ i ] | u(v) |
| doi_alpha_present_flag[ i ] | u(1) |
| if ( doi_alpha_present_flag[ i ] ) { |  |
| if (doi_nuh_layer_id_present_flag) |  |
| doi_alpha_nuh_layer_id[ i ] | u(6) |
| if (doi_pic_partition_flag) |  |
| doi_alpha_partition_id[ i ] | u(v) |
| } |  |
| if( i > 0 ) { |  |
| if (doi_offset_params_present_flag ) { |  |
| doi_top_left_x[ i ] | u(v) |
| doi_top_left_y[ i ] | u(v) |
| } |  |
| if( doi_resampling_enabling_flag ) { |  |
| doi_width_minus1[ i ] | u(v) |
| doi_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).