| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0102-v1 |
| --- | --- |

| Title: | Non-EE2: Intra mode coding based on HoG of neighboring templates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Myungoh Hong Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung-Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6477 myungoh.hong@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes an intra mode coding modification utilizing a histogram of gradients (HoG) extracted from neighboring templates. The proposed method first derives 8 intra prediction modes from the HoG of neighboring template (e.g., above, left, above-left). The intra mode with the smallest template cost is selected as the Most Dominant Intra Prediction (MDIP) mode and a flag to indicate its usage is signaled. To further reduce signaling overhead, 16 improbable intra modes with small HoG amplitude are removed from MPM, SMPM, and non-MPM list. Experimental results for the proposed method, implemented on top of the ECM-14.0, are reported as follows:

- For AI configuration: -0.09%/-0.04%/-0.02%, 101.5%(EncT), 101.4%(DecT).

- For RA configuration: -0.03%/-0.13%/0.05%, 100.3%(EncT), 99.9%(DecT).

Introduction

In the current ECM, applying DIMD involves deriving up to five intra modes from reconstructed neighboring samples. These directional predictors are then combined with a non-directional predictor (either planar or block vector-based) using weights calculated from the histogram of gradients (HoG). This HoG-based approach enhances intra prediction by combining existing modes to generate a new prediction mode. Unlike DIMD, this contribution focus on intra mode coding improvement based on HoG.

For example, as shown in Fig. 1, the HoG for each intra mode indicates that vertical intra prediction mode and its neighboring modes are highly correlated with the current block. On the other hand, the horizontal intra prediction mode and its neighboring modes may be irrelevant to the current block. Therefore, the proposed method utilizes the HoG to improve existing intra mode coding by introducing the most dominant intra prediction (MDIP) mode and the 16 least probable intra prediction modes. The detailed process is described in the following subsection.

Figure 1. Example of the HoG

Proposed methods

The proposed method derives 8 intra prediction modes based on a histogram of gradients (HoG) from neighboring templates. The HoG computation follows the same process as in DIMD, but template area differs. If the block size is less than 256, a 3-line template area is used. Otherwise, template area is extended to 4-lines as shown in Fig. 2. Specifically, these 8 intra prediction modes are derived by the HoG based on predefined template areas as follows:

- 4 intra prediction modes are derived from above and left template area

- 2 intra prediction modes are derived from left template area

- 2 intra prediction modes are derived from above template area

- During the above processes, duplicate intra prediction modes are excluded. Instead, neighboring modes are added until 8 intra prediction modes are identified.

After constructing 8 intra prediction modes, they are ordered by computing their template cost. The intra prediction mode with the minimum template cost is designated as the Most Dominant Intra Prediction (MDIP) mode and signaled before MPM information at each coding unit (CU) level. The above ordering process utilizes the left and top 1-line segments of the current block as the template area. In order to avoid redundancy, the MDIP mode is excluded from MPM, secondary MPM, and non-MPM. To further reduce signaling overhead, after computing the HoG, 16 improbable intra modes with small HoG amplitude are also removed from MPM, SMPM, and non-MPM list.

Figure 2. Template area to compute the HoG

Experimental results

The proposed method is tested on top of ECM-14.0 [1] under the common test conditions [2].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.11% | -0.20% | -0.13% | 101.3% | 101.2% | 99.2% | 100.2% |
| Class A2 | -0.05% | -0.03% | -0.03% | 101.2% | 101.2% | 99.4% | 100.2% |
| Class B | -0.07% | -0.06% | 0.06% | 101.6% | 101.7% | 99.1% | 100.0% |
| Class C | -0.12% | 0.02% | -0.06% | 101.5% | 101.3% | 100.1% | 100.0% |
| Class E | -0.12% | 0.08% | 0.00% | 101.5% | 101.3% | 100.1% | 100.0% |
| Overall | -0.09% | -0.04% | -0.02% | 101.5% | 101.4% | 99.6% | 100.1% |
| Class D | -0.10% | -0.03% | -0.12% | 102.4% | 103.6% | 100.1% | 100.0% |
| Class F | -0.13% | -0.08% | 0.05% | 100.7% | 100.9% | 99.5% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.15% | -0.11% | 100.2% | 100.6% | 100.2% | 99.8% |
| Class A2 | 0.00% | -0.05% | 0.00% | 100.1% | 99.4% | 100.2% | 99.7% |
| Class B | -0.03% | -0.08% | 0.12% | 100.4% | 99.7% | 100.0% | 99.9% |
| Class C | -0.05% | -0.24% | 0.14% | 100.5% | 100.0% | 99.7% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.13% | 0.05% | 100.3% | 99.9% | 100.0% | 99.9% |
| Class D | -0.03% | -0.28% | -0.31% | 100.7% | 100.3% | 100.0% | 100.0% |
| Class F | -0.02% | -0.44% | -0.18% | 100.5% | 100.2% | 100.0% | 99.9% |

References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AI2017, Jul. 2024.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).