| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0104-v1 |
| --- | --- |

| Title: | AHG9: On signalling of prompt in NNFPA SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Tan Chul Keun Kim Junghak Nam Jangwon Lee Jaehyun Lim Seung hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

Signalling of input prompt in NNPFC SEI message when the neural-network post-filter (NNPF) includes spatial extrapolation filtering was adopted in the 35th JVET meeting. This contribution asserted that the capability for flexibly change the input prompt without having to send an update NNPFC is desirable so that the same filter once it is sent in an NNPFC can be used with different prompt at the activation.

To support the asserted desirable capability, it is proposed to allow the input prompt signalling in the NNPFC to be updated at activation. The detail of the proposal is as follows:

  - Add an inference such that when nnpfc_spatial_extrapolation_prompt_present_flag is not present, its value is inferred to be equal to 0.

  - In the extension part of NNPFA SEI message, signal a new flag (i.e., nnpfa_spatial_extrapolation_prompt_update_flag) to specify whether an update to input prompt is present in the NNPFA SEI message.

    - It is constrained that when nnpfc_spatial_extrapolation_prompt_present_flag is equal to 0, the value of nnpfa_spatial_extrapolation_prompt_update_flag, if present, shall be equal to 0.

  - When nnpfa_spatial_extrapolation_prompt_update_flag is equal to 1, signal updated input prompt (i.e., nnpfa_prompt).

# Problem Statement

In the 35th JVET meeting, it was agreed to add signalling for prompt in NNPFC SEI message when the filter includes spatial extrapolation filter. The syntax and semantics of this prompt feature is as shown below.

-------------------------Snippet from the specification draft start-------------------------

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … | u(16) |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | se(v) |
| nnpfc_spatial_extrapolation_right_offset | se(v) |
| nnpfc_spatial_extrapolation_top_offset | se(v) |
| nnpfc_spatial_extrapolation_bottom_offset | se(v) |
| nnpfc_spatial_extrapolation_prompt_present_flag | u(1) |
| if( nnpfc_spatial_extrapolation_prompt_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

nnpfc_spatial_extrapolation_prompt_present_flag equal to 1 specifies that nnpfc_prompt syntax element is present and nnpfc_alignment_zero_bit_c syntax element may be present. nnpfc_spatial_extrapolation_prompt_present_flag equal to 0 specifies that nnpfc_prompt syntax element and nnpfc_alignment_zero_bit_c syntax element are not present.

nnpfc_alignment_zero_bit_c shall be equal to 0.

nnpfc_prompt specifies the text string prompt used for generating the contents of the spatial extrapolation image area. When nnpfc_spatial_extrapolation_prompt_present_flag is equal to 1, nnpfc_prompt shall not be a null string.

-------------------------Snippet from the spefication draft start-------------------------

It is asserted that the above design does not allow the input prompt to be changed without sending another update of NNPFC SEI message. The capability for flexibly change the input prompt without having to send an update NNPFC SEI message (i.e., change the input prompt at the activation of the NNPF) is desirable so that the same filter once it is sent in an NNPFC can be used with different prompt at the activation.

# Proposals

We propose to allow the input prompt signalling in the NNPFC to be updated at activation. The detail of the proposal is as follows:

- Add an inference such that when nnpfc_spatial_extrapolation_prompt_present_flag is not present, its value is inferred to be equal to 0.

- In the extension part of NNPFA SEI message, signal a new flag (i.e., nnpfa_spatial_extrapolation_prompt_update_flag) to specify whether an update to input prompt is present in the NNPFA SEI message.

    - It is constrained that when nnpfc_spatial_extrapolation_prompt_present_flag is equal to 0, the value of nnpfa_spatial_extrapolation_prompt_update_flag, if present, shall be equal to 0.

- When nnpfa_spatial_extrapolation_prompt_update_flag is equal to 1, signal updated input prompt (i.e., nnpfa_prompt).

# Text implementation

Add the following inference for when nnpfc_spatial_extrapolation_prompt_present_flag is not present.

…

nnpfc_spatial_extrapolation_prompt_present_flag equal to 1 specifies that nnpfc_prompt syntax element is present and nnpfc_alignment_zero_bit_c syntax element may be present. nnpfc_spatial_extrapolation_prompt_present_flag equal to 0 specifies that nnpfc_prompt syntax element and nnpfc_alignment_zero_bit_c syntax element are not present. When not present, the value of nnpfc_spatial_extrapolation_prompt_present_flag is inferred to be equal to 0.

…

Changes to the syntax and semantics of NNPFA SEI message are as follows:

…

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_target_base_flag | u(1) |
| nnpfa_no_prev_clvs_flag | u(1) |
| if( nnpfa_persistence_flag ) |  |
| nnpfa_no_foll_clvs_flag | u(1) |
| nnpfa_num_output_entries | ue(v) |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |
| nnpfa_output_flag[ i ] | u(1) |
| if( more_data_in_payload( ) ) { |  |
| nnpfa_spatial_extrapolation_prompt_update_flag | u(1) |
| if( nnpfa_spatial_extrapolation_prompt_update_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfa_alignment_zero_bit | u(1) |
| nnpfa_prompt | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

…

nnpfa_spatial_extrapolation_prompt_update_flag equal to 1 specifies that nnpfa_prompt syntax element is present and nnpfa_alignment_zero_bit syntax element may be present. nnpfa_spatial_extrapolation_prompt_update_flag equal to 0 specifies that nnpfa_prompt syntax element and nnpfa_alignment_zero_bit syntax element are not present. When not present, the value of nnpfa_spatial_extrapolation_prompt_update_flag is inferred to be equal to 0.

When nnpfc_spatial_extrapolation_prompt_present_flag is equal to 0, the value of nnpfa_spatial_extrapolation_prompt_update_flag, if present, shall be equal to 0.

nnpfa_alignment_zero_bit shall be equal to 0.

nnpfa_prompt specifies the text string prompt used for generating the contents of the spatial extrapolation image area generated by the target NNPF. When nnpfa_spatial_extrapolation_prompt_update_flag is equal to 1, nnpfa_prompt shall not be a null string. When nnpfa_prompt is present, nnpfc_prompt text within the process DeriveInputTensors( ) specified in clause 8.28.2.2 is replaced by nnpfa_prompt text.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).