| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0105-v1 |
| --- | --- |

| Title: | AHG9: On signalling of complexity information in SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Tan Jangwon Lee Chul Keun Kim Junghak Nam Jaehyun Lim Seung hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes to include signalling of complexity information in SEI processing order (SPO) SEI message. It is asserted that although complexity information may be present in NNPFC SEI message that is included in an SPO SEI message, it is desirable to also have complexity information to be present in SPO SEI message as well to inform the overall expected complexity of the processing chain specified by the SPO SEI message. More motivation to have such signalling in SPO SEI message includes:

- Although some SEI messages that are included in SPO SEI message may include complexity information (e.g., NNPFC SEI message), however, this would require decoder to parse into each individual member SEI message which may not be desirable.

- There are SEI messages that define process but do not have the signalling of complexity information. When a processing chain includes mix of SEI messages that have complexity information and SEI messages that do not have such information, it is not possible to infer such information without explicitly signalling it.

- The way SEI messages that are included in a processing chain can be different. For example, some may need to be processed in breadth-first while others in depth-first approach. Given such possibility, the complexity signalling that may be present in each NNPFC SEI message may not be usable.

To implement the asserted desirable functionality, the following changes is proposed:

- Include signalling of complexity information in SPO SEI message. The signalling of complexity information may be similar to the design of complexity signalling for NNPFC SEI message.

- The complexity information in SPO SEI message, when present, specifies the maximum complexity for invoking all SEIs in the processing chain specified by the SPO SEI message.

# Problem Statement

SEI processing order (SPO) SEI message may include one or more SEI messages that together form a processing chain. Within a CLVS, multiple SPO SEI messages may be present and decoder may choose which processing chain to use / invoke. It is asserted that the decision of which processing chain to use may include the complexity that decoder needs / may expect when invoking the processing chain.

Although, complexity information may be present in NNPFC SEI message that is included in an SPO SEI message, in our assertion, it is desirable to also have complexity information to be present in SPO SEI message to inform the overall expected complexity of the processing chain specified by the SPO SEI message.

The non-exhausted list provides reasons why it is good to have complexity signalling in SPO SEI message:

- Although some SEI messages that are included in SPO SEI message may include complexity information (e.g., NNPFC SEI message), however, this would require decoder to parse into each individual member SEI message which may not be desirable.

- There are SEI messages that define process but do not have the signalling of complexity information. When a processing chain includes mix of SEI messages that have complexity information and SEI messages that do not have such information, it is not possible to infer such information without explicitly signalling it.

- The way SEI messages that are included in a processing chain can be different. For example, some may need to be processed in breadth-first while others in depth-first approach. Given such possibility, the complexity signalling that may be present in each NNPFC SEI message may not be usable.

# Proposals

We propose the following changes to the design of SPO SEI message:

- Include signalling of complexity information in SPO SEI message. The signalling of complexity information may be similar to the design of complexity signalling for NNPFC SEI message.

- The complexity information in SPO SEI message, when present, specifies the maximum complexity for invoking all SEIs in the processing chain associated with the SEI processing order SEI message.

# Text implementation

…

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_for_human_viewing_idc | u(2) |
| po_for_machine_analysis_idc | u(2) |
| po_reserved_zero_4bits | u(4) |
| po_num_sei_messages_minus2 | u(7) |
| po_breadth_first_flag | u(1) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_processing_degree_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(12) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| po_complexity_info_present_flag | u(1) |
| if( po_complexity_info_present_flag ) { |  |
| po_parameter_type_idc | u(2) |
| if( po_parameter_type_idc != 2 ) |  |
| po_log2_parameter_bit_length_minus3 | u(2) |
| po_num_parameters_idc | u(6) |
| po_num_kmac_operations_idc | ue(v) |
| po_total_kilobyte_size | ue(v) |
| } |  |
| } |  |

…

po_complexity_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the complexity of the processing chain specified by this SEI processing order SEI message are present. po_complexity_info_present_flag equal to 0 specifies that no syntax elements that indicates the complexity of the processing chain specified by this SEI processing order SEI message are present.

po_parameter_type_idc equal to 0 indicates that the processes required by the processing chain specified by this SEI processing order SEI message uses only integer parameters. po_parameter_type_flag equal to 1 indicates that the processes required by the processing chain specified by this SEI processing order SEI message may use floating point or integer parameters. po_parameter_type_idc equal to 2 indicates that the processes required by the processing chain specified by this SEI processing order SEI message uses only binary parameters. po_parameter_type_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore SPO SEI messages with po_parameter_type_idc equal to 3.

po_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the processes required by the processing chain specified by this SEI processing order SEI message do not use parameters of bit length greater than 8, 16, 32, and 64, respectively. When po_parameter_type_idc is present and po_log2_parameter_bit_length_minus3 is not present, the processes required by the processing chain specified by this SEI processing order SEI message do not use parameters of bit length greater than 1.

po_num_parameters_idc indicates the maximum number of parameters needed by processes required by the processing chain specified by this SEI processing order SEI message in units of a power of 2 048. po_num_parameters_idc equal to 0 indicates that the maximum number of parameters needed by processes required by the processing chain specified by this SEI processing order SEI message is unknown. The value po_num_parameters_idc shall be in the range of 0 to 52, inclusive. Values of po_num_parameters_idc greater than 52 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore SPO SEI messages with po_num_parameters_idc greater than 52.

If the value of po_num_parameters_idc is greater than zero, the variable maxNumParameters is derived as follows:

maxNumParameters = ( 2 048  <<  po_num_parameters_idc ) − 1	(xx)

It is a requirement of bitstream conformance that the number of parameters shall be less than or equal to maxNumParameters.

po_num_kmac_operations_idc greater than 0 indicates that the maximum number of multiply-accumulate operations per sample of the processes required by the processing chain specified by this SEI processing order SEI message is less than or equal to po_num_kmac_operations_idc * 1 000. po_num_kmac_operations_idc equal to 0 indicates that the maximum number of multiply-accumulate operations of the processes is unknown. The value of po_num_kmac_operations_idc shall be in the range of 0 to 232 − 2, inclusive.

po_total_kilobyte_size greater than 0 indicates a total size in kilobytes required to store the uncompressed parameters for the processes required by the processing chain specified by this SEI processing order SEI message. The total size in bits is a number equal to or greater than the sum of bits used to store each parameter. po_total_kilobyte_size is the total size in bits divided by 8 000, rounded up. po_total_kilobyte_size equal to 0 indicates that the total size required to store the parameters for the processes required by the processing chain specified by this SEI processing order SEI message is unknown. The value of po_total_kilobyte_size shall be in the range of 0 to 232 − 2, inclusive.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).