| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0106-v1 |
| --- | --- |

| Title: | AHG14: The extension of SADL library |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nianxiang Fu, Weijie Bao, Zhenzhong Chen | Email: | {nianxiangfu, baoweijie, zzchen}@whu.edu.cn |
| Source: | Wuhan University |  |  |

_____________________________

- Abstract

This contribution introduces the extension to the BatchNorm operator in the Small AdHoc Deep Learning (SADL) library. The extension features an efficient algorithm for computing the integer approximation of the square root operation in BatchNorm without the need for loops.

Overview of extensions

Table 1 below summarizes the extensions in the SADL library [1].

Table 1: The overview of extensions in the SADL library.

| Language | Pure C++, header only |
| --- | --- |
| Compatibility | Support PyTorch and TensorFlow, while also updating the Onnx to the SADL converter |
| Extended layers | BatchNorm |
| Type support | float, int32, int16, int8 |
| License | BSD 3-Clause |

Extended features

BatchNorm

Extension description: BatchNorm normalizes the input tensor X by using the affine parameters and the running mean and variance accumulated during training. This operation involves standardizing the input based on the running mean and variance, followed by scaling and shifting using learnable affine parameters gamma (scale) and beta (shift), as follows:

BNx=γx - μσ2 + ϵ +β,

where x represents the elements of the input tensor; μ and σ2 denote the running mean and variance, respectively; ϵ denotes a small constant for numerical stability (to prevent division by zero); γ and β denote scale and shift parameters, respectively.

In the original SADL library, the integer approximation of the square root operation in BatchNorm uses a bitwise square root algorithm, which computes the integer square root bit by bit through binary operations. However, this method requires loop operations, which may not be suitable for NN hardware. In this contribution, we propose an efficient non-iterative algorithm to address this issue. The proposed algorithm is described below.

Specifically, given a positive number 𝑎, its square root is 𝑏, and the following equation holds:
b=a=a12=2log2 a2.

When 𝑎 is a positive integer, we use the following equation to approximate log2 a as an integer:

log2_a=⌊log2 a⌋=bit_widtha-1,

where bit_widtha calculates the number of bits needed to store the value a, which is a cheap bitwise operation. Then, using the following formula, we can approximate 𝑏 as an integer:
log2_b=⌊log2_a2⌋=log2_a≫1,
b=2log2_b=2≪log2_b.

In this way, we obtain a rough integer approximation b. We then need to refine it to get closer to the true value. We define the difference between 𝑎 and the square of b as f(b)=a-b2. Our goal is to minimize

f(b) to zero. According to Newton’s method, the following equation holds:

b1=b-fbf'b=b+a-b22b=b+a-b2b≪1,

b2=b1-fb1f'b1=b1+a-b122b1=b1+a-b12b1≪1,

……,

bn=bn-1-fbn-1f'bn-1=bn-1+a-bn-122bn-1=bn-1+a-bn-12bn-1≪1.

Theoretically, by iterating this process, we can obtain a refined b. In practice, based on our experiments, we found that two iterations yield a sufficiently accurate result, meaning b2 is already a satisfactory solution.

In conclusion, our algorithm computes a rough binary approximation of the square root and refines it using two iterations of Newton’s method. This results in an accurate estimate, requiring only a few bitwise and arithmetic operations, making it an efficient, non-iterative algorithm.

Changes details

Changes are as follows:

- Replaced the original iterative algorithm for the integer approximation of the square root operation in the BatchNorm layer with a more efficient, non-iterative approach.

Conclusion

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Wuhan University.

Reference

- F. Galpin, F. Urban, T. Poirier, X. Li. AhG14: SADL update. JVET-AI0057, 35th Meeting, Sapporo, JP, 12–19 July 2024.

Acknowledgement

We are grateful to InterDigital Inc. for their valuable guidance and assistance. Their contributions have made a significant impact on this work.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.