| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0107-v2 |
| --- | --- |

| Title: | EE2-3.4: GPM mode extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn, Junghak Nam, Naeri Park Jaehyun Lim, Seunghwan Kim (LGE), Kevin Reuzé, Ya Chen, Philippe Bordes Mikaël Le Pendu (InterDigital) | Tel: Email: | yongjo.ahn@lge.com Kevin.Reuze@InterDigital.com |
| Source: | LG Electronics Inc., InterDigital Inc. |  |  |

_____________________________

- Abstract

This contribution reports results of EE2 Test 3.4 on GPM mode extension. In this test, the number of GPM partitioning modes is extended with the additional angles and distances. Especially, different numbers of total GPM extension modes were studied to find the trade-off between encoding gain and complexity along with block shape and size adaptation. This test includes four sub-tests as below:

- Test 3.4a: GPM mode extension with 112 modes

- Test 3.4b: GPM mode extension with block size and shape adaptation (keep 64 modes)

- Test 3.4d: Test 3.4b not applied on IBC-GPM nor SGPM

- Test 3.4e: GPM mode extension with block size and shape adaptation (extend 112, 84, 70, 64 modes)

Introduction

In the VVC and ECM, GPM supports 64 partitioning modes from the combination between 20 angles and 4 distances. In the initial contribution of GPM, 32 angles with 11.25 degrees spacing were proposed. During CE discussion, it was simplified *64 GPM modes with 20 slope based angles and 4 distances to get a better trade-off between signaling overhead and coding gain. (*64 GPM modes = 8 angles × 4 distances + 8 angles × 3 distances + 4 angles × 2 distances)

Figure 1. An example of 64 GPM partitioning modes in VVC/ECM

Proposed methods

The EE2 test-3.4 proposed to extend the number of GPM partitioning modes with the additional angles and distances. Technical details of the proposed method are described in the following subsections:

Additional angles

In the proposed GPM mode extension, the number of GPM angles are extended to 32. Especially, test 3.4a/e proposed 12 additional slope based angles with equal spacing [1], and test 3.4b/d proposed 12 additional slope based un-equal spacing [2].

Figure 2. The proposed 32 angles for Test 3.4a/e

Figure 3. The proposed 32 angles for Test 3.4b/d

Test 3.4a: GPM mode extension with 112 modes

In Test 3.4a, the total number of GPM partitioning modes is extended to 112 modes. Firstly, the number of GPM angles are extended to 32. In the current ECM, GPM supports 20 slope based angles and indices with un-equal spacing. It was proposed that the angle index near vertical angle direction (horizontal split directions) are removed since most nature objects or motions are vertically. The proposed extension supports 12 additional slope based angles with equal spacing. The additional angles can be combined with 4 distances same as the conventional angles, and total of *42 GPM modes are newly added. (*42 GPM modes = 6 angles × 4 distances + 6 angles × 3 distances) In addition, the proposed method of test 3.4a adds 6 GPM modes for 6 horizontal and vertical distances. In the current GPM, six distance indices for horizontal and vertical angles (0, 8, 16, 24) are removed because these six partitioning modes are likely to overplay with binary tree and ternary tree split. However, blending process can be applied to the boundary samples of GPM partitions, it makes different coding results compared with binary tree and ternary tree split on CU partitioning level.

Figure 4. The proposed angles and distances for 112 GPM modes

Test 3.4b and d: GPM mode extension with block size and shape adaptation (keep 64 GPM modes)

In this contribution, it is proposed to adapt the partitions used by GPM, SGPM and IBC-GPM to be dependent on the block shape, similar to wide angle intra prediction. In particular, it is asserted that diagonal splits are favored for a given block shape, therefore diagonal and inverted diagonal splits are included for each block shape, as well as splits with angles close to those. This means that the range of angles available GPM, SGPM and IBC-GPM is increased from 20 to 32.

For each width/height ratio from width/height=1/8 to width/height=8 a list of GPM partitions is proposed so that each block shape uses the most relevant partitions. The partitions are ordered so that the first 8 partitions constitute the IBC-GPM first set, the next 40 are the ones to be used for IBC-GPM second set, and the first 26 are the ones to be used for SGPM.

A difference with contribution AI0063 is that test 3.4b and d do not increase the number of modes for regular GPM which improves the encoding runtime trade-off.

For a visual representation, the allowed modes are the following for three different block shapes:

Figure 5. Allowed angles for blocks with W/H=1 (identical to ECM-14.0)

Figure 6. Allowed angles for blocks with W/H=2

Figure 7. Allowed angles for blocks with W/H=4

In test 3.4b the change is applied to GPM, SGPM and IBC-GPM. IN test 3.4d the change is only applied to GPM.

Test 3.4e: GPM mode extension with block size and shape adaptation (extended 112, 84, 70, 64 modes)

In Test 3.4e, the total number of GPM partitioning modes is extended to 112, 84, 70, or 64 modes according to block size and shapes. In this test, the proposed method has six categories according to the block sizes and shapes. The number of extended GPM modes can be applied by the block sizes and shapes. The details of the number of GPM modes according to block sizes and shapes are shown as below:

- Small blocks (cuWidth*cuHeight <= 64): 64 GPM modes (same as 64 GPM modes in VVC/ECM)

- 2NxN blocks: 84 GPM modes

Figure 8. Allowed angles for 2NxN blocks

- 4NxN blocks: 70 GPM modes

Figure 9. Allowed angles for 4NxN blocks

- Nx2N blocks: 84 GPM modes

Figure 10. Allowed angles for Nx2N blocks

- Nx4N blocks: 70 GPM modes

Figure 11. Allowed angles for Nx4N blocks

- Large 2Nx2N blocks: 112 GPM modes (same as GPM modes in Figure 4)

Experimental results

The proposed GPM mode extensions have been implemented in ECM-14.0 [3] and evaluated under all intra (AI), random access (RA), and low-delay B (LDB) common test configurations [4].

In this EE test, four simulations are tested on top of ECM-14.0 as below:

- Test 3.4a: GPM mode extension with 112 modes

- Test 3.4b: GPM mode extension with block size and shape adaptation (keep 64 modes)

- Test 3.4d: Test 3.4b not applied on IBC-GPM nor SGPM

- Test 3.4e: GPM mode extension with block size and shape adaptation (extend 112, 84, 70, 64 modes)

Table 1. Performance of Test 3.4a (GPM mode extension with 112 modes)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.22% | -0.12% | 101.2% | 99.7% | 101.7% | 100.1% |
| Class A2 | -0.03% | 0.10% | 0.02% | 101.1% | 100.5% | 101.4% | 99.8% |
| Class B | -0.02% | -0.02% | 0.02% | 101.7% | 99.6% | 103.7% | 100.1% |
| Class C | -0.05% | -0.10% | 0.14% | 102.7% | 100.9% | 108.4% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.06% | 0.03% | 101.8% | 100.2% | 104.1% | 100.0% |
| Class D | -0.03% | -0.18% | -0.18% | 102.3% | 101.1% | 112.1% | 100.1% |
| Class F | 0.02% | -0.09% | 0.01% | 101.7% | 100.3% | 105.5% | 100.1% |
| Class TGM | -0.14% | -0.11% | -0.09% | 101.5% | 99.8% | 102.4% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.12% | -0.48% | 102.6% | 99.9% | 104.5% | 100.3% |
| Class C | 0.02% | -0.03% | 0.17% | 103.0% | 100.1% | 109.9% | 100.2% |
| Class E | 0.08% | 0.43% | -0.22% | 102.6% | 100.3% | 107.8% | 100.3% |
| Overall | 0.03% | 0.05% | -0.20% | 102.8% | 100.1% | 107.1% | 100.3% |
| Class D | 0.15% | 0.84% | 0.22% | 103.7% | 100.5% | 112.5% | 100.1% |
| Class F | -0.07% | -0.31% | 0.17% | 101.9% | 99.7% | 106.2% | 100.0% |
| Class TGM | -0.22% | -0.18% | -0.13% | 101.7% | 96.1% | 103.1% | 100.4% |

Table 2. Performance of Test 3.4b (GPM mode extension with block size and shape adaptation (keep 64 modes))

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.07% | -0.11% | 100.0% | 99.6% | 101.5% | 99.9% |
| Class A2 | -0.02% | 0.00% | 0.01% | 100.5% | 100.6% | 100.6% | 100.1% |
| Class B | -0.02% | 0.01% | 0.12% | 100.1% | 100.1% | 101.9% | 100.0% |
| Class C | -0.02% | -0.11% | -0.07% | 100.2% | 99.4% | 103.8% | 100.1% |
| Class E | -0.06% | 0.09% | 0.11% | 100.5% | 99.9% | 103.2% | 100.1% |
| Overall | -0.03% | -0.02% | 0.02% | 100.2% | 99.9% | 102.3% | 100.0% |
| Class D | -0.02% | -0.05% | -0.08% | 100.0% | 99.7% | 104.5% | 100.0% |
| Class F | -0.09% | 0.17% | -0.16% | 100.2% | 99.7% | 101.8% | 99.9% |
| Class TGM | -0.09% | -0.04% | -0.09% | 99.9% | 100.2% | 104.6% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.24% | 0.04% | 100.3% | 99.8% | 99.8% | 101.3% |
| Class A2 | -0.12% | -0.07% | -0.17% | 99.9% | 100.4% | 100.1% | 100.0% |
| Class B | -0.06% | -0.13% | -0.16% | 100.1% | 100.1% | 100.0% | 99.9% |
| Class C | -0.01% | -0.28% | 0.19% | 100.3% | 99.7% | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.18% | -0.03% | 100.1% | 100.0% | 100.1% | 100.2% |
| Class D | 0.02% | -0.32% | -0.30% | 100.4% | 100.0% | 100.1% | 100.1% |
| Class F | 0.06% | -0.41% | -0.02% | 100.5% | 99.8% | 100.1% | 99.9% |
| Class TGM | -0.21% | -0.22% | -0.20% | 99.9% | 99.8% | 100.9% | 99.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.04% | -0.22% | 100.6% | 100.5% | 100.1% | 100.1% |
| Class C | 0.00% | 0.29% | -0.35% | 99.3% | 99.9% | 100.0% | 100.3% |
| Class E | 0.10% | -0.69% | 0.68% | 100.2% | 100.4% | 101.1% | 100.0% |
| Overall | 0.01% | -0.09% | -0.04% | 100.1% | 100.3% | 100.3% | 100.2% |
| Class D | 0.03% | 0.45% | 0.70% | 100.4% | 99.9% | 100.0% | 100.1% |
| Class F | -0.14% | -0.60% | -0.29% | 100.3% | 100.3% | 99.9% | 100.1% |
| Class TGM | -0.38% | -0.32% | -0.22% | 100.3% | 100.5% | 99.4% | 100.0% |

Table 3. Performance of Test 3.4d (Test 3.4b not applied on IBC-GPM nor SGPM)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.30% | -0.12% | 100.5% | 100.1% | 99.9% | 100.4% |
| Class A2 | -0.10% | 0.02% | -0.15% | 100.6% | 100.5% | 100.2% | 99.5% |
| Class B | -0.05% | -0.05% | -0.01% | 99.5% | 100.0% | 100.0% | 99.9% |
| Class C | -0.03% | 0.03% | 0.04% | 100.0% | 100.0% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.06% | -0.05% | 100.0% | 100.1% | 100.0% | 100.0% |
| Class D | 0.03% | -0.15% | 0.04% | 99.8% | 100.2% | 99.9% | 100.1% |
| Class F | 0.02% | -0.03% | 0.04% | 100.2% | 100.1% | 100.1% | 99.8% |
| Class TGM |  |  |  |  |  |  |  |

Table 4. Performance of Test 3.4e (GPM mode extension with block size and shape adaptation (extend 112, 84, 70, 64 modes)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.20% | -0.07% | 100.7% | 100.1% | 101.5% | 99.8% |
| Class A2 | -0.11% | 0.07% | -0.10% | 100.5% | 99.7% | 101.6% | 99.8% |
| Class B | -0.06% | -0.03% | -0.05% | 100.4% | 100.3% | 103.6% | 100.1% |
| Class C | -0.08% | -0.03% | 0.04% | 100.8% | 100.6% | 108.5% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.04% | -0.04% | 100.6% | 100.2% | 104.0% | 100.0% |
| Class D | 0.01% | -0.02% | 0.12% | 101.0% | 100.1% | 112.0% | 100.1% |
| Class F | -0.02% | -0.06% | 0.02% | 100.9% | 100.1% | 105.5% | 99.9% |
| Class TGM | -0.13% | -0.02% | -0.10% | 101.0% | 99.8% | 102.3% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.10% | -0.67% | 100.1% | 99.9% | 104.6% | 100.2% |
| Class C | -0.06% | 0.06% | 0.23% | 101.9% | 100.0% | 109.9% | 100.3% |
| Class E | -0.13% | 0.02% | -0.48% | 101.5% | 100.7% | 107.5% | 100.1% |
| Overall | -0.06% | 0.07% | -0.32% | 101.1% | 100.1% | 107.0% | 100.2% |
| Class D | 0.07% | 0.40% | 0.34% | 101.9% | 100.6% | 112.7% | 100.1% |
| Class F | -0.28% | -0.62% | 0.23% | 100.9% | 100.6% | 106.6% | 100.1% |
| Class TGM | -0.19% | -0.19% | -0.12% | 100.8% | 96.5% | 103.8% | 100.2% |

Reference(s)

- Y. Ahn, J. Nam, N. Park, J. Lim, S. Kim, “AHG12: GPM mode extension,” JVET-AI0067, 35th JVET Meeting, Sapporo, JP, 12–19 July 2024.

- K. Reuzé, Y. Chen, P. Bordes, M, Le Pendu, “AHG12: Improvements on GPM partitioning splits,” JVET-AI0063, 35th JVET Meeting, Sapporo, JP, 12–19 July 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, 35th JVET Meeting, Sapporo, JP, 12–19 July 2024.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).