| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0108-v1 |
| --- | --- |

| Title: | AHG9: On signalling of instance count in generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Tan Jangwon Lee Junghak Nam Chul Keun Kim Jaehyun Lim Seung hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current design of generative face video (GFV) SEI message allows the signalling of instance count syntax element (i.e., gfv_cnt), which specifies the order of the GFV SEI messages with a particular id (i.e., gfv_id) within a picture unit (PU), to be not unique. For example, the following case is allowed: two GFV SEI messages have the same value of gfv_id and both have gfv_cnt equal to 0, in which the values of gfv_nn_base_flag are different. It is asserted that it is more desirable if the value of gfv_cnt is unique for each of gfv_id within the same PU.

This contribution proposes some modifications (i.e., proposed item 1 and item 2) that ensure gfv_cnt is unique for each gfv_id in the same PU. If the uniqueness of gfv_cnt is deemed not important, a constraint is proposed (i.e., item 3) to ensure that the GFV SEI message containing the base filter is present before the GFV SEI message containing the updated filter.

Proposed item 1: update the constraint about repetition of GFV SEI message.

- Specify that the value of instance count (i.e., syntax element gfv_cnt) is unique for a particular GFV SEI message Id within the same picture unit.

- When there are two GFV SEI messages in the same picture unit have the same Id (i.e., same value of gfv_id) and both have the same values instance count (i.e., gfv_cnt equal to 0), then the second SEI is a repetition of the first SEI (i.e., they shall have the same payload).

Proposed item 2: allow two or more GFV SEI messages to contain TranslatorNN.

- Both aspects from item 1.

- Signal the syntax element gfv_nn_present_flag without condition.

- A picture is considered base picture if the gfv_cnt is equal to 0 or gfv_nn_present_flag is equal to 0.

- Only base picture with gfv_cnt equal to 0 can have base filter.

Proposed item 3: add constraint on the presence of base and updated filters

- When two or more GFV SEI messages with the same value of gfv_id are present in the same picture unit and they all have the value of gfv_cnt equal to 0 but with different values of gfv_nn_base_flag, the first of these GFV SEI messages, in decoding order, shall have gfv_nn_base_flag equal 1.

# Problem Statement

Generative face video (GFV) SEI message is currently included in the TuC for future extension of VSEI (JVET-AI2032). The current design includes the signalling of instance count (i.e., gfv_cnt) that specifies the order of the GFV SEI messages with a particular Id (i.e., gfv_id) within a picture unit (PU). The instance count of the first GFV SEI message of a particular Id in a PU shall be 0.

It is asserted that the value of gfv_cnt for GFV SEI message of a particular Id within a PU is not unique. This instance count is supposed to be unique within the same gfv_id so that it is easy for decoder to understand that when there are two GFV SEI messages within the same PU with the same value of gfv_id and the same value of gfv_cnt, the second SEI messages must simply be a duplication / repetition of the first one and hence it may ignore it. However, this is not the case based on authors’ understanding of the text that specifies how to determine whether a GFV SEI message is a repetition of other GFV SEI message in the same PU, as it is specified as follows:

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag.

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

The above determination has a consequence such that it is possible / allowed to have two GFV SEI messages with the same values of gfv_id and the same values of gfv_cnt but the second one is not a repetition of the first one. For example, when two GFV SEI messages have the same value of gfv_id and both have gfv_cnt equal to 0, in which the values of gfv_nn_base_flag are different.

In the next section, we present 3 proposed items. Item 2 should be considered only if item 1 is agreed. Item 3 should be considered only if item 1 is not agreed.

# Proposals

## Item 1: update the constraint about repetition of GFV SEI message

We propose the following:

- Specify that the value of instance count (i.e., syntax element gfv_cnt) is unique for a particular GFV SEI message Id within the same picture unit.

- When there are two GFV SEI messages in the same picture unit have the same Id (i.e., same value of gfv_id) and both have the same values instance count (i.e., gfv_cnt equal to 0), then the second SEI is a repetition of the first SEI (i.e., they shall have the same payload).

Note that the consequence of this constraint is that only GFV SEI message with gfv_cnt equal to 0 may contain TranslatorNN filter.

The text implementation of the proposed item is as follows:

…

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag.

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

When two or more GFV SEI messages that are present in the same picture units have the same values of gfv_id and gfv_cnt, they shall have the same SEI payload content.

…

## Item 2: allow two or more GFV SEI messages to contain TranslatorNN

As noted in above, item 1 has consequence that only GFV SEI message with gfv_cnt equal to 0 may contain TranslatorNN filter, i.e., GFV SEI message has gfv_cnt equal to 0, gfv_base_pic_flag equal to 1, and gfv_nn_present_flag equal to 1. The translatorNN itself can be either a base filter or an updated one.

However, it may be argued that it is desirable to allow both base filter and updated filters to be present in two different GFV SEI messages with the same gfv_id in the same picture unit. To support that, we propose the following modifications:

- Both aspects from item 1.

- Signal the syntax element gfv_nn_present_flag without condition.

- A picture is considered base picture if the gfv_cnt is equal to 0 or gfv_nn_present_flag is equal to 0.

- Only base picture with gfv_cnt equal to 0 can have base filter.

The text implementation of the proposed item is as follows:

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| gfv_nn_present_flag | u(1) |
| if( gfv_cnt = = 0 ¦ ¦ gfv_nn_present_flag ) |  |
| gfv_base_pic_flag /* indicate if the current decoded output picture is a base picture */ | u(1) |
| if( gfv_base_pic_flag ) { /* specify TranslatorNN( ) */ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| if( gfv_cnt = = 0 ) |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_alignment_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| gfv_chroma_key_info_present_flag | u(1) |
| if( gfv_chroma_key_info_present_flag ) { |  |
| … |  |
| } |  |
| } else |  |
| gfv_drive_pic_fusion_flag /* indicate if DrivePicture is input to GenerativeNN( ) */ | u(1) |
| … |  |
| } |  |

…

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit.

The gfv_cnt of the first GFV SEI message, in decoding order, with a particular value of gfv_id within a picture unit shall be equal to 0. When gfv_cnt assigned to currGfvCnt is greater than 0, a GFV SEI message with the same gfv_id value and gfv_cnt equal to currGfvCnt − 1 shall be present in the same picture unit and precede the current GFV SEI message in decoding order.

The value of gfv_cnt shall be in the range of 0 to 65 535, inclusive.

gfv_nn_present_flag equal to 1 indicates that a neural network that may be used as a TranslatorNN( ) is contained or indicated by the SEI message. gfv_nn_present_flag equal to 0 indicates that a neural network that may be used as a TranslatorNN( ) is not contained or indicated by the SEI message.

When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN( ) is referenced in the semantics of the GFV SEI message, the following constraint applies:

–	If gfv_cnt is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise (gfv_cnt is greater than 0), there shall be at least one GFV SEI message that is present in either the current picture unit or a preceding picture unit in output order in the current CLVS and has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN( ) is referenced in the semantics of this SEI message, the following applies for deriving the applicable TranslatorNN( ):

–	If gfv_cnt is greater than 0 and there exists one or more preceding GFV SEI messages in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN( ) is defined by the last preceding GFV SEI message in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise, the applicable TranslatorNN( ) is defined by a GFV SEI message that is present in the last preceding picture unit puB in output order in the current CLVS that has the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1. When there are multiple such GFV SEI messages present in the picture unit puB that have the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN( ) is defined by the last of such GFV SEI messages in decoding order.

gfv_base_pic_flag equal to 1 indicates that the current decoded output picture corresponds to a base picture. gfv_base_pic_flag equal to 0 indicates that the current decoded output picture does not correspond to a base picture or this SEI message does not specify syntax elements for a base picture. When gfv_base_pic_flag is not present, it is inferred to be equal to 0.

When a GFV SEI message is the first GFV SEI message, in decoding order, that has a particular gfv_id value within the current CLVS, the value of gfv_base_pic_flag shall be equal to 1.

When a GFV SEI message with a particular gfv_id value has gfv_base_pic_flag equal to 1, the base picture for that particular gfv_id value, which is the current cropped decoded picture, remains valid for the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that is within the current CLVS, follows the current decoded picture in output order, and is associated with a GFV SEI message having that particular gfv_id value and gfv_base_pic_flag equal to 1, whichever is earlier.

gfv_nn_present_flag equal to 1 indicates that a neural network that may be used as a TranslatorNN( ) is contained or indicated by the SEI message. gfv_nn_present_flag equal to 0 indicates that a neural network that may be used as a TranslatorNN( ) is not contained or indicated by the SEI message. When gfv_nn_present_flag is not present, it is inferred to be 0.

When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following constraint applies:

–	If gfv_cnt is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise (gfv_cnt is greater than 0), there shall be at least one GFV SEI message that is present in either the current picture unit or a preceding picture unit in output order in the current CLVS and has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of this SEI message, the following applies for deriving the applicable TranslatorNN:

–	If gfv_cnt is greater than 0 and there exists one or more preceding GFV SEI messages in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last preceding GFV SEI message in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise, the applicable TranslatorNN is defined by a GFV SEI message that is present in the last preceding picture unit puB in output order in the current CLVS that has the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1. When there are multiple such GFV SEI messages present in the picture unit puB that have the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last of such GFV SEI messages in decoding order.

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] specify a neural network that may be used as a TranslatorNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_alignment_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_alignment_zero_bit_b, and nnpfc_payload_byte[ i ], respectively.

When the value of gfv_cnt is greater than 0 and gfv_nn_present_flag is equal to 1, the value of gfv_nn_base_flag is inferred to be equal to 0.

When a GFV SEI message with a particular value of gfv_id and gfv_cnt greater than 0 and gfv_nn_present_flag equal 1 is present in a picture unit, it is constrained that there shall the GFV SEI with that particular gfv_id and gfv_cnt equal to 0, gfv_nn_present_flag equal to 1, and gfv_nn_base_flag equal to 1 is also present in the picture unit.

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag.

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

When two or more GFV SEI messages that are present in the same picture units have the same values of gfv_id and gfv_cnt, and they shall have the same SEI payload content.

…

## Item 3: add constraint on the presence of base and updated filters

If it is more desirable to keep the current design that allows gfv_cnt to be not unique for a particular gfv_id, we proposed the following constraint:

- When two or more GFV SEI messages with the same value of gfv_id are present in the same picture unit and they all have the value of gfv_cnt equal to 0 but with different values of gfv_nn_base_flag, the first of these GFV SEI messages, in decoding order, shall have gfv_nn_base_flag equal 1.

The text implementation of the proposed item is as follows:

…

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag.

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

It is constrained that when two or more GFV SEI messages with the same value of gfv_id are present in the same picture unit and they all have the value of gfv_cnt equal to 0 but with different values of gfv_nn_base_flag, the first of these GFV SEI messages, in decoding order, shall have gfv_nn_base_flag equal 1.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).