| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0111-v1 |
| --- | --- |

| Title: | AHG9: On the presence of translator NN filter in GFV SEI message in generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Tan Junghak Nam Jangwon Lee Chul Keun Kim Jaehyun Lim Seung hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current design of signalling gfv_nn_present_flag and gfv_nn_base_flag in generative face video (GFV) SEI message has the following problems:

- It does not have any constraint that requires the presence of the base filter. While it is constrained that when a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1, this filter is not required to be a base filter. In such case, it is not clear how to use such filter.

- While there is text to specify whether a reference neural network filter shall be present in the case the current GFV SEI message does not contain one, there is no text to specify where the base filter shall be present in the case the current GFV SEI message contains an updated filter.

- When gfv_nn_present_flag is equal to 0, the referenced TranslatorNN is chosen regardless whether that filter is base or update filter. It would be desirable to allow the choice of using base filter or update filter.

This contribution proposes the following modifications to address the above asserted problems:

Proposal 1: Update the existing constraint such that when a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present, equal to 1 and the value of gfv_nn_base_flag shall be equal to 1.

Proposal 2: Choose one of the following options:

Option 1: Add a constraint that specifies that when two GFV SEI messages of the same id contain translator NN filter and the value of gfv_nn_base_flag are both equal to 1, the two SEI contains the same / identical translator NN filter.

Option 2: Add a constraint that specifies that when a GFV SEI message with gfv_nn_present_flag equal to 1 and gfv_nn_base_flag is equal to 0, the reference base filter for the GFV SEI message is the first GFV SEI message with the same gfv_id and gfv_nn_base_flag equal to 1 in the picture unit that precedes the current picture unit in output order.

Proposal 3: Move the signalling of gfv_nn_base_flag to be unconditionally present. When gfv_nn_present_flag is equal to 0 then the GFV SEI message uses NN from previous GFV SEI message that contains NN and with the same value of gfv_nn_base_flag.

# Introduction

Generative face video (GFV) SEI message is currently included in the TuC for future extension of VSEI (JVET-AI2032). The current design includes the signalling of translator neural network which can be present in the GFV SEI message itself, in other location specified by URI, or in other GFV SEI message with the same Id. The following is the semantic of gfv_nn_present_flag:

gfv_nn_present_flag equal to 1 indicates that a neural network that may be used as a TranslatorNN( ) is contained or indicated by the SEI message. gfv_nn_present_flag equal to 0 indicates that a neural network that may be used as a TranslatorNN( ) is not contained or indicated by the SEI message. When gfv_nn_present_flag is not present, it is inferred to be 0.

When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following constraint applies:

–	If gfv_cnt is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise (gfv_cnt is greater than 0), there shall be at least one GFV SEI message that is present in either the current picture unit or a preceding picture unit in output order in the current CLVS and has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of this SEI message, the following applies for deriving the applicable TranslatorNN:

–	If gfv_cnt is greater than 0 and there exists one or more preceding GFV SEI messages in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last preceding GFV SEI message in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise, the applicable TranslatorNN is defined by a GFV SEI message that is present in the last preceding picture unit puB in output order in the current CLVS that has the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1. When there are multiple such GFV SEI messages present in the picture unit puB that have the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1, the applicable TranslatorNN is defined by the last of such GFV SEI messages in decoding order.

Futhermore, the neural network itself can be a base neural network or an update neural network that refers to the base neural network. The semantics of gfv_nn_base_flag is as follows:

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] specify a neural network that may be used as a TranslatorNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_alignment_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_alignment_zero_bit_b, and nnpfc_payload_byte[ i ], respectively.

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag.

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

Based on authors’ analysis of the design, the signalling of gfv_nn_preset_flag and gfv_nn_base_flag may be several problems that need to be addressed.

# Problem 1 and proposed solution

## Problem description

In our assertion, the current design of gfv_nn_present_flag and gfv_nn_base_flag has the following problems:

- It does not have any constraint that requires the presence of the base filter. While it is constrained that when a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1, this filter is not required to be a base filter. In such case, it is not clear how to use such filter.

- While there text to specify whether the neural network filter shall be present in the case the current GFV SEI message does not contain one, there is no text to specify where the base filter shall be present in the case the current GFV SEI message contains an updated filter.

## Proposed solution

Proposal 1: Update the existing constrint such that when a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present, equal to 1 and the value of gfv_nn_base_flag shall be equal to 1.

Proposal 2: Choose one of the following options:

Option 1: Add a constraint that specifies that when two GFV SEI messages of the same id contain translator NN filter and the value of gfv_nn_base_flag are both equal to 1, the two SEI contains the same / identical translator NN filter.

Option 2: Add a constraint that specifies that when a GFV SEI message with gfv_nn_present_flag equal to 1 and gfv_nn_base_flag is equal to 0, the reference base filter for the GFV SEI message is the first GFV SEI message with the same gfv_id and gfv_nn_base_flag equal to 1 in the picture unit that precedes the current picture unit in output order.

# Problem 2 and proposed solution

## Problem description

In the current design, when gfv_nn_present_flag is equal to 0, the referenced TranslatorNN is chosen from the filter available regardless whether that filter is base or update filter. It would be desirable to allow the choice of using base filter or update filter.

## Proposed solution

Proposal 3: Move the signalling of gfv_nn_base_flag to be unconditionally present. When gfv_nn_present_flag is equal to 0 then the GFV SEI message uses NN from previous GFV SEI message that contains NN and with the same value of gfv_nn_base_flag.

# Text implementation

## For proposal 1

…

When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1 and gfv_nn_base_flag shall be equal to 1.

…

When gfv_nn_present_flag is equal to 1 and gfv_base_flag is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message, gfv_nn_present_flag equal to 1, and gfv_nn_base_flag equal to 1.

…

## For proposal 2 – Option 1

…

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] specify a neural network that may be used as a TranslatorNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_alignment_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_alignment_zero_bit_b, and nnpfc_payload_byte[ i ], respectively.

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag.

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

When a GFV SEI message gfv_a with gfv_nn_present_flag equal to 1 and gfv_nn_base_flag equal to 1 and another GFV SEI message gfv_b with gfv_nn_present_flag equal to 1 and gfv_nn_base_flag equal to 1, gfv_a and gfv_b have the same gfv_id, and they are present in different picture units, the translator NN filters in the two SEI messages shall be indentical.

…

## For proposal 2 – Option 2

…

When a GFV SEI message with gfv_nn_present_flag equal to 1 and gfv_nn_base_flag equal to 0, the reference base filter for the GFV SEI message is the first GFV SEI message with the same gfv_id and gfv_nn_base_flag equal to 1 in the picture unit that precedes the current picture unit in output order.

...

## For proposal 3

…

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| gfv_nn_base_flag | u(1) |
| if( gfv_cnt = = 0 ) |  |
| gfv_base_pic_flag /* indicate if the current decoded output picture is a base picture */ | u(1) |
| if( gfv_base_pic_flag ) { /* specify TranslatorNN( ) */ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_alignment_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| gfv_chroma_key_info_present_flag | u(1) |
| if( gfv_chroma_key_info_present_flag ) { |  |
| … |  |
| } |  |
| } else |  |
| gfv_drive_pic_fusion_flag /* indicate if DrivePicture is input to GenerativeNN( ) */ | u(1) |
| … |  |
| } |  |

…

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit.

The gfv_cnt of the first GFV SEI message, in decoding order, with a particular value of gfv_id within a picture unit shall be equal to 0. When gfv_cnt assigned to currGfvCnt is greater than 0, a GFV SEI message with the same gfv_id value and gfv_cnt equal to currGfvCnt − 1 shall be present in the same picture unit and precede the current GFV SEI message in decoding order.

The value of gfv_cnt shall be in the range of 0 to 65 535, inclusive.

gfv_nn_base_flag equal to 1 specifies that TranslatorNN( ) used by this SEI is a base neural-network filter. gfv_nn_base_flag equal to 0 specifies that TranslatorNN( ) used by this SEI is a not a base neural-network filter.

…

gfv_nn_present_flag equal to 1 indicates that a neural network that may be used as a TranslatorNN( ) is contained or indicated by the SEI message. gfv_nn_present_flag equal to 0 indicates that a neural network that may be used as a TranslatorNN( ) is not contained or indicated by the SEI message. When gfv_nn_present_flag is not present, it is inferred to be 0.

When a GFV SEI message with gfv_cnt equal to 0 is present in the first picture unit in a CLVS in decoding order, gfv_nn_present_flag shall be present and equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of the GFV SEI message, the following constraint applies:

–	If gfv_cnt is equal to 0, there shall be at least one GFV SEI message present in a preceding picture unit in output order in the current CLVS and having the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

–	Otherwise (gfv_cnt is greater than 0), there shall be at least one GFV SEI message that is present in either the current picture unit or a preceding picture unit in output order in the current CLVS and has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of this SEI message, the following applies for deriving the applicable TranslatorNN:

–	If gfv_cnt is greater than 0 and there exists one or more preceding GFV SEI messages in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1 and has the same value of gfv_nn_base_flag as that in the current GFV SEI message, the applicable TranslatorNN is defined by the last preceding GFV SEI message in decoding order in the current picture unit that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1 and has the same value of gfv_nn_base_flag as that in the current GFV SEI message.

–	Otherwise, the applicable TranslatorNN is defined by a GFV SEI message that is present in the last preceding picture unit puB in output order in the current CLVS that has the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1 and has the same value of gfv_nn_base_flag as that in the current GFV SEI message. When there are multiple such GFV SEI messages present in the picture unit puB that have the same value of gfv_id as the current GFV SEI message and gfv_nn_present_flag equal to 1 and has the same value of gfv_nn_base_flag as that in the current GFV SEI message, the applicable TranslatorNN is defined by the last of such GFV SEI messages in decoding order.

…

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] specify a neural network that may be used as a TranslatorNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_alignment_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_alignment_zero_bit_b, and gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_alignment_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_alignment_zero_bit_b, and nnpfc_payload_byte[ i ], respectively.

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag.

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).