| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0115-v1 |
| --- | --- |

| Title: | EE2-3.11: Regression-based GPM extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ke Jia Jie Chen Xinwei Li Ru-Ling Liao Yan Ye | Tel: Email: | {feijia.jk, jiechen.cj, sid.lxw, ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2 Test 3.11, which studies template matching (TM) and merge with motion vector difference (MMVD) extensions to regression-based GPM. In Test 3.11a, only TM extension is applied to regression-based GPM. In Test 3.11b, both TM and MMVD extensions are applied to regression-based GPM. It is reported that on top of ECM-14.0, the overall coding performance for {Y, U, V, EncT, DecT} is :

- Test 3.11a: Regression-based GPM with TM

  - {xx} in RA

  - {-0.10%, 0.01%, -0.20%, 101.4%, 99.8%} in LDB

- Test 3.11b: Regression-based GPM with TM and MMVD

  - {-0.08%, -0.15%, -0.12%, 101.6%, 100.0%} in RA

  - {-0.12%, 0.01%, -0.37%, 103.9%, 99.7%} in LDB

Introduction

In ECM-14.0, template matching (TM) based motion vector refinement and merge with motion vector difference (MMVD) was applied to regular geometric partitioning mode (GPM). For GPM-TM, a CU-level flag is signaled to jointly indicate whether the TM is used for the two partitions. That is, when the flag is on, the motion vectors of both two partitions are refined by TM; otherwise, none of the motion vectors are refined. For GPM-MMVD, two partition-level flags are signaled to separately indicate whether the MMVD is used for each partition. When the flag indicates MMVD is used for a partition, the index of the MVD selected by the encoder is further signaled. Both TM and MMVD can improve the motion accuracy for GPM and thus give coding performance increase.

However, in the ECM-14.0, TM and MMVD is only applied to regular GPM, but not applied to regression-based GPM. In order to improve coding efficiency, it is proposed to extend the regression-based GPM by applying TM and MMVD in the last meeting [1]. This contribution reports the simulation results of EE2 Test 3.11 [2].

Proposed method

In Test 3.11a, only TM extension is applied. When regression-based GPM is used for the current block, a TM flag is signaled to indicate whether the motion vector is to be refined by TM. If the TM flag is true, the motion vectors of both partitions are refined. The template used in the TM process consists of above and left reconstructed samples (AL shape). The TM search process is the same as that of regular GPM-TM.

In Test 3.11b, both TM and MMVD extensions are applied. The TM extension is same as Test 3.11a. When TM flag is true, the motion vectors of both partitions are refined by TM. When TM flag is false, an MMVD flag is signaled to indicate whether the motion vectors of partitions are refined by the MVD. If the MMVD flag is true, an MVD index is signaled to indicate the MVD. In order to reduce the encoder complexity of MMVD extension, the number of MVD candidates is reduced to 40.

Simulation results

The proposed methods are implemented on top of ECM-14.0 and tested under CTC for enhanced compression tool testing [3]. The results are shown in the following tables.

Table 1 Simulation results of Test 3.11a

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.09% | -0.27% | -0.02% | 100.1% | 99.7% | 100.0% | 100.0% |
| Class C | -0.06% | -0.05% | 0.00% | 100.8% | 99.7% | 100.1% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.08% | -0.20% | -0.13% | 99.9% | 99.6% | 100.2% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.02% | -0.12% | 101.9% | 100.9% | 100.1% | 99.9% |
| Class C | -0.04% | -0.19% | -0.14% | 101.7% | 99.9% | 99.2% | 99.9% |
| Class E | -0.17% | 0.34% | -0.40% | 100.3% | 97.8% | 100.0% | 99.8% |
| Overall | -0.10% | 0.01% | -0.20% | 101.4% | 99.8% | 99.8% | 99.9% |
| Class D | 0.09% | 0.87% | 0.47% | 102.2% | 100.3% | 100.4% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 2 Simulation results of Test 3.11b

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.19% | -0.02% | 101.3% | 100.6% | 100.3% | 100.3% |
| Class A2 | -0.04% | -0.03% | -0.21% | 101.5% | 100.1% | 100.1% | 99.9% |
| Class B | -0.11% | -0.25% | -0.12% | 101.4% | 99.8% | 100.3% | 99.8% |
| Class C | -0.07% | -0.07% | -0.12% | 102.2% | 99.6% | 100.7% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.15% | -0.12% | 101.6% | 100.0% | 100.4% | 100.0% |
| Class D | -0.03% | -0.04% | -0.19% | 101.5% | 99.8% | 100.9% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | 0.11% | -0.27% | 104.1% | 100.6% | 100.2% | 99.9% |
| Class C | -0.11% | -0.05% | 0.07% | 104.6% | 99.6% | 100.8% | 99.9% |
| Class E | -0.16% | -0.10% | -1.11% | 102.8% | 98.4% | 99.9% | 100.3% |
| Overall | -0.12% | 0.01% | -0.37% | 103.9% | 99.7% | 100.3% | 100.0% |
| Class D | 0.04% | 0.85% | 0.42% | 103.3% | 100.0% | 100.6% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Conclusion

In this contribution, the simulation results of EE2-3.11 are reported. Test 3.11a achieves xx gain and -0.10% gain in RA and LDB respectively, and test 3.11b achieves -0.08% gain and -0.12% gain in RA and LDB respectively. Neither of the tests have much run time impact. Thus, it is recommended to adopt one of tests into the next version of ECM.

References

- K. Jia, J. Chen, X. Li, R.-L. Liao, Y. Ye, “Non-EE2: Regression-based GPM extension”, JVET-AI0115, Jul. 2024.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AI2024, Aug. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Aug. 2024.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).