| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0117-v3 |
| --- | --- |

| Title: | Non-EE2: Regression-based GPM with intra and inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ke Jia Jie Chen Xinwei Li Ru-Ling Liao Yan Ye | Tel: Email: | {feijia.jk, jiechen.cj, sid.lxw, ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to extend the regression-based GPM by adding a new mode in which one partition is inter coded and the other one is intra coded. The proposed extension was implemented on top of ECM14.0, and it is reported that the overall performance impact is {-0.05%, -0.01%, -0.02%, 101.9%, 100.7%} in random access.

In v2, the full results were provided.

In v3, a code bug which was reported by the crosschecker during the meeting time was fixed and updated experimental results with the bug fix was added.

Introduction

In ECM-14.0, intra and inter prediction can be respectively applied to the two partitions of a coding block in geometric partitioning mode (intra-inter GPM) [1]. When GPM is used, a first flag is signaled to indicate whether the first partition is intra. If the first flag is false, a second flag is signaled to indicate whether the second partition is intra. If the first flag is true, the second partition must be inter predicted and the signalling of the second flag is skipped. When the intra-inter GPM is used, the predictor block is generated by weighting inter predicted samples and intra predicted samples. For inter predicted partition, GPM-TM and GPM-MMVD can be applied. For intra predicted partition, the intra prediction mode candidates are derived by using DIMD/TIMD method or derived based on the GPM split mode. At most, three intra prediction mode candidates can be derived. Besides the regular GPM, ECM also adopted regression-based GPM [2], in which the two integer blending matrices (W0 and W1) are derived with the following equations:

W0(x,y) = a.x + b.y + c  		and 	W1(x,y) = 1 - W0(x,y)

The parameters (a,b,c) are derived by solving MSE minimization problem on the template (one top row and one left column).

However, each of partitions in regression-based GPM can only be inter predicted in ECM-14.0. In order to improve coding efficiency, it is proposed to extend the regression-based GPM to support the case that two partitions are respectively predicted with intra and inter.

Proposed method

It is proposed to extend the regression-based GPM to support the case that two partitions are respectively predicted with intra and inter.

When regression-based GPM is used for the current block, a CU level flag is signaled to indicate whether to use intra-inter prediction. The intra prediction mode candidates are the first 6 modes in the most probable modes (MPM) list and the MV candidates for the inter prediction are from regular GPM MV candidate list. Then, an intra-inter pair list is constructed by combining these intra prediction mode candidates and MV candidates. For each pair in the intra-inter pair list, the two integer blending matrices (W0 and W1) are derived. The matrix derivation method is the same as that of the current inter-inter regression-base GPM. The only difference is that the predictor of the template is generated by blending intra predicted samples and inter predicted samples. Then the intra-inter pair list is re-ordered with the template cost, and the index of the selected intra-inter pair is signalled. For the inter prediction partition, overlapped block motion compensation (OBMC) and luma mapping with chroma scaling (LMCS) are performed after motion compensation. The final predicted samples are generated by weighting inter predicted samples and intra predicted sample with W0 and W1 matrices.

The regression-based GPM with intra and inter prediction is only enabled in RA.

Simulation results

The proposed methods are implemented on top of ECM-14.0 and tested under CTC for enhanced compression tool testing [2]. The results are shown in the following table.

Table 1 Simulation results of regression-based GPM with intra and inter prediction

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.06% | -0.07% | 102.6% | 102.2% | 100.0% | 101.3% |
| Class A2 | 0.00% | 0.12% | -0.03% | 102.4% | 101.1% | 99.7% | 102.1% |
| Class B | -0.08% | -0.09% | -0.03% | 101.4% | 99.9% | 100.0% | 104.8% |
| Class C | -0.07% | 0.02% | 0.02% | 101.6% | 100.4% | 99.7% | 109.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.01% | -0.02% | 101.9% | 100.7% | 99.9% | 104.6% |
| Class D | -0.04% | -0.10% | -0.03% | 101.1% | 101.2% | 100.2% | 112.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

During the meeting time, a variable initialization bug in the source codes was reported by the crosschecker and we fixed it. The simulation was re-launched with fixed codes and the coding results are provided in the following table.  It is observed that by fixing the reported bug, the coding gain of class D and class C slightly increased.

Table 2 Simulation results with bug fix

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.08% | -0.09% | 0.01% | 102.0% | 100.3% | -0.07% | -0.08% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | -0.13% | -0.04% | 101.3% | 100.1% | -0.05% | -0.13% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Conclusion

In this contribution, the regression-based GPM with intra and inter prediction is proposed. Overall, it gives {-0.05%, -0.01%, -0.02%, 101.9%, 100.7%} in RA. It is recommended to study the proposed method in the next round of EE2.

References

- Y. Kidani, H. Kato, K. Kawamura, H. Jang, S. Kim, J. Lim, Z. Deng, K. Zhang and L. Zhang, “EE2-3.1: GPM with inter and intra prediction (JVET-X0166)”, JVET-Y0065, Jan. 2022.

- P. Bordes, K. Reuzé, F. Galpin, F. Urban, K. Naser, F. Le Leannec and E. Francois, “EE2-2.11: Regression-based GPM blending (tests a,b,c)”, JVET-AG0112, Jan. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Aug. 2024.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).