| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0121-v1 |
| --- | --- |

| Title: | AHG11: Report of retraining LOP3 with new sequences from JVET-AH0120 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten | Email: | {du.liu, jacob.strom, mitra.damghanian, per.wennersten}@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the performance of retraining LOP3 with the new sequences proposed in JVET-AH0120. The training dataset includes the current LOP training set and the additional dataset from JVET-AH0120. Two tests were performed, and the results are reported as below.

Test 1: Training LOP3 with all the sequences from JVET-AH0120.

Run 1, float model vs. NNVC-10 float: RA {0.05%, 3.75%, 3.89%}, AI {0.03%, 3.60%, 3.94%}

Run 2, float model vs. NNVC-10 float: RA {0.25%, 1.71%, 2.32%}, AI {0.06%, 1.56%, 1.99%}

Test 2: Training LOP3 with the non-CG sequences from JVET-AH0120.

Float model vs. NNVC-10 float: RA {0.07%, -0.74%, -0.88%}, AI {0.05%, -1.03%, -1.29%}

Int16 model vs. NNVC-10 int16: RA {0.07%, -0.74%, -1.32%}, AI {0.05%, -1.10%, -1.18%}

# Introduction

At the Rennes meeting, new training sequences were proposed in JVET-AH0120 [1]. It was suggested at the NNVC BOG to study the training impact using the new sequences [2].

# Training and inference

In this contribution, the impact of training with the new dataset was studied. The tested training dataset includes the current LOP2 dataset and the new dataset from JVET-AH0120. The training dataset extracted from JVET-AH0120 follows the same procedure as the extraction of the LOP2 dataset, where the same encoder, decoder, and configuration were used. A bug has been fixed since last meeting and new datasets have been generated.

JVET-AH0120 includes 9 sequences, where 2 of them are CG sequences. The full dataset from JVET-AH0120 has a size of 35 GB, and the size is 27 GB excluding the CG sequences. The current LOP2 dataset has 4.0 TB. Thus, in terms of size, the new dataset adds a small portion of data to the existing dataset. The validation dataset remains the same as before.

The LOP3 model is retrained using the new dataset only for stage 3. The batch size, epochs, and milestones are the same as what has been used in LOP3. Each training has 60 epochs and the milestones for switching the learning rate are 41, 51, and 55. At epoch 58 the loss function switches from L1 loss to MSE loss.

Two tests were performed:

Test 1: Training LOP3 with the full dataset from JVET-AH0120.

Test 2: Training LOP3 with the non-CG sequences from JVET-AH0120.

Inference uses NNVC-10.0 with NN-Intra enabled. The results are in comparison to the NNVC-10 (NN-Intra + LOP3). The encoding time and decoding time are not reliable since they are not run on the same machine.

## Test 1 Training LOP3 with the full dataset from JVET-AH0120

Two runs of trainings were carried out. The float model was evaluated and compared to NNVC-10 float (NN-Intra + LOP3).

Run 1:

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.11% | 2.76% | 4.90% | 98% | 99% |
| Class A2 | 0.04% | 3.26% | 2.56% | 130% | 192% |
| Class B | 0.03% | 4.02% | 3.76% | 92% | 61% |
| Class C | 0.04% | 4.51% | 4.30% | 95% | 59% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 3.75% | 3.89% | 101% | 83% |
| Class D | -0.01% | 4.66% | 4.96% | 96% | 72% |
| Class F | -0.07% | 2.58% | 1.77% | 92% | 71% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-10.0 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.01% | 3.50% | 4.80% | 124% | 284% |
| Class A2 | 0.07% | 3.20% | 2.63% | 113% | 323% |
| Class B | 0.03% | 3.26% | 3.98% | 95% | 137% |
| Class C | 0.05% | 3.77% | 3.91% | 107% | 188% |
| Class E | -0.01% | 4.42% | 4.36% | 100% | 162% |
| Overall | 0.03% | 3.60% | 3.94% | 106% | 197% |
| Class D | 0.07% | 3.90% | 4.09% | 104% | 153% |
| Class F | -0.07% | 2.93% | 2.80% | 90% | 91% |

Run2:

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.18% | 1.23% | 3.34% | 99% | 99% |
| Class A2 | 0.27% | 1.01% | 1.12% | 142% | 158% |
| Class B | 0.27% | 1.90% | 2.41% | 106% | 116% |
| Class C | 0.26% | 2.37% | 2.33% | 133% | 138% |
| Class E |  |  |  |  |  |
| Overall | 0.25% | 1.71% | 2.32% | 118% | 125% |
| Class D | 0.30% | 2.17% | 2.89% | 142% | 121% |
| Class F | 0.21% | 1.52% | 1.70% | 116% | 107% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-10.0 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.04% | 1.83% | 2.33% | 140% | 322% |
| Class A2 | 0.07% | 1.14% | 1.23% | 123% | 280% |
| Class B | 0.04% | 1.52% | 1.93% | 111% | 206% |
| Class C | 0.09% | 1.81% | 2.34% | 144% | 390% |
| Class E | 0.05% | 1.43% | 2.02% | 127% | 353% |
| Overall | 0.06% | 1.56% | 1.99% | 127% | 295% |
| Class D | 0.07% | 1.58% | 2.24% | 146% | 336% |
| Class F | 0.07% | 1.51% | 1.98% | 94% | 132% |

## Test 2 Training LOP3 with the non-CG sequences from JVET-AH0120

The best model was selected from five trainings and evaluated. Both float model and int16 models were evaluated. Quantization used the LOP3 quantization script.

Float:

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.10% | -0.72% | -1.09% | 90% | 66% |
| Class A2 | 0.07% | -1.59% | -0.61% | 108% | 69% |
| Class B | 0.05% | -0.64% | -1.11% | 106% | 97% |
| Class C | 0.08% | -0.24% | -0.64% | 98% | 83% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -0.74% | -0.88% | 101% | 81% |
| Class D | -0.07% | -0.84% | -0.54% | 111% | 83% |
| Class F | 0.15% | -0.80% | -0.25% | 98% | 71% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-10.0 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.05% | -0.96% | -1.24% | 128% | 216% |
| Class A2 | 0.05% | -1.41% | -0.74% | 125% | 221% |
| Class B | 0.02% | -1.10% | -1.43% | 116% | 184% |
| Class C | 0.10% | -0.76% | -1.06% | 135% | 317% |
| Class E | 0.01% | -0.95% | -1.95% | 131% | 295% |
| Overall | 0.05% | -1.03% | -1.29% | 126% | 238% |
| Class D | 0.07% | -0.86% | -1.00% | 130% | 298% |
| Class F | 0.02% | -1.16% | 0.23% | 122% | 166% |

Int16:

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 int16 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.13% | -0.41% | -1.35% | 117% | 245% |
| Class A2 | 0.09% | -1.68% | -0.73% | 117% | 226% |
| Class B | 0.02% | -0.80% | -1.91% | 125% | 252% |
| Class C | 0.08% | -0.21% | -0.99% | 116% | 244% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -0.74% | -1.32% | 119% | 243% |
| Class D | 0.05% | -0.66% | -0.90% | 112% | 237% |
| Class F | 0.16% | -0.88% | -0.21% | 127% | 235% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-10.0 int16 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.07% | -1.10% | -0.68% | 121% | 292% |
| Class A2 | 0.06% | -1.58% | -0.68% | 115% | 243% |
| Class B | 0.04% | -1.13% | -1.45% | 118% | 290% |
| Class C | 0.08% | -0.79% | -1.07% | 113% | 264% |
| Class E | 0.02% | -0.99% | -1.90% | 110% | 289% |
| Overall | 0.05% | -1.10% | -1.18% | 115% | 276% |
| Class D | 0.04% | -0.75% | -0.95% | 114% | 269% |
| Class F | 0.08% | -1.21% | 0.31% | 115% | 279% |

# Conclusions

It is observed that Test 1 does not show clear benefit for Class F nor for the overall figure. Test 2 using non-CG sequences provides better performance than Test 1. Comparing to NNVC-10, Test 2 shows minor luma loss and minor chroma gain.

# References

[1] AHG11 Response to Call for training materials JVET-AG2036, JVET-AH0120, F. Galpin, R. Gendrot, E. Francois, V. Allie (InterDigital), Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024.

[2] BoG on NNVC training materials, JVET-AH0354, E. Alshina, F. Galpin, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024.

[3] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-AF2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).