| JOINT Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0126-v2 |
| --- | --- |

| Title: | EE2: Enhanced inter AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Lei Zhao1, Yang Wang1, Na Zhang1, Wenbin Yin1, Mehdi Salehifar2, Li Zhang2 1 No. 27 North Third Ring West Road, Haidian District, Beijing Fangheng Fashion Center, 100098, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-3.2 which was proposed in JVET-AI0126. Two tests are conducted. In Test 3.2a, the affine AMVP candidate list is enhanced based on template matching and non-adjacent candidates. On top of Test 3.2a, Test 3.2b further checks additional non-adjacent candidates for the regular inter AMVP candidate list. The impact on coding efficiency and runtime of the proposed method over ECM-14.0 are reported for {Y, U, V, EncT, DecT} as below:

Test 3.2a:

RA: { -0.01%, -0.03%, -0.02%, 100.2%, 100.2%  }

LDB: { -0.14%, 0.04%, -0.38%, 100.9%, 100.1% }

Test 3.2b:

RA: { -0.05%, -0.08%, -0.01%, 100.5%, 100.2% }

LDB: { -0.14%, 0.04%, -0.38%, 100.9%, 100.1% }

# Introduction

In ECM-14.0, the regular inter AMVP candidate list is constructed by checking spatial adjacent candidates, temporal collocated candidates, spatial non-adjacent candidates, and HMVP candidates. The list comprises 5 candidates which are refined and reordered based on template matching, and the candidate with the minimal template cost is selected as the MVP. On the other hand, the affine AMVP candidate list contains 2 candidates without refinement and reordering, and the index of the selected affine MVP candidate is signalled in the bitstream.

In JVET-AI0126 [1], it was proposed to enhance the regular AMVP list and affine AMVP list through template matching and additional non-adjacent candidates. The methods were included into EE2 for further investigation.

# Proposed method

Test 3.2a enhances the affine AMVP list construction. Template matching is used to refine and reorder the affine AMVP candidates. An initial list with up to 10 candidates is built. Besides the existing candidates in the current ECM, additional non-adjacent affine candidates in the target reference picture are checked if the current list is not full. The positions of the non-adjacent candidates in the reference picture are the same as the positions of the temporal candidates in the affine merge mode. The first 2 candidates with lowest template costs are selected to construct the final affine AMVP list, and the index of the selected affine MVP candidate is signalled in the bitstream.

On top of Test 3.2a, Test 3.2b further checks additional non-adjacent spatial candidates for the regular inter AMVP candidate list construction. An initial list with up to 10 candidates is built. Same as the current ECM, the candidate with the minimal template cost is selected as the MVP. For simplicity, the changes to the regular inter AMVP candidates are applied to non-low-delay pictures only.

# Experimental results

The proposed methods were implemented on top of ECM-14.0 and tested following the common test conditions [2]. The simulation results of the proposed methods compared to ECM-14.0 are summarized in the following tables.

Table 1: Coding performance of Test 3.2a

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.01% | -0.08% | 100.3% | 99.9% | 99.7% | 100.3% |
| Class A2 | -0.01% | 0.01% | -0.13% | 100.6% | 100.8% | 100.4% | 100.1% |
| Class B | 0.00% | -0.09% | 0.00% | 100.2% | 100.4% | 100.1% | 99.8% |
| Class C | -0.03% | -0.03% | 0.08% | 99.9% | 99.8% | 100.4% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.02% | 100.2% | 100.2% | 100.1% | 100.1% |
| Class D | -0.04% | -0.18% | 0.05% | 99.3% | 100.0% | 99.9% | 100.0% |
| Class F | 0.07% | 0.01% | 0.01% | 99.6% | 99.7% | 99.9% | 99.9% |
| Class TGM | -0.06% | -0.02% | -0.08% | 100.4% | 100.5% | 100.3% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | 0.10% | -0.29% | 101.0% | 100.2% | 100.1% | 99.7% |
| Class C | -0.14% | 0.01% | -0.16% | 101.1% | 99.8% | 100.0% | 100.0% |
| Class E | -0.18% | 0.00% | -0.82% | 100.6% | 100.4% | 99.1% | 99.7% |
| Overall | -0.14% | 0.04% | -0.38% | 100.9% | 100.1% | 99.8% | 99.8% |
| Class D | -0.27% | 0.45% | 0.32% | 101.0% | 100.7% | 100.1% | 100.0% |
| Class F | -0.02% | -0.24% | 0.39% | 101.2% | 100.9% | 100.5% | 99.9% |
| Class TGM | -0.17% | -0.14% | -0.12% | 100.4% | 100.5% | 100.0% | 99.9% |

Table 2: Coding performance of Test 3.2b

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.32% | -0.20% | 100.1% | 100.5% | 100.0% | 100.1% |
| Class A2 | -0.08% | -0.04% | -0.08% | 100.2% | 100.4% | 99.9% | 100.2% |
| Class B | -0.06% | -0.09% | 0.01% | 101.0% | 100.1% | 100.1% | 99.9% |
| Class C | -0.06% | 0.10% | 0.15% | 100.4% | 99.8% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.08% | -0.01% | 100.5% | 100.2% | 100.0% | 100.0% |
| Class D | -0.06% | -0.12% | 0.00% | 100.5% | 100.1% | 100.0% | 100.1% |
| Class F | -0.02% | -0.04% | -0.06% | 100.6% | 100.4% | 100.0% | 100.1% |
| Class TGM | -0.08% | -0.07% | -0.13% | 100.7% | 100.9% | 99.7% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | 0.10% | -0.29% | 101.0% | 100.2% | 100.1% | 99.7% |
| Class C | -0.14% | 0.01% | -0.16% | 101.1% | 99.8% | 100.0% | 100.0% |
| Class E | -0.18% | 0.00% | -0.82% | 100.6% | 100.4% | 99.1% | 99.7% |
| Overall | -0.14% | 0.04% | -0.38% | 100.9% | 100.1% | 99.8% | 99.8% |
| Class D | -0.27% | 0.45% | 0.32% | 101.0% | 100.7% | 100.1% | 100.0% |
| Class F | -0.02% | -0.24% | 0.39% | 101.2% | 100.9% | 100.5% | 99.9% |
| Class TGM | -0.17% | -0.14% | -0.12% | 100.4% | 100.5% | 100.0% | 99.9% |

# Conclusions

This contribution reports results of EE2-3.2 which proposes enhancements to the inter AMVP mode. The results show that the gain of the inter AMVP mode can be further improved. It is suggested to adopt the proposed method to ECM.

# References

[1] Z. Deng, K. Zhang, N. Zhang, Y. Wang, W. Yin, L. Zhao, L. Zhang, “Non-EE2: Enhanced inter AMVP”, JVET-AI0126, Jul. 2024.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AI2017, Jul. 2024.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).