| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0127-v1 |
| --- | --- |

| Title: | AHG9: SPO SEI messages in multi-layer bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Jill Boyce Nokia Technologies | Email: | miska.hannuksela@nokia.com jill.boyce@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The SEI processing order (SPO) SEI message presently applies across all layers of a multi-layer bitstream. It is asserted that different output layer sets and different layers may require different processing chains. Therefore, the contribution proposes to allow specifying processing chains that are specific to an output layer set (OLS) or a layer as follows:

- It is proposed to allow inclusion of an SPO SEI message in a scalable nesting SEI message as follows:

- When there is an SPO SEI message contained in a scalable nesting SEI message with sn_ols_flag equal to 1 for the OLS that was decoded, the decoding system may choose to apply the processing chain of that SPO SEI message to the cropped decoded pictures of that OLS.

- When there is an SPO SEI message contained in a scalable nesting SEI message with sn_ols_flag equal to 0 for any output layer of the OLS that was decoded, the decoding system may choose to apply the processing chain of that SPO SEI message to the cropped decoded pictures of that output layer.

- It is proposed to disallow subpicture-specific scalable nesting of an SPO SEI message, the motivation being simplification of the handling of the processing chain.

The specification text is proposed to VVC, since the scalable nesting SEI message is specified in VVC.

- It is proposed to move such content of VSEI clause 8.30.3 that concerns selection of the processing chain and derivation of PoPicList to VVC clause D.12.13. The motivation is to include all options to select the processing chain and derive PoPicList in a single clause.

# Problem Statement

This contribution is related to the use of SEI processing order SEI messages in multi-layer bitstreams, which has not been considered earlier as far as the authors are aware. Multi-layer bitstreams are asserted to have at least the following additional needs in comparison to single-layer bitstreams:

- Different output layer sets may require different processing chains.

For example, an output layer set that contains both a base layer and a spatial enhancement layer may not benefit from a spatial resolution upsampling NNPF, whereas an output layer set that contains only the base layer may take advantage of a spatial resolution upsampling NNPF. In both cases, the processing chains may also have additional stages, such as film grain synthesis, which may or may not differ between output layer sets.

- Different layers may require different processing chains.

For example, a texture view may be post-processed differently than a depth view. Processing of a texture view could include colour transform and film grain synthesis, and a depth view could be enhanced with a different NNPF than what is used for enhancing a texture view.

# Overview of the proposed changes

The contribution includes the following proposed aspects:

- It is proposed to allow inclusion of an SPO SEI message in a scalable nesting SEI message as follows:

- When there is an SPO SEI message contained in a scalable nesting SEI message with sn_ols_flag equal to 1 for the OLS that was decoded, the decoding system may choose to apply the processing chain of that SPO SEI message to the cropped decoded pictures of that OLS.

- When there is an SPO SEI message contained in a scalable nesting SEI message with sn_ols_flag equal to 0 for any output layer of the OLS that was decoded, the decoding system may choose to apply the processing chain of that SPO SEI message to the cropped decoded pictures of that output layer.

- It is proposed to disallow subpicture-specific scalable nesting of an SPO SEI message, the motivation being simplification of the handling of the processing chain.

The specification text is proposed to VVC, since the scalable nesting SEI message is specified in VVC.

- It is proposed to move such content of VSEI clause 8.30.3 that concerns selection of the processing chain and derivation of PoPicList to VVC clause D.12.13. The motivation is to include all options to select the processing chain and derive PoPicList in a single clause.

# Proposed specification text changes

The proposed text move from VSEI to VVC is indicated with cyan highlights.

The proposed text changes are indicated with yellow highlights.

## Changes to VVC

In clause D.6.2, replace nestingNumLayers with NestingNumLayers.

### D.12.13 Use of the SEI processing order and processing order nesting SEI messages

For purposes of interpretation of the SEI processing order (SPO) and processing order nesting (PON) SEI messages, the following are specified:

–	The list SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 210, and 211.

–	The list SpoProcessSeiList is set to consist of the payloadType values 19, 142, 155, 210, and 211.

–	The syntax structrure sei_pon_nested_message( ), the container of SEI messages, is set to be identical to the syntax structure sei_message( ).

The SEI messages contained in a PON SEI message are referred to as PON-nested SEI messages. When a generalized cubemap project SEI message is present with gcmp_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated generalized cubemap project SEI message in the same CLVS that is a PON-nested SEI message.

A special NNPF cascading case is defined as the case when such two NNPFs are both activated for a picture: the two PPFs are both NNPFs, one of the two NNPFs has nnpfc_purpose equal to 4 and the other has multiple input pictures, and neither of the two NNPFs is associated with an SPO SEI message. In this case, the two NNPFs are implicitly considered as belonging to one processing chain, and the NNPF with nnpfc_purpose equal to 4 is applied first. The special NNPF cascading case is handled as specified in clause D.12.11 and the general post-processing filtering process using NNPFs specified in Rec. ITU-T H.274 | ISO/IEC 23002-7.

Except for the special NNPF cascading case, each processing chain containing multiple SEI message types is indicated by an SPO SEI message with a particular value of po_id. Except for the special NNPF cascading case, any SEI message for which the payloadType is present in SpoProcessSeiList but is not indicated by an SPO SEI message is in its own processing chain.

Processing chains are alternatives to each other, i.e., at most one processing chain can be chosen to be applied by a decoding system at one time.

The value of sn_subpic_flag shall be equal to 0 in all scalable nesting SEI messages that contain an SPO SEI message in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall allow and ignore all SPO SEI messages contained in scalable nesting SEI messages having sn_subpic_flag equal to 1.

After decoding the bitstream BitstreamToDecode as specified in clause 8, a decoding system should choose any of the following processing chains specified by SPO SEI messages and apply the chosen processing chain as specified below:

–	The decoding system chooses a processing chain specified in an SPO SEI message that is not contained in a scalable nesting SEI message and sets TargetPoId to be equal to the value of po_id in that SPO SEI message. PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream BitstreamToDecode. The handling of a processing chain as specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 is invoked with PoPicList and TargetPoId as inputs.

–	The decoding system chooses a processing chain specified in an SPO SEI message that is contained in a scalable nesting SEI message with sn_ols_flag equal to 1 and NestingOlsIdx[ i ] equal to TargetOlsIdx for any value of i in the range of 0 to sn_num_olsss_minus1, inclusive. TargetPoId is set equal to the value of po_id in that SPO SEI message. PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream BitstreamToDecode. The handling of a processing chain as specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 is invoked with PoPicList and TargetPoId as inputs.

–	The following ordered steps apply for each value of j in the range of 0 to NumOutputLayersInOls[ TargetOlsIdx ] − 1, inclusive:

–	The decoding system chooses at most one processing chain specified in an SPO SEI message that is contained in a scalable nesting SEI message with sn_ols_flag equal to 0 and NestingLayerId[ k ] equal to OutputLayerIdInOls[ TargetOlsIdx ][ j ] for any value of k in the range of 0 to NestingNumLayers − 1, inclusive, and sets TargetPoId to be equal to the value of po_id in that SPO SEI message.

–	PoPicList is set to be the list of cropped decoded pictures in output order that resulted from decoding the bitstream BitstreamToDecode and have nuh_layer_id equal to OutputLayerIdInOls[ TargetOlsIdx ][ j ].

–	When a processing chain was chosen for this value of j, the handling of a processing chain as specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 is invoked with PoPicList and TargetPoId as inputs.

NOTE – Even if an SEI message type of a particular payloadType value is included in a chosen processing chain specified by an SPO SEI message included in a scalable nesting SEI message, an SEI message seiA with that payloadType value does not necessarily apply to a picture picA when the nuh_layer_id values of picA and the SEI NAL unit containing the SEI message seiA differ..

## Changes to VSEI

    - Handling of a processing chain

      - General

Processing chains are alternatives to each other, i.e., at most one processing chain can be chosen to be applied by a decoding system at one time.

A special NNPF cascading case is defined as the case when such two NNPFs are both activated for a picture: the two PPFs are both NNPFs, one of the two NNPFs has nnpfc_purpose equal to 4 and the other has multiple input pictures, and neither of the two NNPFs is associated with an SPO SEI message. In this case, the two NNPFs are implicitly considered as belonging to one processing chain, and the NNPF with nnpfc_purpose equal to 4 is applied first.

Except for the special NNPF cascading case, each processing chain containing multiple SEI message types is indicated by an SPO SEI message with a particular value of po_id. Except for the special NNPF cascading case, any SEI message for which the payloadType is present in SpoProcessSeiList but is not indicated by an SPO SEI message is in its own processing chain.

Inputs to this process are:

–	a list of pictures PoPicList that specifies the pictures in output order to which the processing chain is applied,

–	a target po_id value TargetPoId that identifies the SPO SEI message specifying the chosen processing chain.

When po_breadth_first_flag is equal to 1, the breadth-first handling of a processing chain specified in clause 8.30.3.2 is applied. When po_breadth_first_flag is equal to 0, the decoding system should apply the depth-first handling of a processing chain specified in clause 8.30.3.3 but may alternatively apply the breadth-first handling of a processing chain specified in clause 8.30.3.2.

NOTE – When po_breadth_first_flag is equal to 0, the depth-first handling of a processing chain is recommended to avoid the need of processing any SEI messages applying to subsequent picture units in output order.

      - Breadth-first handling of a processing chain

A decoding system may choose and apply a processing chain as follows:

–	First, the bitstream is decoded, and the list PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream, and a processing chain is chosen.

–	For each of the SEI message types of the chosen processing chain, the following applies in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values:

–	The following applies for each picture picA in PoPicList in output order, when an SEI message associated with the i-th SEI message type is present in PoSeiList derived for picA or a picture for which an NNPF that generated picA was activated by a preceding process in the processing chain:

–	When picA is not a cropped decoded picture, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from picA instead of the syntax elements indicating properties for the respective cropped decoded picture.

–	The semantics of the SEI message, or the semantics of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	When the i-th SEI message type is present in SpoProcessingList, the process implied by the SEI message is performed and PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

      - Depth-first handling of a processing chain

A decoding system may choose and apply a processing chain as follows:

–	First, the bitstream is decoded, and the list PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream, and a processing chain is chosen.

–	The following is repeatedly applied, in output order, for each picture picA in PoPicList:

–	The following applies for each SEI message of PoSeiList derived for picA in an increasing order of list indexes for PoSeiList:

–	When the current SEI message is not the first in the set of the SEI messages, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from the pictures in the updated PoPicList instead of the syntax elements indicating properties for the respective cropped decoded pictures.

–	The semantics of the SEI message, or of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	The process implied by the SEI message is invoked repeatedly, in output order, for picA and each of the pictures in PoPicList that are, or correspond to, interpolated or extrapolated pictures generated by the application of the process implied by any preceding SEI message, if any, to picA. After each invocation of the process, PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).