| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0129-v1 |
| --- | --- |

| Title: | AHG9: Miscellaneous comments on the SPO SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input contribution proposes the following items related to the SEI processing order (SPO) SEI message:

- It is proposed that the special NNPF cascading case applies when neither the NNPFA SEI messages nor the NNPFC SEI messages for the cascade are included in PON SEI message(s), whereas the presence of NNPFA and NNPFC SEI message types in an SPO SEI message does not impact whether the respective NNPFs qualify for the special NNPF cascading case. Some editorial changes to the special NNPF cascading case are also suggested.

- It is proposed that an SEI message is regarded as its own processing chain when it is not included in a PON SEI message regardless of whether the respective SEI message type is included in an SPO SEI message.

- To align VVC and VSEI regarding SEI message types that may be used in SPO and PON SEI messages, it is proposed to:

  - Include the payload type values of the frame packing arrangement, display orientation, annotated regions, and sample aspect ratio information SEI messages to SeiProcessingOrderSeiList.

  - Include the payload type value of the display orientation SEI message to SpoProcessSeiList.

  - Remove the constraints on the sphere rotation and omnidirectional viewport SEI messages from the semantics of the PON SEI message.

- It is proposed to remove the payload type of the NNPFC SEI message from SpoProcessSeiList.

- To clarify the use of the NNPFC SEI message in an SPO SEI message, the following is proposed to:

  - Add an informative NOTE suggesting examples when an NNPFC SEI message is reasonable in an SPO SEI message.

  - Add the following constraint: "When an po_sei_payload_type[ i ] indicates an NNPFC SEI message for a particular NNPF, the same SPO SEI message shall contain po_sei_payload_type[ j ] that indicates a respective NNPFA SEI message for the same NNPF with j greater than i."

- The processing order of SEI message types is proposed to be constrained to be the same throughout the bitstream when the breadth-first handling of a processing chain is applied.

# On the special NNPF cascading case

## Problem statement

For convenience, the specification text that the comments concern is copied below:

    - Handling of a processing chain

      - General

…

A special NNPF cascading case is defined as the case when such two NNPFs are both activated for a picture: the two PPFs are both NNPFs, one of the two NNPFs has nnpfc_purpose equal to 4 and the other has multiple input pictures, and neither of the two NNPFs is associated with an SPO SEI message. In this case, the two NNPFs are implicitly considered as belonging to one processing chain, and the NNPF with nnpfc_purpose equal to 4 is applied first.

Except for the special NNPF cascading case, each processing chain containing multiple SEI message types is indicated by an SPO SEI message with a particular value of po_id. Except for the special NNPF cascading case, any SEI message for which the payloadType is present in SpoProcessSeiList but is not indicated by an SPO SEI message is in its own processing chain.

The following comments are expressed:

- It should be clarified that for the special NNPF cascading case, the two NNPFs that are activated for a picture are not included in PON SEI message(s). Any SEI message included in a PON SEI message is explicitly associated with at least one processing chain specified by an SPO SEI message.

- The requirement "neither of the two NNPFs is associated with an SPO SEI message" seems incorrect, since it would disable using these NNPFs in a processing chain having additional optional processes before or after them. It is sufficient to require the NNPFC and NNPFA SEI messages for the two NNPFs not to be included in PON SEI messages.

- Editorial: In "when such two NNPFs are both activated for a picture", the word "such" does not point to anything and should be removed.

- Editorial: The requirement "the two PPFs are both NNPFs" is redundant, since the special NNPF case only applies to NNPFs and additionally the abbreviation PPF has not been defined.

- Editorial: It is suggested to consider using "implicit NNPF cascading case" rather than "special NNPF cascading case".

## Proposed VSEI specification text

    - Handling of a processing chain

      - General

…

An implicit A special NNPF cascading case is defined as the case when such two NNPFs are both activated for a picture by NNPFA SEI messages that are not included in PON SEI messages, the two PPFs are both NNPFs, one of the two NNPFs has nnpfc_purpose equal to 4 and the other has multiple input pictures, and the NNPFC SEI messages of the two NNPFs are not included in PON SEI messages neither of the two NNPFs is associated with an SPO SEI message. In this case, the two NNPFs are implicitly considered as belonging to one processing chain, and the NNPF with nnpfc_purpose equal to 4 is applied first.

Except for the implicit special NNPF cascading case, each processing chain containing multiple SEI message types is indicated by an SPO SEI message with a particular value of po_id. Except for the implicit special NNPF cascading case, any SEI message for which the payloadType is present in SpoProcessSeiList but is not indicated by an SPO SEI message is in its own processing chain.

# On the processing chains with only a single SEI message type

## Problem statement

For convenience, the specification text that the comments concern is copied below:

    - Handling of a processing chain

      - General

…

Except for the special NNPF cascading case, any SEI message for which the payloadType is present in SpoProcessSeiList but is not indicated by an SPO SEI message is in its own processing chain.

A legacy decoder may process any SEI message that specifies a process and is not included in a PON SEI message. Thus, such an SEI message forms its own processing chain even if it is included in an SPO SEI message.

## Proposed VSEI specification text

    - Handling of a processing chain

      - General

…

Except for the special NNPF cascading case, when the bitstream includes one or more SEI messages that are not included in PON SEI messages and have a particular payloadType and any SEI message for which the payloadType is present in SpoProcessSeiList but is not indicated by an SPO SEI message, the SEI message type for the payloadType is in its own processing chain.

# Mismatch between SeiProcessingOrderSeiList and the constraints in the PON SEI message

## Problem statement

SeiProcessingOrderSeiList is specified in VVC as follows: "The list SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 210, and 211." In other words, SeiProcessingOrderSeiList includes the following SEI messages:

- Filler payload

- User data registered by Rec. ITU-T T.35

- User data unregistered

- Film grain characteristics

- Mastering display colour volume

- Colour transform information

- Alternative transfer characteristics

- Ambient viewing environment

- Content colour volume

- Equirectangular projection

- Generalized cubemap projection

- Region-wise packing

- Alpha channel information

- Depth representation information

- Neural-network post-filter characteristics

- Neural-network post-filter activation

The semantics of the PON SEI message include a bullet list of SEI message specific constraints after the following paragraph:

It is a requirement of bitstream conformance that the semantics and effect of an SEI message that is not a PON-nested SEI message shall not depend on any PON-nested SEI message. Consequences of this constraint include the following specific constraints, in which an associated SEI message is considered to be an SEI message that affects the semantics or effect of a particular SEI message:

The bullet list specified in the semantics of the PON SEI message includes the following SEI messages that are not present in SeiProcessingOrderSeiList:

- Frame packing arrangement

- Sphere rotation

- Omnidirectional viewport

- Sample aspect ratio information

- Annotated regions

- Display orientation

## Discussion and overview of the proposal

A frame packing arrangement SEI message can be used in a processing chain to describe the frame packing arrangement after region-wise unpacking has been performed as indicated by a region-wise packing SEI message in the processing chain. It is therefore proposed to include the payloadType value (45) of the frame packing arrangement SEI message in SeiProcessingOrderSeiList.

The sphere rotation SEI message provides information on rotation angles between the global coordinate axes (e.g., used for displaying) and the local coordinate axes (e.g., used in the decoded content). In our understanding, the sphere rotation SEI message is used to indicate properties and is not defining a process. Furthermore, it is asserted that no SEI message is a process that would change the rotation angles to be different than what they are in the decoded content. It is therefore proposed not to include the sphere rotation SEI message in SeiProcessingOrderSeiList and exclude it from the constraints specified in the semantics of the PON SEI message.

In our understanding, the omnidirectional viewport SEI message is used to indicate properties and is not defining a process. Furthermore, it is asserted that no SEI message is a process that would change any of the omnidirectional viewports to be different than what they are in the decoded content. It is therefore proposed not to include the omnidirectional viewport SEI message in SeiProcessingOrderSeiList and exclude it from the constraints specified in the semantics of the PON SEI message.

The horizontal and vertical resolution resampling ratios are expressed separately in the NNPFC SEI message. When such as an NNPF is applied, the intended displaying behaviour (stretching the image vs. changing the sample aspect ratio) can be clarified by including a sample aspect ratio information SEI message subsequent to applying the NNPF in the processing chain. It is therefore proposed to include the payloadType value (204) of the sample aspect ratio information SEI message in SeiProcessingOrderSeiList.

An annotated regions SEI message can be used in a processing chain to describe the regions after region-wise unpacking has been performed as indicated by a region-wise packing SEI message in the processing chain. It is therefore proposed to include the payloadType value (202) of the annotated regions SEI message in SeiProcessingOrderSeiList.

The display orientation SEI message includes display_orientation_transform_type, which specifies the rotation and mirroring to be applied to the picture. It is therefore proposed to include the payloadType value of the display orientation SEI message (47) in SeiProcessingOrderSeiList and SpoProcessSeiList.

## Proposed VVC specification text

### D.12.13 Use of the SEI processing order and processing order nesting SEI messages

For purposes of interpretation of the SEI processing order and processing order nesting (PON) SEI messages, the following are specified:

–	The list SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 45, 47, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 202, 204, 210, and 211.

–	The list SpoProcessSeiList is set to consist of the payloadType values 19, 47, 142, 155, 210, and 211.

…

## Proposed VSEI specification text

### 8.30.2.2 Processing order nesting SEI message semantics

…

–	When a sphere rotation SEI message is present with sphere_rotation_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated sphere rotation SEI message in the same CLVS that is a PON-nested SEI message.

…

–	When an omnidirectional viewport SEI message is present with omni_viewport_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated omnidirectional viewport SEI message in the same CLVS that is a PON-nested SEI message.

# On the NNPFC SEI message in SpoProcessSeiList

## Problem statement

JVET-AI2005 specifies both the NNPFC SEI message (payload type 210) and the NNPFA SEI message (payload type 211) as processes for the processing order:

–	The list SpoProcessSeiList is set to consist of the payloadType values 19, 142, 155, 210, and 211.

An NNPFC SEI message defines an NNPF, whereas an NNPFA SEI message is actually invoking the NNPF inference. The proponents did not identify any good reason to exclude an NNPFA SEI message from an SPO SEI message and only include the respective NNPFC SEI message in the SPO SEI message. Therefore, an NNPFC SEI message in an SPO SEI message is considered to indicate properties.

## Overview of the proposal

It is proposed to remove the payload type of the NNPFC SEI message from SpoProcessSeiList.

## Proposed VVC specification text

### D.12.13 Use of the SEI processing order and processing order nesting SEI messages

For purposes of interpretation of the SEI processing order and processing order nesting (PON) SEI messages, the following are specified:

–	The list SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 210, and 211.

–	The list SpoProcessSeiList is set to consist of the payloadType values 19, 142, 155, 210, and 211.

…

# On the NNPFC SEI message in the SPO SEI message

## Problem statement

It may be desired to omit an NNPFC SEI message from the SEI message types of an SPO SEI message for example when there is a single NNPF present in the bitstream and the NNPF may be used as a standalone process (not part of any processing chain defined by any SPO SEI message) and as a process within a processing chain defined by an SPO SEI message. In this case, the NNPFC SEI message is not included in a PON SEI message, and including the NNPFC SEI message in an SPO SEI message may not provide characteristics that would be necessary for interpreting a processing chain.

It may be desired to include an NNPFC SEI message among the SEI message types of an SPO SEI message for example in the following cases:

–	The NNPFC SEI message type includes an SEI prefix that includes the nnpfc_purpose syntax element, which provides a hint to a decoding system which kinds of processing is included in the respective NNPF in the processing chain.

–	The NNPF is to be invoked only as a part of a processing chain and hence the NNPFC SEI message type is associated with po_sei_wrapping_flag[ i ] equal to 1 and the respective NNPFC SEI message(s) are included in PON SEI message(s).

There are no constraints on the order of respective NNPFC and NNPFA SEI messages in an SPO SEI message. It would be logical to require NNPFC SEI message to appear earlier than the respective NNPFA SEI message, aligned with their bitstream order.

## Overview of the proposal

The following items are proposed:

- It is proposed to add an informative NOTE suggesting examples when an NNPFC SEI message is reasonable in an SPO SEI message.

- When an NNPFC SEI message is present as an i-th SEI message type in an SPO SEI message, it is proposed to require the respective NNPFA SEI message type to be present as the j-th SEI message type in the SPO SEI message with j greater than i.

## Proposed VSEI specification text

NOTE: An NNPFC SEI message type has the value of po_sei_payload_type[ i ] equal to the payload type value of the NNPFC SEI message. Examples of reasons for including an NNPFC SEI message type in an SPO SEI message include the following:

–	The NNPFC SEI message type includes an SEI prefix that includes the nnpfc_purpose syntax element, which provides a hint to a decoding system which kinds of processing is included in the respective NNPF in the processing chain.

–	The NNPF is to be invoked only as a part of a processing chain and hence the NNPFC SEI message type is associated with po_sei_wrapping_flag[ i ] equal to 1 and the respective NNPFC SEI message(s) are included in PON SEI message(s).

When an po_sei_payload_type[ i ] indicates an NNPFC SEI message for a particular NNPF, the same SPO SEI message shall contain po_sei_payload_type[ j ] that indicates a respective NNPFA SEI message for the same NNPF with j greater than i.

# On the processing order for breadth-first handling

## Problem statement

The phrase "a non-decreasing order of po_sei_processing_order[ i ] values" could be interpreted to allow dynamic changes of the processing order picture by picture. However, such dynamic changes would be incompatible with the breadth-first handling of the processing chain, which assumes the same processing order throughout the bitstream.

The phrases "a non-decreasing order of po_sei_processing_order[ i ] values" in SPO SEI semantics and "a non-decreasing order of the corresponding po_sei_processing_order[ i ]" in the breadth-first handling of a processing chain could be interpreted to allow different processing orders from each other. However, different processing orders would not make sense.

## Overview of the proposal

The processing order of SEI message types is proposed to be constrained to be the same throughout the bitstream when the breadth-first handling of a processing chain is applied.

## Proposed VSEI specification text

## 8.30.1.2	SEI processing order SEI message semantics

…

PoSeiOrder, which is a list of SEI message types, is derived to include the list of SEI message types in any non-decreasing order of po_sei_processing_order[ i ] values for all values of i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive. When the breadth-first handling of a processing chain is applied, PoSeiOrder is derived once and only once for an SEI processing order SEI message and applied to all pictures. When the depth-first handling of a processing chain is applied, PoSeiOrder for an SEI processing order SEI message is derived independently for picA and may change from the previous PoSeiOrder for the same SEI processing SEI message.

Let the values of poIdx[ i ] be such that PoSeiOrder[ i ] is the poIdx[ i ]-th SEI message type indicated by this SPO SEI message for all values of i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive.

PoSeiList is derived as follows:

–	PoSeiList is initially empty.

–	The following applies in a non-decreasing order of po_sei_processing_order[ i ] values increasing order of i for all values of i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, unless terminated earlier as specified below:

–	When an SEI message seiA associated with the i-th SEI message type in PoSeiOrder persists for picA, the following applies:

–	If the decoding system can interpret and supports the functionality indicated by seiA, seiA is added at the end of PoSeiList.

–	Otherwise, if po_sei_importance_flag[ poIdx [ i ] ] is equal to 1 and po_sei_processing_degree_flag[ poIdx [ i ] ] is equal to 0, the derivation of PoSeiList is terminated.

–	Otherwise, if po_sei_importance_flag[ poIdx [ i ] ] is equal to 1 and po_sei_processing_degree_flag[ poIdx [ i ] ] is equal to 1, the processing chain specified by this SPO SEI message should not be performed for picA, PoSeiList is set to be empty, and the derivation of PoSeiList is terminated.

…

      - Breadth-first handling of a processing chain

A decoding system may choose and apply a processing chain as follows:

–	First, the bitstream is decoded, and the list PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream, and a processing chain is chosen.

–	For each of the SEI message types of the chosen processing chain, the following applies in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values increasing order of i for all values of i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive:

–	The following applies for each picture picA in PoPicList in output order, when an SEI message associated with the i-th SEI message type in PoSeiOrder is present in PoSeiList derived for picA or a picture for which an NNPF that generated picA was activated by a preceding process in the processing chain:

–	When picA is not a cropped decoded picture, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from picA instead of the syntax elements indicating properties for the respective cropped decoded picture.

–	The semantics of the SEI message, or the semantics of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	When the i-th SEI message type in PoSeiOrder is present in SpoProcessingList, the process implied by the SEI message is performed and PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).