| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0130-v1 |
| --- | --- |

| Title: | AHG9: Applying neural-network post-filters to multi-layer bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Jill Boyce Nokia Technologies | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com jill.boyce@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Use cases where it would be desirable to take pictures from multiple layers as input to an NNPF include:

- Usage of an object mask, depth map, or alpha map of an auxiliary layer to select areas of a texture-layer picture to be filtered.

- Enhancing the quality of a lower-quality view of mixed-quality stereoscopic video.

The contribution proposes to enable inputs to an NNPF from multiple layers as follows:

- It is allowed to include an NNPFA SEI message within a scalable nesting SEI message.

- When an NNPFA SEI message is included in scalable nesting SEI message, the input pictures for the NNPF are derived to be the pictures that are present in the same access unit and have layer identifier values indicated in the scalable nesting SEI message.

All the proposed text changes are for VVC. The proposed text does not change the specification text for non-nested NNPFC and NNPFA SEI messages.

# Introduction

This contribution uses the term cross-layer NNPF to describe an NNPF that takes inputs from more than one layer of the bitstream. Use cases for cross-layer NNPF include the following:

- Usage of an object mask, depth map, or alpha map auxiliary layer as input to an NNPF to select areas to be filtered and/or to provide per-pixel filtering strengths.

- When stereoscopic video has been encoded using a mixed quality such that one view has a higher picture quality than the other, a quality enhancement NNPF can take both views as input to enhance the lower-quality view.

The contribution proposes the cross-layer NNPF to enable the use cases described above.

# Overview of the proposal

The NNPF design enables the use of multiple input pictures. nnpfc_num_input_pics_minus1 + 1 specifies the count of input pictures to the NNPF. In the VVC interface text for the NNPFC and NNPFA SEI messages, the input pictures are selected within the current CLVS in inverse output order starting from the current picture.

This input document proposes to specify in the VVC interface text for the NNPFC and NNPFA SEI messages that when an NNPFA SEI message is included in scalable nesting SEI message, the input pictures for the NNPF are derived to be the pictures that are present in the same access unit and have layer identifier values indicated in the scalable nesting SEI message.

Due to the constraints in the NNPFC design, all the input pictures are required to have the same width, height, and bit depth.

If a monochrome layer is used as input to an NNPF and the NNPF takes three-component input pictures, the chroma components of the input picture are set to mid-level grey.

# Proposed specification text

### D.12.11	Use of the neural network post-filter characteristics SEI message and the neural network post-filter activation SEI message

- D.12.11.1	General

When an NNPFA SEI message is not included in a scalable nesting SEI message, the specifications of subclause D.12.11.2 apply for the use of the NNPFA SEI message and the associated NNPFC SEI messages.

When an NNPFA SEI message is included in a scalable nesting SEI message with sn_ols_flag equal to 0 and sn_subpic_flag equal to 0, the specifications of subclause D.12.11.3 apply for the use of the NNPFA SEI message and the associated NNPFC SEI messages.

- D.12.11.2	Use of NNPFC and NNPFA SEI messages not included in scalable nesting SEI messages

[no changes relative to the current text of D.12.11]

- D.12.11.3	Use of a scalable-nested NNPFA SEI message with the associated NNPFC SEI messages

Let currAu be an access unit for which the scalable-nested NNPFA SEI message is active.

If currAu does not include each picture that has nuh_layer_id equal to NestingLayerId[ i ] and PictureOutputFlag equal to 1 for each value of i in the range of 0 to NestingNumLayers − 1, inclusive, the NNPF is not applied to currAu. Otherwise, the NNPF may be applied to currAu as follows. [Ed. (MH): In D.7.1., change nestingNumLayers to be NestingNumLayers.]

Let currPic be the picture that is present in currAu and has nuh_layer_id equal to NestingLayerId[ 0 ].

An NNPFC SEI message that has nnpfc_id equal to nnpfa_target_id is an associated NNPFC SEI message.

NOTE – It is not required to include an associated NNPFC SEI message in a scalable nesting SEI message.

It is a requirement of bitstream conformance that both PictureRateUpsamplingFlag and TemporalExtrapolationFlag shall be equal to 0 for the associated NNPFC SEI message.

It is a requirement of bitstream conformance that nnpfc_num_input_pics_minus1 + 1 shall be equal to NestingNumLayers.

It is a requirement of bitstream conformance that the values of inWidth[ i ], inHeight[ i ], and inBitDepth[ i ] shall be the same for each picture that is present in currAu and has nuh_layer_id equal to NestingLayerId[ i ] for i in the range of 0 to NestingNumLayers − 1, inclusive, where inWidth[ i ], inHeight[ i ], and inBitDepth[ i ] are derived as follows:.

–	inWidth[ i ] is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( pps_conf_win_left_offset + pps_conf_win_right_offset ) for the picture that is present in currAu and has nuh_layer_id equal to NestingLayerId[ i ].

–	inHeight[ i ] is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) for the picture that is present in currAu and has nuh_layer_id equal to NestingLayerId[ i ].

–	inBitDepth[ i ] is set equal to 0 the value of BitDepth of the picture that is present in currAu and has nuh_layer_id equal to NestingLayerId[ i ].

For purposes of interpretation of the scalable-nested NNPFA SEI message with the associated NNPFC SEI messages, the following variables are specified:

–	CroppedWidth is set equal to inWidth[ 0 ].

–	CroppedHeight is set equal to inHeight[ 0 ].

–	When nnpfc_inp_order_idc is equal to 0, 2 or 3, the luma sample array of the picture that is present in currAu and has nuh_layer_id equal to NestingLayerId[ i ] is assigned to the luma sample array CroppedYPic[ i ] for all values of i in the range of 0 to NestingNumLayers − 1, inclusive.

–	When nnpfc_inp_order_idc is equal to 1, 2, or 3, the following applies for each value of i in the range of 0 to NestingNumLayers − 1, inclusive:

–	If there is a picture that is present in currAu, has nuh_layer_id equal to NestingLayerId[ i ], and has chroma sample arrays, these chroma sample arrays are assigned to CroppedCbPic[ i ] and CroppedCrPic[ i ].

–	Otherwise, all the sample values of CroppedCbPic[ i ] and and CroppedCrPic[ i ] are set equal to ( 1 << ( inBitDepth[ 0 ] − 1 ) ).

–	BitDepthY and BitDepthC are both set equal to inBitDepth[ 0 ].

–	ChromaFormatIdc is set equal to sps_chroma_format_idc applying to currPic.

–	When nnpfc_auxiliary_inp_idc is equal to 1, the following applies for each value of i in the range of 0 to NestingNumLayers − 1, inclusive:

–	StrengthControlVal[ i ] is set equal to the value of ( firstSliceQpY + QpBdOffset ) ÷ ( 63 + QpBdOffset ), where firstSliceQpY is equal to SliceQpY of the first slice of the picture that is present in currAu and has nuh_layer_id equal to NestingLayerId[ i ].

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).