| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0132-v1 |
| --- | --- |

| Title: | AHG9: Usage of NNPFC SEI message to define the generator NN of the GFV SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that a precise interoperability between a generative face video (GFV) encoder and decoder can be achieved by letting the encoder to specify which specific generator neural network (GenerativeNN) the decoder is intended to use. This contribution is a follow-up of JVET-AG0088 and proposes to enable the use of a neural-network post-filter (NNPF) defined in the NNPF characteristics (NNPFC) SEI message as the GenerativeNN of the GFV SEI message design. The proposal is summarized as follows:

- When an NNPF is used as a GenerativeNN, it is required to have the temporal extrapolation purpose.

- The NNPF auxiliary input mechanism is extended as follows to input face parameters and matrices to the NNPF inference:

  - nnpfc_auxiliary_inp_idc & 4 greater than 0 specifies that the auxiliary input is derived from SEI message(s) of a given payload type value (nnpfc_sei_aux_payload_type).

  - When nnpfc_sei_aux_payload_type is equal to the payload type value of the GFV SEI message, nnpfc_sei_aux_id is present and indicates the gfv_id value of the GFV SEI message for which this NNPF serves as the GenerativeNN.

  - When nnpfc_sei_aux_payload_type is equal to the payload type value of the GFV SEI message, the auxiliary inputs are inputDriveKeyPoint, inputDriveMatrix, inputBaseKeyPoint, and inputBaseMarix as derived in the semantics of the GFV SEI message.

# Earlier work

JVET-AG0088 proposed to enable the use of a neural-network post-filter (NNPF) defined in the NNPF characteristics (NNPFC) SEI message as the generator neural network (GenerativeNN) for the generative face video (GFV) SEI message. Ingredients of JVET-AG0088 included the following:

- The use of the NNPF auxiliary input mechanism to input face parameters and matrices to the NNPF inference, including the following aspects:

  - nnpfc_auxiliary_inp_idc & 2 greater than 1 specifies that the auxiliary input is derived from SEI message(s) of a given payload type value (nnpfc_sei_aux_payload_type).

  - When nnpfc_sei_aux_payload_type is equal to the payload type value of the GFV SEI message, nnpfc_sei_aux_id is present and indicates the gfv_id value of the GFV SEI message for which this NNPF serves as the GenerativeNN.

  - When nnpfc_sei_aux_payload_type is equal to the payload type value of the GFV SEI message, the auxiliary inputs are inputDriveKeyPoint, inputDriveMatrix, inputBaseKeyPoint, and inputBaseMarix as derived in the semantics of the GFV SEI message.

- Addition of generative modelling among the purposes indicated by nnpfc_purpose.

  - Generative modelling is proposed to be added as the next available bit in nnpfc_purpose.

  - It is proposed to constrain generative modelling and general visual quality enhancement to be mutually exclusive.

The meeting report (JVET-AG1000) summarizes the discussion on JVET-AG0088 as follows:

The NNPF is suitable for this purpose, as the input would be base pictures and drive pictures, and parameter as aux information, output are the synthesized pictures.

It was commented that it would be interesting to analyse the size of the NNPF representation expressed by NNR. On the other hand, if an URL of an existing synthesis method would be signalled, the advantage of having precise matching between encoder and decoder would be retained.

It is not the goal of this contribution to replace anything in the existing SEI.

Generally this option of signalling the synthesis method is agreed to be beneficial, and would get rid of any interoperability problem.

It was commented that another approach could be to make the GFV SEI self contained in this regard, to include the whole signalling in it, similar as already for the parameter translator. This might be simpler than being forced to implement the entire NNPF SEI additionally.

A new purpose would need to be defined in NNPF, and if that purpose is generically “generative modelling”, it might be much less constrained than for the mere purpose of face generation.

Further study was requested on these aspects.

This contribution is a follow-up of JVET-AG0088. The aspects requested to be studied further have been investigated and the proposal has been updated accordingly.

# Introduction

## Motivation

The generative face video (GFV) SEI message utilizes a generator neural network (GenerativeNN) to create a picture derived from a base picture, a drive picture (optional), and the face key points and matrices derived from the GFV SEI messages of the base picture and the current picture. The specific GenerativeNN is not identified in the GFV SEI message.

It is asserted that better interoperability can be achieved by letting the encoder to specify which specific GenerativeNN the decoder is intended to use. This contribution proposes a mechanism to provide means for the encoder to identify GenerativeNN in the bitstream so that the decoder is able to use the intended GenerativeNN.

## Use of the neural-network post-filter characteristics (NNPFC) SEI message to define the GenerativeNN

The neural-network post-filter characteristics (NNPFC) SEI message provides the formatting of the input tensors to the neural network inference, the formatting of the output tensors resulting from the neural network inference, and characterization of the complexity of the neural network. The NNPFC SEI message allows to define a base NNPF and zero or more NNPF updates. Both the base NNPF and an NNPF update can be defined as a URI or included in the NNPFC SEI message as an NNR (ISO/IEC 15938-17) bitstream. These features are asserted to be useful for the GenerativeNN.

One of the comments on JVET-AG0088 was to consider making the GFV SEI self-contained by including signalling like the NNPFC SEI message within the GFV SEI message. This could be realized by copy-pasting the NNPFC SEI message syntax excluding nnpfc_purpose and nnpfc_id into the GFV SEI message syntax with updated syntax element names. The added syntax should be gated, since it may not be used in all GFV SEI messages. It may not be required to duplicate the semantics of the added syntax, but they can refer to the respective NNPFC semantics, e.g., "gfv_gennn_base_flag has the same semantics as nnpfc_base_flag". However, the approach is asserted to have the following downsides:

- The NNPFC SEI message syntax is about 2.5 pages long. Hence, the GFV SEI message syntax would become significantly longer too.

- The NNPFC SEI message allows for future extensions. For example, it has a metadata extension part that may be extended. It may be desirable to keep such future extensions aligned in both NNPFC and GFV SEI messages designed this way.

- The NNPFC-like syntax within the GFV SEI message has its own persistence, different from the other parts of the GFV SEI message. This could be complicated to specify and implement.

This contribution proposes to use the NNPFC SEI message to define the GenerativeNN. The GenerativeNN is hence regarded as a neural-network post-filter (NNPF).

The temporal extrapolation NNPF purpose has been adopted into VSEI v4. Each inference of the GenerativeNN generates a subsequent picture in output order. Hence, it is asserted that the temporal extrapolation purpose is suitable to be used for the GenerativeNN and no new NNPFC purpose is needed, unlike what was proposed in JVET-AG0088.

## Use of the NNPF auxiliary input mechanism

The NNPFC SEI message provides the extensible auxiliary input mechanism, which enables to input data other than the input pictures to the neural network inference. This contribution proposes to use the auxiliary input mechanism to pass the face key points and matrices to the inference of GenerativeNN defined as an NNPF.

The value range of nnpfc_auxiliary_inp_idc is specified to be up to 255, inclusive. In VSEI version 3, only one auxiliary input type, namely the QP-based StrengthControlVal[ i ], was defined, which is indicated by nnpfc_auxiliary_inp_idc equal to 1. In the draft VSEI version 4, another auxiliary input type, namely a text prompt, has been added and is indicated by nnpfc_auxiliary_inp_idc & 2 being greater than 0. It is envisioned that it can be useful to continue to specify nnpfc_auxiliary_inp_idc as a bit field to allow mixed use of auxiliary input types.

The value range for specifying auxiliary input types is relatively limited. Consequently, rather than specifying an auxiliary input type specifically for the GFV SEI message only, this contribution proposes a generic auxiliary input type for deriving auxiliary input from another SEI message that is identified by its payload type value (nnpfc_sei_aux_payload_type). Thus, if any other future SEI message is used to specify auxiliary input for an NNPF defined in a NNPFC SEI message, the same auxiliary input type can be used.

It is proposed to use bit position 3 for the proposed auxiliary input type, i.e., when nnpfc_auxiliary_inp_idc & 4 is greater than 0, the auxiliary input derived from SEI messages of payload type value nnpfc_sei_aux_payload_type is in use for the NNPF inference.

Additional syntax elements that are gated by the payload type value nnpfc_sei_aux_payload_type can be present in the NNPFC SEI message. For nnpfc_sei_aux_payload_type equal to the payload type value of the GFV SEI message (denoted as constant SEI_GFV_PAYLOAD_TYPE), it is proposed to include nnpfc_sei_aux_id, which indicates the gfv_id value of the GFV SEI message for which this NNPF serves as the GenerativeNN.

# Proposed specification text

| … |  |
| --- | --- |
| nnpfc_auxiliary_inp_idc | ue(v) |
| if( nnpfc_auxiliary_inp_idc & 4 ) { |  |
| nnpfc_sei_aux_payload_type | u(16) |
| if( nnpfc_sei_aux_payload_type = = SEI_GFV_PAYLOAD_TYPE ) |  |
| nnpfc_sei_aux_id | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| … |  |

…

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF or auxiliary input tensor(s) are provided to the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor and no auxiliary input tensors are provided to the NNPF.

nnpfc_auxiliary_inp_idc equal to 1, 2 or 3 ( nnpfc_auxiliary_inp_idc & 3 ) greater than 0 specifies that auxiliary input data is derived as specified in Equation 95.

( nnpfc_auxiliary_inp_idc & 4 ) greater than 0 specifies that auxiliary input data or auxiliary input tensor(s) are derived based on a given payload type value of an SEI message and a given identifier value.

When nnpfc_auxiliary_inp_idc is equal to 2 or 3 ( nnpfc_auxiliary_inp_idc & 2 ) is greater than 0, nnpfc_spatial_extrapolation_prompt_present_flag shall be equal to 1.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 255, inclusive. Values of 28 to 255, inclusive, for nnpfc_auxiliary_inp_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_auxiliary_inp_idc in the range of 48 to 255, inclusive.

When ( nnpfc_auxiliary_inp_idc & 1 ) is equal to 1 the auxiliary input data consists of includes strengthControlScaledVal[ i ].

When ( nnpfc_auxiliary_inp_idc & 2 ) is equal to 2 the auxiliary input data consists of includes nnpfc_prompt character values.

When nnpfc_auxiliary_inp_idc is equal to 3, the auxiliary input data consists of strengthControlScaledVal[ i ] and nnpfc_prompt character values.

nnpfc_sei_aux_payload_type specifies the SEI message payload type value that is used to derive auxiliary input data or auxiliary input tensor(s) to the NNPF. nnpfc_sei_aux_payload_type values not equal to SEI_GFV_PAYLOAD_TYPE are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_sei_aux_payload_type not equal to SEI_GFV_PAYLOAD_TYPE.

It is a requirement of bitstream conformance that when ( nnpfc_auxiliary_inp_idc & 4 ) is greater than 0 and nnpfc_sei_aux_payload_type is equal to SEI_GFV_PAYLOAD_TYPE, TemporalExtrapolationFlag shall be equal to 1, nnpfc_num_input_pics_minus1 shall be equal to 1, and nnpfc_extrapolated_pics_minus1 shall be equal to 0.

nnpfc_sei_aux_id specifies an identifying number that may be used to identify which one(s) of the SEI messages of payload type equal to nnpfc_sei_aux_payload_type are used to derive auxiliary input data or auxiliary input tensor(s) to the NNPF. nnpfc_sei_aux_id shall be in the range of 0 to 232 − 2, inclusive. When nnpfc_sei_aux_payload_type is equal to SEI_GFV_PAYLOAD_TYPE, it is a requirement of bitstream conformance that when a generative face video SEI message with gfv_id equal to nnpfc_sei_aux_id is present in a picture unit, the picture unit shall include an NNPFA SEI message with nnpfa_target_id equal to nnpfc_id.

…

When ( nnpfc_auxiliary_inp_idc & 4 ) is equal to 4 and nnpfc_sei_aux_payload_type is equal to SEI_GFV_PAYLOAD_TYPE, the following applies:

–	inputBaseY, inputBaseCb (when applicable), inputBaseCr (when applicable), inputBaseKeyPoint, and inputBaseMatrix are derived as specified for the semantics of the generative face video SEI message that has gfv_id equal to nnpfc_sei_aux_id and gfv_base_pic_flag equal to 1. If there are multiple such generative face video SEI messages present, the derivation is based on the last one preceding, in decoding order, the picture unit containing an NNPFA SEI message with nnpfa_target_id equal to nnpfc_id used for activating the NNPF.

–	The associated generative face video SEI message is the generative face video SEI message that has gfv_id equal to nnpfc_sei_aux_id and is present in the picture unit that contains an NNPFA SEI message with nnpfa_target_id equal to nnpfc_id. Subsequently, the syntax elements of the generative face video SEI message refer to the associated generative face video SEI message.

–	inputDriveKeyPoint and inputDriveMatrix are derived as specified for the semantics of the associated generative face video SEI message.

–	When gfv_drive_pic_fusion_flag is equal to 0, sample values of CroppedYPic[ 0 ], CroppedCbPic[ 0 ] (when applicable), and CroppedCrPic[ 0 ] (when applicable) are all set equal to 0.

NOTE – When gfv_drive_pic_fusion_flag is equal to 0, an input picture to the NNPF that has sample arrays with sample value 0 is a dummy picture as defined in the generative face video SEI message. When gfv_drive_pic_fusion_flag is equal to 1, the sample arrays CroppedYPic[ 0 ], CroppedCbPic[ 0 ] (when applicable), and CroppedCrPic[ 0 ] (when applicable) are from the current picture.

–	The input picture with index 1 is overwritten with the base picture as defined in the generative face video SEI message as follows: CroppedYPic[ 1 ] is set equal to inputBaseY, CroppedCbPic[ 1 ] is set equal to inputBaseCb (when applicable), and CroppedCrPic[ 1 ] is set equal to inputBaseCr.

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, the number of auxiliary input tensors numAuxInputTensors, and the auxiliary input tensors auxInputTensor[ j ], if any, for each value of j in the range of 0 to numAuxInputTensors − 1, inclusive, is specified as follows:

if( ( nnpfc_auxiliary_inp_idc & 4 ) &&
		( nnpfc_sei_aux_payload_type  = =  SEI_GFV_PAYLOAD_TYPE ) )
		numAuxInputTensors = 2
	else
		numAuxInputTensors = 0
for( i = 0; i < numInputPics; i++ ) {
	…
	// add to the end of the for loop
	if( ( nnpfc_auxiliary_inp_idc & 4 ) &&
		( nnpfc_sei_aux_payload_type  = =  SEI_GFV_PAYLOAD_TYPE ) ) {		if( i  = =  0 ) {
			auxInputTensor0[ 0 ][ i ] = inputDriveKeyPoint
			auxInputTensor1[ 0 ][ i ] = inputDriveMatrix
		} else { // i  = =  1, base picture
			auxInputTensor0[ 0 ][ i ] = inputBaseKeyPoint
			auxInputTensor1[ 0 ][ i ] = inputBaseMatrix
		}
	}
}

…

An NNPF PostProcessingFilter( ) is the target NNPF as derived in the semantics of the NNPFA SEI message. The NNPF PostProcessingFilter( ) inputs inputTensor, followed by numAuxInputTensors tensors auxInputTensorj where j is in the range of 0 to numAuxInputTensors − 1, inclusive. The following example process may be used, with the NNPF PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contain Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc:

if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {	(98)
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).