| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0134-v1 |
| --- | --- |

| Title: | Non-EE2: Enhanced derivation of affine merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Zhipin Deng1, Na Zhang1, Yang Wang1, Wenbin Yin1, Mehdi Salehifar2, Li Zhang2 1No. 27 North Third Ring West Road, Haidian District, Beijing Fangheng Fashion Center, 100098, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This contribution proposes to improve the derivation of affine merge candidates. Firstly, additional affine temporal candidates can be derived by scaling the collocated CPMVs to the reference pictures with index 0, and the collocated CPMVs referring to both lists can be utilized to obtain the temporal affine candidates. Secondly, synthetic affine candidates are introduced by combining CPMVs of existing candidates. It is reported that on top of ECM-14.0, simulation results of the proposed method are as below:

RA: 0.xx % (Y), 0.xx % (U), 0.xx % (V), xx.x% (EncT), xx.x% (DecT) .

- Introduction

ECM-14.0 incorporates temporal affine candidate derivation [1] to enable the inheritance of affine motion from collocated reference pictures, where CPMVs of the current block are derived by scaling the collocated CPMVs to the reference picture with the scaling factor closest to 1. To generate bi-directional affine candidates for a non-low-delay B frame, the collocated CPMVs referring to only one reference list is scaled to the corresponding reference pictures in the two reference lists of the current frame.

- Proposed methods

Two aspects are proposed to enhance the derivation of affine candidate list.

Aspect #1:

Additional affine temporal candidates can be derived by scaling the collocated CPMVs to the reference picture with index 0, and the collocated CPMVs referring to both lists can be utilized to obtain the temporal affine candidates. In addition, when ARMC is applied, all temporal affine candidates are sorted based on template-matching costs, and candidates with the minimum costs are inserted into the subblock merge list.

Aspect #2:

After the subblock merge list is constructed, additional synthetic affine candidates are introduced by combining the motion of two existing affine candidates. Specifically, the CPMVs and reference index of a candidate in List 0 can be coupled with those of another candidate in List 1 to generate a new affine candidate. At most 5 synthetic candidates can be appended to the end of the current subblock merge list. The final subblock merge list is reordered if ARMC is applied.

The maximum allowed candidate number in the final subblock merge list stays unchanged and the proposed two aspects are only applied on non-low-delay B frames.

- Experimental results

The proposed method has been implemented on top of ECM-14.0 [2]. Results under RA configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method on ECM-14.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.10% | -0.08% | 0.12% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.07% | -0.17% | 0.04% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

- Conclusions

An improved affine merge candidate derivation is presented in this contribution. Simulation results on top of ECM-14.0 indicate 0.xx% coding gain in RA configurations with negligible runtime change. It is recommended to include the proposed methods in the next round of EE2.

- References

[1] C. Ma, X. Xiu, C.-W. Kuo, W. Chen, X.Wang, “EE2-3.9: Affine candidates derived from temporal collocated pictures,” JVET-AI0197, Jul. 2024.

[2] ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Jul. 2024.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).