| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0139-v2 |
| --- | --- |

| Title: | AHG9: On signalling partition information in BRI and DOI SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Tan Junghak Nam Jangwon Lee Jaehyun Lim Seung hwan Kim | Tel: Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

Bitdepth range information (BRI) SEI message and display overlays information (DOI) SEI message are currently included in the TuC for future extensions of VSEI draft. These two SEI messages include signalling of partition which may refer to the signalling that are present in other SEI message, that is constituent rectangles (CR) SEI, or in SPS of the bitstream. It is asserted that such design may have functional challenges and become an issue in cases where the CR SEI is unavailable or not intended for use, and also when the bitstream in which this BRI SEI message and / or DOI SEI message are present is coded with codec that does not support subpicture (e.g., older codecs like HEVC or AVC).

It is asserted that the design of BRI SEI message and DOI SEI message would be more robust and simpler if the signalling of partition information are not dependent on information that is present somewhere else. This contribution proposes to signal for partition information in BRI and DOI SEI message explicitly.

Problem statement

BRI (Bitdepth Range Information) and DOI (Display Overlays Information) SEI messages are currently included in the TuC for future extensions of VSEI (JVET-AI2032). In the current design, the bitdepth range and display overlays are handled through separate layer pictures, subpictures, or constituent rectangles. However, in our assertion, this design may encounter functional challenges due to its reliance on information that is present in other SEI message, that is Constituent Rectangles (CR) SEI. This could become an issue in cases where the Constituent Rectangles SEI is unavailable or not intended for use, and also when the bitstream in which this BRI SEI message and / or DOI SEI message are present is coded with codec that does not support subpicture (e.g., older codecs like HEVC or AVC).

In authors’ opinion, it would help the design of BRI and DOI SEI messages to be more robust and simpler by simply signal the partition information explicitly without dependency to other SEI message or subpicture signalling. The following sections shows modifications to the BRI and DOI SEI messages to change the signalling of partition information.

Update to the signalling of BRI SEI message

The proposed modifications are as follows:

- Exclude subpictures and constituent rectangles SEI message from the methods used to represent the bitdepth range.

- Define information that explicitly represents the bitdepth range within the BRI SEI message.

  - Define the size information of the partition.

  - Define the position information of the partition.

Text that are added are indicated by cyan highlight. Text that are removed are indicated by red, strickenthrough font.

| bitdepth_range_info( payloadSize ) { | Descriptor |
| --- | --- |
| bri_orig_bit_depth_minus1 | u(4) |
| bri_num_ranges_minus1 | u(2) |
| if ( bri_num_ranges_minus1 > 0 ) { |  |
| bri_nuh_layer_id_present_flag | u(1) |
| bri_pic_partition_flag | u(1) |
| bri_pic_partition_present_flag | u(1) |
| if( bri_pic_partition_flag ) { |  |
| if( bri_pic_partition_present_flag ) { |  |
| bri_partition_type_flag | u(1) |
| bri_partition_id_len_minus1 | u(4) |
| bri_partition_width_minus1 | ue(v) |
| bri_partition_height_minus1 | ue(v) |
| } |  |
| } |  |
| for( i = 0; i <= bri_num_ranges_minus1; i++ ) { |  |
| if ( bri_nuh_layer_id_present_flag ) |  |
| bri_nuh_layer_id[ i ] | u(6) |
| if( bri_pic_partition_flag ) { |  |
| if ( bri_pic_partition_present_flag ) { |  |
| bri_partition_id[ i ] | u(v) |
| bri_partition_top_left_x[ i ] | ue(v) |
| bri_partition_top_left_y[ i ] | ue(v) |
| } |  |
| bri_num_bits_in_range_minus1[ i ] | u(4) |
| bri_bit_offset[ i ] | u(4) |
| } |  |
| } |  |

The Bitdepth range information (BRI) SEI message enables reconstruction of a target picture of the same bit depth as an original source picture by inverting operations performed by the encoder to allocate bit ranges of the original source picture to one or more ranges coded in separate layer pictures, subpictures, or constituent rectangles.

Use of this SEI message requires the definition of the following variables, where i is the layer identifier of a layer that may be present in the current CVS:

–	A chroma format indicator, denoted herein by ChromaFormatIdc[ i ], as described in clause 7.3

–	A bit depth for the samples, denoted herein by BitDepth[ i ].

–	An array of picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ], respectively.

–	An array of subpicture counts, denoted by NumSubpics[ i ].

–	Arrays of the width and height of the subpictures, denoted herein by SubPicWidth[ i ][ j ] and SubPicHeight[ i ][ j ] respectively, where j is the subpicture index in 0 .. NumSubpics[ i ] – 1.

bri_orig_bit_depth_minus1 plus 1 specifies the bit depth of the original picture.

The variable OrigBitDepth is set equal to bri_orig_bit_depth_minus1 + 1.

bri_num_ranges_minus1 + 1 specifies the number of ranges used for allocating bits of the original picture.

The variable briNumRanges is set equal to bri_num_ranges_minus1 + 1.

bri_nuh_layer_id_present_flag equal to 1 specifies that the bri_nuh_layer_id[ i ] syntax element is present in the SEI message. bri_nuh_layer_id_present_flag equal to 0 specifies that the bri_nuh_layer_id[ i ] syntax element is not present in the SEI message.

bri_pic_partition_present_flag equal to 1 specifies that the bri_partition_width_minus1, bri_partition_height_minus1, bri_partition_top_left_x[ i ], bri_partition_top_left_y[ i ] syntax elements are present in the SEI message. bri_pic_partition_present_flag equal to 0 specifies that the bri_partition_width_minus1, bri_partition_height_minus1, bri_partition_top_left_x[ i ], bri_partition_top_left_y[ i ] syntax elements are not present in the SEI message.

When bri_nuh_layer_id_present_flag is equal to 0 and bri_num_ranges_minus1 is greater than 0, the value of bri_pic_partition_present_flag shall be equal to 1.

It is a requirement of bitstream conformance that at least one of bri_nuh_layer_id_present_flag or bri_pic_partition_present_flag shall be equal to 1 when bri_num_ranges_minus1 is greater than 0.

bri_partition_width_minus1 + 1, when present, indicates the width of the partition, which is equal to the width of the target picture in luma samples that can be reconstructed from the allocated bit ranges described in this SEI message.

bri_partition_height_minus1 + 1, when present, indicates the height of the partition, which is equal to the height of the target picture in luma samples that can be reconstructed from the allocated bit ranges described in this SEI message.

bri_pic_partition_flag equal to 1 specifies that the bri_partition_id[ i ] syntax element is present. bri_pic_partition_flag equal to 0 specifies that the bri_partition_id[ i ] syntax element is not present.

bri_partition_type_flag equal to 1 specifies that each range is coded as a constituent rectangle. bri_partition_type_flag equal to 0 specifies that each range is coded as a subpicture.

When bri_partition_type_flag equal to 1, it is a requirement of bitstream conformance that there is a constituent rectangles SEI message preceding the BRI SEI message in decoding order in the current PU.

bri_partition_id_len_minus1 + 1 specifies the length of the bri_rect_id[ i ] and bri_subpic_id[ i ] syntax elements.

bri_nuh_layer_id[ i ], when present, specifies the layer identifier of the texture component of the i-th display overlay. When not present, the value of bri_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the BRI SEI message.

If this SEI message is present in any layer in the current AU, it is a requirement of bitstream conformance that a BRI SEI message with the same payload is present in the layer with layer identifier bri_nuh_layer_id[ 0 ].

bri_partition_top_left_x[ i ], bri_partition_top_left_y[ i ] specify the horizontal and vertical positions, respectively, of the top left corner of the i-th partition in the coded picture, in luma samples.

bri_partition_id[ i ], when present and bri_partition_type_flag equal to 1, specifies the cr_rect_id[ j ] of the texture component of the i-th display overlay. bri_partition_id[ i ], when present and bri_partition_type_flag equal to 0, specifies the subpicture index of the texture component of the i-th display overlay.  When not present, the value of bri_partition_id[ i ] is inferred to be equal to 0.

When bri_partition_type_flag equal to 1, bri_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ bri_nuh_layer_id[ i ] ] – 1.  When bri_partition_type_flag equal to 0, bri_partition_id[ i ] shall be in the range of 0 to NumSubpics[ bri_nuh_layer_id[ i ] ] – 1, inclusive.

bri_num_bits_in_range_minus1[ i ] + 1 specifies the number of bits in the i-th range. bri_num_bits_in_range_minus1[ i ] shall be in the range of 0 to BitDepth[ i ] – 1, inclusive.

The variable NumBits[ i ] is set equal to bri_num_bits_in_range_minus1[ i ] + 1.

The sum of bri_num_bits_in_range_minus1[ i ] for i in 0 to bri_num_ranges_minus1 shall be less than or equal to OrigBitDepth.

bri_bit_offset[ i ] specifies a bit shifting parameter used to derive the target picture. bri_bit_offset[ i ] shall be in the range of 0 to BitDepth[ i ] – NumBits[ i ].

The variables CodedRangeRegion[ i ] CodedWidth[ i ], and CodedHeight[ i ] are derived as follows:

–	If bri_pic_partition_flag is equal to 0, the following applies:

if bri_pic_partition_present_flag is equal to 1, the following applies:

–	CodedWidth[ i ] is set equal to bri_partition_width_minus1 + 1.

–	CodedHeight[ i ] is set equal to bri_partition_height_minus1 + 1.

–	CodedRangeRegion[ i ] is set equal to the region defined by CodedWidth[ i ] and CodedHeight[ i ] and bri_partition_top_left_x[ i ], bri_partition_top_left_y[ i ].

–	Otherwise (i.e., if bri_pic_partition_present_flag is equal to 0), the following applies:

–	CodedWidth[ i ] is set equal to PicWidthInLumaSamples[ bri_nuh_layer_id[ i ] ] ].

–	CodedHeight[ i ] is set equal to PicWidthInLumaSamples[ bri_nuh_layer_id[ i ] ].

–	CodedRangeRegion[ i ] is set equal to the cropped decoded picture from the layer with layer identifier bri_nuh_layer_id[ i ] in the AU

–	Otherwise, if bri_partition_type_flag is equal to 0, the following applies:

–	CodedWidth[ i ] is set equal to SubPicWidth[ bri_nuh_layer_id[ i ] ][ bri_partition_id[ i ] ].

–	CodedHeight[ i ] is set equal to SubPicHeight[ bri_nuh_layer_id[ i ] ][ bri_partition_id[ i ] ].

–	CodedRangeRegion[ i ] is set equal to the subpicture with subpicture index bri_partition_id[ i ] from the layer with layer identifier bri_nuh_layer_id[ i ].

–	Otherwise (bri_partition_type_flag is equal to 1), the following applies:

–	CodedWidth[ i ] is set equal to CrRectWidth[ bri_nuh_layer_id[ i ] ][ bri_rect_id[ i ] ].

–	CodedHeight[ i ] is set equal to CrRectHeight[ bri_nuh_layer_id[ i ] ][ bri_rect_id[ i ] ].

–	CodedRangeRegion[ i ] is set equal to the constituent rectangle with cr_rect_id[ j ] equal to bri_partition_id[ i ] from the layer with layer identifier bri_nuh_layer_id[ i ].

It is a requirement of bitstream conformance that CodedWidth[ i ] equal CodedWidth[ j ] and CodedHeight[ i ] equal CodedHeight[ i ] for all i and j in 0 .. bri_num_ranges – 1.

The samples SampleToCode[ i ] of CodedRangeRegion[ i ], for i in 0 .. briNumRanges, are indicated to have been pre-processed prior to encoding from an original source picture of resolution CodedWidth[ 0 ] x CodedHeight[ 0 ] with chroma format ChromaFormatIdc and bit depth OrigBitDepth, according to the process of subclause 8.X.2.1.

Each sample, TargetSample, of a target picture of resolution CodedWidth[ 0 ] x CodedHeight[ 0 ] with chroma format ChromaFormatIdc and bit depth OrigBitDepth, may be reconstructed using the Target sample reconstruction process described in subclause 8.X.2.2, for each sample, DecodedSample[ i ] of CodedRangeRegion[ i ] for i in 0 .. briNumRanges.

Update to the signalling of DOI SEI message

The proposed modifications are as follows:

- Exclude subpictures and constituent rectangles SEI message from the methods used to represent the display overlays.

- Define information that explicitly represents the display overlay within the DOI SEI message.

  - Define the size information of the display overlay.

  - Define the position information of the display overlay.

Text that are added are indicated by cyan highlight. Text that are removed are indicated by red, strickenthrough font.

| display_overlays_info( payloadSize ) { | Descriptor |
| --- | --- |
| doi_id | u(6) |
| doi_cancel_flag | u(1) |
| if( !doi_cancel_flag ) { |  |
| doi_persistence_flag | u(1) |
| doi_num_display_overlays_minus2 | ue(v) |
| doi_target_pic_size_present_flag | u(1) |
| if( doi_target_pic_size_present_flag ) |  |
| doi_target_pic_width_minus1 | u(16) |
| doi_target_pic_height_minus1 | u(16) |
| } |  |
| doi_nuh_layer_id_present_flag | u(1) |
| doi_pic_partition_flag | u(1) |
| doi_pic_partition_present_flag | u(1) |
| if( doi_pic_partition_flag ) { |  |
| doi_partition_type_flag | u(1) |
| doi_partition_id_len_minus1 | u(4) |
| } |  |
| doi_offset_params_present_flag | u(1) |
| if( doi_offset_params_present_flag ) |  |
| doi_offset_param_length_minus1 | u(4) |
| doi_resampling_enabled_flag | u(1) |
| if( doi_resampling_enabled_flag ) |  |
| doi_size_param_length_minus1 | u(4) |
| for( i = 0; i < doi_num_display_overlays_minus2 + 2; i++ ) { |  |
| if ( doi_nuh_layer_id_present_flag ) |  |
| doi_nuh_layer_id[ i ] | u(6) |
| if ( doi_pic_partition_flag ) |  |
| doi_partition_id[ i ] | u(v) |
| if ( doi_pic_partition_present_flag ) { |  |
| doi_partition_top_left_x[ i ] | ue(v) |
| doi_partition_top_left_y[ i ] | ue(v) |
| doi_partition_width_minus1[ i ] | ue(v) |
| doi_partition_height_minus1[ i ] | ue(v) |
| } |  |
| doi_alpha_present_flag[ i ] | u(1) |
| if ( doi_alpha_present_flag[ i ] ) { |  |
| if (doi_nuh_layer_id_present_flag) |  |
| doi_alpha_nuh_layer_id[ i ] | u(6) |
| if (doi_pic_partition_flag) |  |
| doi_alpha_partition_id[ i ] | u(6) |
| if ( doi_pic_partition_present_flag ) { |  |
| doi_alpha_partition_top_left_x[ i ] | ue(v) |
| doi_alpha_partition_top_left_y[ i ] | ue(v) |
| doi_alpha_partition_width_minus1[ i ] | ue(v) |
| doi_alpha_partition_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( i > 0 ) { |  |
| if (doi_offset_params_present_flag ) |  |
| doi_top_left_x[ i ] | u(v) |
| doi_top_left_y[ i ] | u(v) |
| } |  |
| if( doi_resampling_enabling_flag ) { |  |
| doi_width_minus1[ i ] | u(v) |
| doi_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

The display overlays information (DOI) SEI message provides metadata to enable formation of a target display picture formed by overlaying multiple ordered display overlays in a specified order. A display overlay contains texture and optionally an alpha channel, each contained within a cropped decoded picture, a subpicture, and/or a constituent rectangle.

Use of this SEI message requires the definition of the following variables, where i is the layer identifier of a layer that may be present in the current CVS:

–	An array of picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ], respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc[ i ], as described in subclause ‎7.3.

–	An array of subpicture counts, denoted by NumSubpics[ i ].

–	Arrays of the width and height of the subpictures, denoted herein by SubPicWidth[ i ][ j ] and SubPicHeight[ i ][ j ] respectively, where j is the subpicture index in 0 .. NumSubpics[ i ] – 1.

doi_id specifies an identifier of the DOI SEI message.

doi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous DOI SEI message with the same doi_id in output order. doi_cancel_flag equal to 0 indicates that display overlays information follows.

doi_persistence_flag specifies the persistence of the DOI SEI message for the CVS.

doi_persistence_flag equal to 0 specifies that the DOI SEI message applies to the current AU only.

doi_persistence_idc equal to 1 specifies that the DOI SEI message applies to the current AU and persists for all subsequent AUs in output order until one or more of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture in the current AU with a DOI SEI message with the same value of doi_id is output that follows the current picture in output order.

doi_num_display_overlays_minus2 plus 2 specifies the number of display overlays for which information is signalled in the SEI message. The value of doi_num_display_overlays_minus2 shall be in the range of 0 to 30, inclusive.

doi_target_pic_size_present_flag equal to 1 specifies that the doi_target_pic_width_minus1 and doi_target_pic_width_minus1 syntax elements are present. doi_target_pic_size_present_flag equal to 0 specifies that the doi_target_pic_width_minus1 and doi_target_pic_width_minus1 syntax elements are present.

doi_target_pic_width_minus1 + 1, when present, indicates the width of the target picture.

doi_target_pic_height_minus1+ 1, when present, indicates the height of the target picture.

doi_nuh_layer_id_present_flag equal to 1 specifies that the doi_nuh_layer_id[ i ] syntax element is present in the SEI message. doi_nuh_layer_id_present_flag equal to 0 specifies that the doi_nuh_layer_id[ i ] syntax element is not present in the SEI message.

doi_pic_partition_flag equal to 1 specifies that display overlay components are coded as constituent rectangles or subpictures. doi_pic_partition_flag equal to 0 specifies that display overlay components are coded as pictures.

It is a requirement of bitstream conformance that at least one of doi_nuh_layer_id_present_flag or doi_pic_partition_flag shall be equal to 1.

doi_pic_partition_present_flag equal to 1 specifies that the doi_partition_width_minus1[ i ], doi_partition_height_minus1[ i ], doi_partition_top_left_x[ i ], doi_partition_top_left_y[ i ] syntax elements are present in the SEI message. Additionally, if the doi_pic_partition_present_flag is set to 1 and the doi_alpha_present_flag[ i ] is set to 1, it indicates that the syntax elements doi_alpha_partition_width_minus1, doi_ alpha_partition_height_minus1, doi_alpha_partition_top_left_x[ i ], and doi_alpha_partition_top_left_y[ i ] are present in the SEI message. doi_pic_partition_present_flag equal to 0 specifies that the doi_partition_width_minus1[ i ], doi_partition_height_minus1[ i ], doi_partition_top_left_x[ i ], doi_partition_top_left_y[ i ], doi_alpha_partition_width_minus1[ i ], doi_ alpha_partition_height_minus1[ i ], doi_alpha_ partition_top_left_x[ i ], and doi_alpha_partition_top_left_y[ i ] syntax elements are not present in the SEI message.

It is a requirement of bitstream conformance that at least one of doi_nuh_layer_id_present_flag or doi_pic_partition_present_flag shall be equal to 1.

doi_partition_type_flag equal to 1 specifies that a display overlay component is coded as a constituent rectangle. doi_partition_type_flag equal to 0 specifies that a display overlay component is coded as a subpicture.

When doi_partition_type_flag equal to 1, it is a requirement of bitstream conformance that there is a constituent rectangles SEI message preceding the DOI SEI message in decoding order in the current PU.

doi_partition_id_len_minus1 + 1 specifies the length of the doi_partition_id[ i ] syntax elements.

doi_offset_params_present_flag[ i ] equal to 1 specifies that offset parameters are present for the i-th display overlay. doi_offset_params_present_flag[ i ] equal to 0 specifies that offset parameters are not present for the i-th display overlay.

doi_offset_param_length_minus1 plus 1 specifies the length of the doi_top_left_x[ i ] and doi_top_left_y[ i ], syntax elements in bits.

doi_resampling_enabled_flag equal to 1 specifies that display overlay components may be resampled in the target display picture. doi_resampling_enabled_flag equal to 0 specifies that display overlay components are not resampled in the target display picture.

doi_size_param_length_minus1 plus 1 specifies the length of the doi_width_minus1[ i ] and doi_height_minus1[ i ] syntax elements in bits.

doi_nuh_layer_id[ i ], when present, specifies the layer identifier of the texture component of the i-th display overlay. When not present, the value of doi_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

If this SEI message is present in any layer in the current AU, it is a requirement of bitstream conformance that a DOI SEI message with the same value of doi_id and the same payload is present in the layer with layer identifier doi_nuh_layer_id[ 0 ].

doi_partition_width_minus1[ i ] + 1, when present, specifies the width of the i-th display overlay in the coded picture, in luma samples.

doi_partition_height_minus1[ i ] + 1, when present, specifies the height of the i-th display overlay in the coded picture, in luma samples.

doi_partition_top_left_x[ i ], doi_partition_top_left_y[ i ] specify the horizontal and vertical positions, respectively, of the top left corner of the i-th display overlay in the coded picture, in luma samples.

doi_partition_id[ i ], when present and doi_partition_type_flag equal to 1, specifies  that the texture component of the i-th display overlay is represented by the j-th constituent rectangle when cr_rect_id[ j ] equal doi_partition_id[ i ]. doi_partition_id[ i ], when present and doi_partition_type_flag equal to 0, specifies the subpicture index of the texture component of the i-th display overlay.  When not present, the value of doi_partition_id[ i ] is inferred to be equal to 0.

When doi_partition_type_flag equal to 1, doi_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ doi_nuh_layer_id[ i ] ] – 1.  When doi_partition_type_flag equal to 0, doi_partition_id[ i ] shall be in the range of 0 .. NumSubpics[ doi_nuh_layer_id[ i ] ] – 1.

doi_alpha_present_flag[ i ] equal to 1 specifies that an alpha component is provided for the i-th display overlay. doi_alpha_present_flag[ i ] equal to 0 specifies that an alpha component is not provided for the i-th display overlay.

doi_alpha_nuh_layer_id[ i ], when present, specifies the layer identifier value of the alpha component of the i-th display overlay. When not present, the value of doi_alpha_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

doi_alpha_partition_id[ i ], when present and doi_partition_type_flag equal to 1, specifies the cr_rect_id[ j ] of the alpha component of the i-th display overlay. doi_partition_id[ i ], when present and doi_partition_type_flag equal to 0, specifies the subpicture index of the alpha component of the i-th display overlay.  When not present, the value of doi_alpha_partition_id[ i ] is inferred to be equal to 0.

When doi_partition_type_flag equal to 1, doi_alpha_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ doi_nuh_layer_id[ i ] ] – 1.  When doi_partition_type_flag equal to 0, doi_alpha_partition_id[ i ] shall be in the range of 0 .. NumSubpics[ doi_nuh_layer_id[ i ] ] – 1.

doi_alpha_partition_width_minus1[ i ] + 1, when present, specifies the width of alpha component of  the i-th display overlay in the coded picture, in luma samples.

doi_alpha_partition_height_minus1[ i ] + 1, when present, specifies the height of alpha component of  the i-th display overlay in the coded picture, in luma samples.

doi_alpha_partition_top_left_x[ i ], doi_alpha_partition_top_left_y[ i ] specify the horizontal and vertical positions, respectively, of the top left corner of alpha component of the i-th display overlay in the coded picture.

doi_top_left_x[ i ] and doi_top_left_y[ i ] specify the horizontal and vertical positions, respectively, of the top left corner of the i-th display overlay in the target display picture, in luma samples. When not present, the values of doi_top_left_x[ i ] and doi_top_left_y[ i ] are inferred to be equal to 0. The length of the syntax elements is doi_offset_param_length_minus1 + 1 bits.

doi_width_minus1[ i ] plus 1 and doi_height_minus1[ i ] plus 1, when present, specify the width and height, respectively, in luma samples arrays of the i-th display overlay in the target display picture. The length of the syntax elements is doi_size_param_length_minus1 + 1 bits.

The variables CodedOverlayTexture[ i ] and CodedOverlayAlpha[ i ] are picture sample arrays with luma resolution CodedOverlayWidth[ i ] × CodedOverlayHeight[ i ], derived as follows:

If doi_pic_partition_flag is equal to 0, the following applies:

If doi_pic_partition_present_flag is equal to 0, the following applies:

–	CodedOverlayWidth[ i ] is set equal to PicWidthInLumaSamples[ doi_nuh_layer_id[ i ] ] ].

–	CodedOverlayHeight[ i ] is set equal to PicWidthInLumaSamples[ doi_nuh_layer_id[ i ] ].

–	If there is a picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ], CodedOverlayTexture[ i ] is set equal to the cropped decoded picture from the layer with layer identifier doi_nuh_layer_id[ i ] in the AU. Otherwise (there is no picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ]), CodedOverlayTexture[ i ] is set equal to the previous cropped decoded picture in output order in the layer with layer identifier doi_nuh_layer_id[ i ].

–	Otherwise, the following applies:

–	If doi_alpha_present_flag[ i ] equal to 1, the following applies:

- If there is a picture in the AU for the layer with layer identifier doi_alpha_nuh_layer_id[ i ], CodedOverlayAlpha[ i ]  is set equal to the cropped decoded picture from the layer with layer identifier doi_alpha_nuh_layer_id[ i ] in the AU.

- Otherwise (there is no picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ]), CodedOverlayAlpha[ i ] is set equal to the previous cropped decoded picture in output order in the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

If doi_pic_partition_present_flag is equal to 1, the following applies:

–	CodedOverlayWidth[ i ] is set equal to doi_partition_width_minus1[ i ] + 1.

–	CodedOverlayHeight[ i ] is set equal to doi_partition_height_minus1[ i ] + 1.

–	CodedOverlayTexture[ i ] is set equal to the region defined by CodedOverlayWidth[ i ], CodedOverlayHeight[ i ], doi_partition_top_left_x[ i ] and doi_partition_top_left_y[ i ].

–	If doi_alpha_present_flag[ i ] equal to 1, the following applies:

–	CodedOverlayAlphaWidth[ i ] is set equal to doi_alpha_partition_width_minus1[ i ] + 1.

–	CodedOverlayAlphaHeight[ i ] is set equal to doi_ alpha_partition_height_minus1[ i ] + 1.

–	CodedOverlayAlpha[ i ] is set equal to the region defined by CodedOverlayAlphaWidth[ i ], CodedOverlayAlphaHeight [ i ], doi_alpha_partition_top_left_x[ i ] and doi_alpha partition_top_left_y[ i ].

Otherwise, if doi_partition_type_flag is equal to 0, the following applies:

–	CodedOverlayTexture[ i ] is set equal to the subpicture with subpicture index doi_partition_id[ i ] from the layer with layer identifier doi_nuh_layer_id[ i ].

–	CodedOverlayWidth[ i ] is set equal to SubPicWidth[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayHeight[ i ] is set equal to SubPicHeight[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	If doi_alpha_present_flag[ i ] equal to 0, the following applies:

–	CodedOverlayAlphaWidth[ i ]  is set equal to SubPicWidth[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlphaHeight[ i ]  is set equal to SubPicHeight[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlpha[ i ]  is set equal to the subpicture with subpicture index doi_alpha_partition_id[ i ] from the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

–	Otherwise (doi_partition_type_flag is equal to 1), the following applies:

–	CodedOverlayWidth[ i ] is set equal to CrRectWidth[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayHeight[ i ] is set equal to CrRectHeight[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayTexture[ i ] is set equal to the constituent rectangle with cr_rect_id[ j ] equal to doi_partition_id[ i ] from the layer with layer identifier doi_nuh_layer_id[ i ].

–	If doi_alpha_present_flag[ i ] equal to 0

–	CodedOverlayAlphaWidth[ i ] is set equal to CrWRectidth[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlphaHeight[ i ] is set equal to CrRectHeight[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlpha is set equal to the constituent rectangle with cr_rect_id[ j ] equal to doi_alpha_partition_id[ i ] from the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).