| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0141-v1 |
| --- | --- |

| Title: | AHG11: Adaptive skip of LOP filtering based on boundary strength partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyukmin Kwon Juyeon Seo Hyunsuk Ko (HYU) Donghyun Kim Sung-Chang Lim (ETRI) | Tel: Email: | +82-31-400-5162 hspeedkwon@hanyang.ac.kr hyeseo@hanyang.ac.kr hyunsuk@hanyang.ac.kr kimddng@etri.re.kr sclim@etri.re.kr |
| Source: | Hanyang University and Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution proposes a method to reduce the decoding time of the LOP filter by adaptively skipping LOP filtering based on boundary strength (BS) partitions. The proposed method determines the number of BS partitions from the BS map, which is used for an input to the LOP architecture, and skips the LOP filtering if the number of BS partitions is lower than BS partition threshold that is signalled in slice header. The experimental results {Y, Cb, Cr | EncT, DecT} of the proposed method based on NNVC-10.0 are reported as follows:

RA: {Y: 0.10% Cb: -0.11% Cr: -0.15% | EncT: 100% DecT: 75%},

LDB: {Y: -0.02% Cb: -1.40% Cr: -1.27% | EncT: 100% DecT: xx%}

Introduction

Compared to the VVC test model (VTM), the coding performance of NNVC has been improved through the use of NN intra and the NN in-loop filter. However, the NN in-loop filter has also increased the complexity of the decoder. To address this complexity, the proposal JVET-AF0193 [1], which disables the NN in-loop filtering for the blocks with small filter gains, was adopted at the 32nd JVET meeting to reduce the decoding time of the NN in-loop filter. However, since this method can only be applied to block-level NN in-loop filtering, it is not activated for slice-level NN in-loop filtering. Therefore, we propose a method that can be applied to both slice-level and block-level NN in-loop filtering. This proposal presents a method to reduce the decoding time of the LOP filter. Additionally, this method can also be applied to both HOP and VLOP filters.

Proposed method

In NNVC, the LOP filtering produces residue samples, which are added to the reconstructed samples to generate the LOP filtered samples [2]. Analysis of the residue samples shows that those with non-zero values after applying the LOP filter form the edges, as shown in Figure 1. In Figure 1, the gray samples represent those with non-zero values after applying the LOP filter, while the black samples indicate those with zero values.

Figure 1. Example of residue samples

Furthermore, the edges formed by residue samples are aligned with the BS edges in the BS map, which is used for an input to the LOP architecture, as illustrated in Figure 2. In Figure 2, the white edges indicate edges with BS equals to 2 and blue edges correspond to BS equals to 1 in the BS map.

Figure 2. Example of BS map

From Figures 1 and 2, the gray samples tend to cluster along these BS edges, and having many BS edges in a certain area can imply that there are also many BS partitions enclosed by those edges. Therefore, it means that LOP filtering is more effective in areas with many BS partitions. In contrast, if there are more black samples than gray samples in a certain area, resulting in fewer BS partitions, it can be assumed that LOP filtering is less effective there. Figure 3 illustrates an example of BS partitions formed by BS edges in the BS map, with the numbers in red indicating the count of BS partitions. In the area highlighted by the red box, the total number of BS partitions is 12.

Figure 3. Example of the number of BS partitions in BS map

Therefore, it is proposed to adaptively apply LOP filtering based on the number of BS partitions to reduce the decoding time of LOP filter.

Skipping LOP filtering based on the number of BS partitions

The proposed method skips the LOP filtering if the number of BS partitions is lower than BS partition threshold as follows:

Threshold># of BS partition

If the above condition is true, the LOP filtering is not applied to the filtering block (e.g., 128x128 or 256x256). Conversely, if the condition is false, the LOP filtering is applied on the filtering block.

To adaptively apply the proposed method at the slice level, it is proposed to signal the bs_block_skip_enabled_flag, which indicates the use of the proposed method, along with the BS partition threshold information represented by bs_partition_threshold_minus2 in the slice header, as shown in Table 1. Note that the proposed method is not applied to I-slice.

Table 1. Signalling of BS partition threshold in slice header

| Slice_header(){ | Descriptor |
| --- | --- |
| … |  |
| if( sps_nnlf_unified_enabled_flag == 1 ){ |  |
| slice_nnlf_unified_mode | ue(v) |
| slice_nnlf_unified_mode-- |  |
| if( slice_nnlf_unified_mode != -1 ){ |  |
| ... |  |
| if( sh_slice_type != I_SLICE ){ |  |
| bs_block_skip_enabled_flag | u(1) |
| if( bs_block_skip_enabled_flag == 1 ) |  |
| bs_partition_threshold_minus2 | u(8) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

Finding the optimal threshold value

The BS partition threshold can have different optimal values for each slice. To find the optimal threshold value, the following equation is used.

Cost=D+λ*(# of LOP filtered blocks)

where D represents the distortion between the original samples and the final filtered samples in a slice, where the final filtered samples may include both LOP filtered and unfiltered samples. λ is a pre-defined value based on the slice QP, used only in the encoder, and the number of LOP filtered blocks is controlled by the BS partition threshold. The optimal threshold is determined by minimizing the cost in the equation above and is signalled in the slice header. When the high threshold is determined, the number of filtered blocks decreases.

This process is referred to as time distortion optimization (TDO), where time is proportional to the number of blocks being filtered.

Inference information

Table 2 shows the network inference information of LOP3 filter. The proposed method does not train and change the LOP3 network architecture.

Table 2. Network information in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | N/A |
|  | Framework | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 205108 |
|  | Parameter Precision (Bits) | int: 16 |
|  | Memory Parameter (MB) | int: 0.2 MB |
|  | Multiply Accumulate (kMAC/pixel) | 16.9 |
| Optional | Total Conv. Layers | 96 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information |  |

Experimental results

The proposed method was implemented on top of NNVC-10.0 [3] and tested under JVET NNVC CTC [4]. The experimental results of the proposed method are summarized in Table 3. In these results, it reduced the decoding time while showing minimal average luma loss and achieving some average gain in chroma.

Table 3. Experimental results of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 with LOP3 & NN intra |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.09% | -0.04% | -0.18% | 101% | 79% |
| Class A2 | 0.10% | -0.10% | -0.03% | 101% | 76% |
| Class B | 0.10% | -0.15% | -0.24% | 100% | 73% |
| Class C | 0.10% | -0.13% | -0.11% | 100% | 75% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | -0.11% | -0.15% | 100% | 75% |
| Class D | 0.11% | 0.09% | -0.14% | 100% | 84% |
| Class F | 0.08% | 0.09% | 0.03% | 101% | 75% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 with LOP3 & NN intra |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -1.17% | -1.12% | 100% | #DIV/0! |
| Class C | -0.06% | -1.47% | -1.30% | 99% | #DIV/0! |
| Class E | 0.00% | -1.68% | -1.47% | 100% | #DIV/0! |
| Overall | -0.02% | -1.40% | -1.27% | 100% | #DIV/0! |
| Class D | 0.04% | -1.40% | -0.40% | 99% | #DIV/0! |
| Class F | -0.14% | -0.79% | -0.90% | 100% | #DIV/0! |

Conclusion

This contribution proposed the method that adaptively skips LOP filtering based on BS partitions to reduce the decoding time of the LOP filter. The proposed method determines the number of BS partitions from the BS map, which is used for an input to the LOP architecture and skips the LOP filtering if the number of BS partitions is lower than BS partition threshold that is signalled in slice header. The experimental results show that the proposed method can effectively reduce decoding time with minimal loss. Thus, it is recommended to be studied in the next round of EE1.

Reference

- R. Chang, L. Wang, X. Xu and S. Liu, “AHG11: Decoder complexity optimization for NNVC in-loop filter”, JVET-AF0193, Oct. 2023.

- F. Galpin, Y. Li, D. Rusanovskyy, J. Ström and L. Wang, “Description of algorithms version 8 and software version 10 in neural network-based video coding (NNVC)”, JVET-AI2019, Jul. 2024.

- NNVC-10.0, https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/v10

- E. Alshina, R.-L. Liao, S. Liu and A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AI2016, Jul. 2024.

Patent rights declaration(s)

Hanyang University and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).