| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0143-v1 |
| --- | --- |

| Title: | EE2-2.17: Modified L-shape template in IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Jongho Kim Woong Lim Donghyun Kim Sung-Chang Lim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr pooney@etri.re.kr woong.lim@etri.re.kr kimddng@etri.re.kr sclim@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution reports the EE2 test 2.17 results for the modified L-shape template in IntraTMP, as proposed in JVET-AI0139. The proposed method excludes top-left samples from the L-shape template. This exclusion affects the calculation of SAD and MRSAD as well as the parameter derivation process in FLM mode. The EE2 test 2.17 results {Y, U, V, EncT, DecT}, based on ECM-14.0, are reported as follows:

AI: {-0.01%, 0.01%, -0.03%, 99.2%, 100.0%};

RA: {0.00%, -0.09%, 0.04%, 99.8%, 99.9%}.

Introduction

In IntraTMP, there are three template types: L-shape, Top, and Left. In the L-shape template, top, left, and top-left samples of a current block are used to calculate SAD and MRSAD, and to derive FLM parameters. At the previous meeting, JVET-AI0139 [1] proposed a modified L-shape template that excludes top-left samples, as illustrated in Figure 1. The top-left samples can have a large impact on small blocks when finding the best matching block, while their impact on larger blocks is minimal. For small blocks, excluding these samples from the L-shape template could better reflect the current block and increase prediction efficiency. This modification also reduces computational complexity when calculating SAD and MRSAD, as well as in the FLM parameter derivation process. Additionally, the current ECM-14.0 SIMD implementation already separates top-left and top samples, making it straightforward to exclude the top-left part.

(a) ECM-14.0           (b) Proposal

Figure 1. An example of L-shape template for 4x4 block

Experimental results

The proposed method was implemented on top of ECM-14.0 [2] and tested under JVET ECM CTC [3]. The EE2 test 2.17 results are summarized in Table 1.

Table 1. Experimental results of EE2 test 2.17 over ECM-14.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.00% | -0.03% | 99.7% | 100.2% | 100.9% | 100.1% |
| Class A2 | 0.00% | -0.04% | -0.06% | 100.8% | 99.7% | 100.4% | 100.1% |
| Class B | -0.01% | -0.02% | 0.03% | 96.8% | 100.0% | 100.0% | 99.9% |
| Class C | 0.00% | -0.04% | -0.10% | 100.8% | 100.2% | 99.5% | 100.0% |
| Class E | -0.01% | 0.18% | 0.00% | 99.2% | 99.9% | 100.0% | 100.0% |
| Overall | -0.01% | 0.01% | -0.03% | 99.2% | 100.0% | 100.1% | 100.0% |
| Class D | -0.03% | 0.24% | 0.03% | 100.4% | 100.4% | 100.1% | 100.0% |
| Class F | -0.04% | 0.04% | -0.09% | 99.4% | 99.7% | 99.5% | 100.0% |
| Class TGM | -0.06% | -0.09% | -0.08% | 98.0% | 99.8% | 100.0% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.30% | -0.19% | 100.2% | 100.2% | 100.1% | 100.3% |
| Class A2 | 0.06% | -0.14% | -0.04% | 100.2% | 99.7% | 100.1% | 100.4% |
| Class B | -0.02% | 0.07% | 0.06% | 99.3% | 99.9% | 99.9% | 99.8% |
| Class C | -0.02% | -0.11% | 0.24% | 99.8% | 99.9% | 100.4% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.09% | 0.04% | 99.8% | 99.9% | 100.1% | 100.1% |
| Class D | 0.00% | -0.11% | -0.10% | 99.3% | 100.2% | 100.1% | 100.0% |
| Class F | 0.02% | -0.29% | -0.09% | 99.8% | 100.3% | 100.2% | 100.0% |
| Class TGM | -0.08% | -0.04% | -0.10% | 99.9% | 99.8% | 99.9% | 100.1% |

Conclusions

This contribution reports the test results of EE2 test 2.17, which involves the modified L-shape template in IntraTMP. The experimental results show the average luma BD-rate change of -0.01% for AI, along with reduced encoding and decoding complexity. It is suggested to adopt the proposed method in the next version of ECM.

References

- J. Lee, J. Kim, W. Lim, D. Kim, and S.-C. Lim, “Non-EE2: Modified L-shape template in IntraTMP”, JVET-AI0139, Jul. 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AI2017, Jul. 2024.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).