| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0144-v1 |
| --- | --- |

| Title: | EE2-5.2: Using luma reconstruction signal for chroma ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports results of EE2 Test-5.2. In the test, the performance of using luma reconstruction signal for chroma ALF is evaluated. The tested method is implemented on top of ECM-14.0 and the simulation results are summarized as follows:

Test-5.2:

AI: xxx%, xxx%, xxx%, xxx%, xxx%.

RA: xxx%, xxx%, xxx%, xxx%, xxx%.

LB: xxx%, xxx%, xxx%, xxx%, xxx%.

Introduction

In ECM14.0, for chroma components, a fixed filter in a 9x9 diamond shape is applied to chroma reconstruction signal and before deblocking filter (DBF) signal to obtain an intermediate signal. Then a chroma ALF filter is signalled and takes chroma reconstruction signal and this intermediate signal as the input. Figure 1 presents the specific chroma ALF filters applied in ECM14.0, where the 9x9 diamond shape with filter taps 0-19 corresponds to chroma reconstruction signal and 5x5 cross shape with filter taps 20-24 corresponds to the intermediate signal generated by the fixed filter.

Figure 1. chroma ALF filter shape in ECM14.0.

At the JVET-AI meeting, using luma reconstruction signal for chroma ALF is proposed [1]. This method was included in Exploration Experiment (EE) 2 as Test-5.2 for further evaluation. This contribution reports the corresponding results of this EE test.

Proposal

In EE2-5.2, it is proposed to additionally utilize the luma reconstruction signal as additional inputs to the chroma ALF. Specifically, the luma reconstruction signal is firstly down-sampled to have the save resolution with the chroma signal, where the down-sampling filter is [1:2:1; 1:2:1]/8. Then, a 5x5 cross filter shape are applied to the down-sampled luma reconstruction signal and added to the chroma ALF filter. Figure 2 presents the chroma ALF filter shape in the proposed method.

Figure 2. chroma ALF filter shape in the proposed method.

Simulation results

The proposed method was implemented on ECM-14.0 reference software [2] and tested by following the common test condition (CTC) as specified in [3]. The results of 5.2 compared to ECM-14.0 are shown in Table 1.

Table 1 Performance of EE2-5.2

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.13% | -0.18% |  |  |  |  |
| Class E | 0.01% | -0.17% | -0.12% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.01% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.18% | -0.18% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | -0.01% | 0.05% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

|  | Low Delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.03% | 0.02% | -0.32% |  |  |  |  |
| Class E | -0.03% | -0.48% | -0.45% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.07% | 0.65% | 0.27% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

References

- C. Ma, X. Xiu, X. Wang, “ Non-EE2: Use Luma Reconstruction Signal for Chroma ALF”, JVET-AI0200, Sapporo, JP, 12–19 July 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation JVET procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).