| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0152-v2 |
| --- | --- |

| Title: | AhG12: An alternative TIMD mode from common reference samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongdong Qin Keqin Ding Jacek Konieczny Alexey Filippov Vasily Rufitskiy Tianyu Dong | Email: | hongdong.qin@tcl.com keqin.ding@tcl.com jacek.k@tcl.com alexey.filippov@tcl.com vasily.rufitskiy@tcl.com tianyu.dong@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

It is proposed to add an alternative TIMD mode from common reference samples (TIMD-CRS) as a sub-mode of TIMD, where the derivation of TIMD modes uses the same reference lines as when the current block is predicted.

The experimental results over ECM-14.0 are summarized as follows:

AI:		-0.07% (Y), 0.02% (U), 0.04% (V)

# Introduction

In ECM, template-based intra mode derivation (TIMD) [1] has been introduced. TIMD uses a template to evaluate regular intra prediction modes (IPM) by comparing predictors on the templates against reconstructed samples. The predictors on the templates are generated from the reference line adjacent to the templates.

Figure 1 TIMD templates and reference samples

# Proposal

It is proposed to introduce an alternative TIMD method, namely TIMD with common reference samples (TIMD-CRS). In TIMD-CRS, the reference line for generating predictors on the templates is the same as the reference line for generating predictors on the current block.

Figure 2 TIMD-CRS templates and reference samples

In Planar mode, some reference samples are mirrored with respect to the reference line: for above template, top-left reference sample is used in place of bottom-left reference sample for predicting current block; for left template, top-left reference sample is used in place of top-right reference sample for predicting current block.

DC mode is unchanged.

For some angular modes and some templates, the prediction direction shall be reversed and point to the reference lines.

PDPC is applied on negative modes, where the sample to be predicted is located between the intersects of the prediction line and the reference lines.

The rest of the mode derivation process (SATD-based cost evaluation, mode sorting, fusion rules) are the same as TIMD.

# Experimental results

The proposed method has been implemented on top of the ECM-14.0 [2]. The simulations were performed following the common test conditions [3] and summarized in the below table.

Test: One RDO is added if RD cost of TIMD-CRS < 1.5 * RD cost of TIMD.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | 0.03% | -0.11% |  |  |  |  |
| Class A2 (-1) | -0.03% | 0.01% | 0.01% |  |  |  |  |
| Class B | -0.06% | 0.00% | 0.12% |  |  |  |  |
| Class C | -0.09% | 0.01% | -0.09% |  |  |  |  |
| Class E | -0.08% | 0.10% | 0.27% |  |  |  |  |
| Overall | -0.07% | 0.02% | 0.04% |  |  |  |  |
| Class D | -0.09% | 0.12% | -0.01% |  |  |  |  |
| Class F | -0.12% | -0.20% | 0.10% |  |  |  |  |

# Conclusion

It is proposed to introduce an alternative TIMD mode where the reference lines for the template are the same as those used for predicting the current block.

It is recommended to study this contribution in the next round of EE2.

# References

- Y. Wang, L. Zhang, K. Zhang, Z. Deng, N. Zhang, “EE2-2.1: Results for template-based intra mode derivation using MPMs”, JVET-W0053, Jul. 2021

- ECM-14.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AI2017, Jul. 2024

- D. Bugdayci Sansli, S. Blasi, J. Lainema, “EE2-2.6: Additional TIMD mode with different cost metric”, JVET-AJ0146, Nov. 2024

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).