| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0155-v2 |
| --- | --- |

| Title: | AhG12: SGPM Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keqin Ding Hongdong Qin Jacek Konieczny Alexey Filippov Vasily Rufitskiy Tianyu Dong | Email: | keqin.ding@tcl.com hongdong.qin@tcl.com jacek.k@tcl.com alexey.filippov@tcl.com vasily.rufitskiy@tcl.com tianyu.dong@tcl.com |
| Source: | TCL |  |  |

_____________________________

- Abstract

It is proposed to introduce multiple improvements to SGPM including: context coding of SGPM index; adaptation of virtual IPM (VIPM) derivation and fast encoding to SGPM.

Introduction

Spatial geometric partitioning mode (SGPM) has been introduced in JVET-AB0155 [1] as bi-prediction mode that behaves similarly to geometric partitioning mode but uses intra modes in both partitions.

In SGPM mode, the decoder receives a 4-bit uniformly coded SPGM index to determine the combination of two intra modes and a partition line from a list derivable at the decoder side.

As to transform coding, the LFNST transform kernel is determined by the direction of its partitioning mode.

Proposal

It is proposed to introduce multiple improvements regarding entropy coding and encoding optimization for SGPM.

Context coding for SGPM index

It is proposed to introduce binary coding with context for SGPM index. Four contexts are used to indicate the relative probability of each bin. The codewords and contexts for SGPM index coding are shown in the table below:

| SPGM index | Code word | Context |
| --- | --- | --- |
| 0 | 0000 | C0 C1 C2 C3 |
| 1 | 0001 | C0 C1 C2 C3 |
| [2, 3] | 001 + 1-bit binary | C0 C1 C2 |
| [4, 7] | 01 + 2-bit binary | C0 C1 |
| [8, 15] | 1 + 3-bit binary | C0 |

HoG-based VIPM derivation

It is proposed to apply HoG-based VIPM derivation as in JVET-AB0067 [2] to SGPM.

The DIMD-liked method is used to derive a VIPM based on the SGPM prediction samples, then the MTS transform set or the LFNST transform set is respectively determined by the derived VIPM. Generation of MPM list using the derived VIPM is also proposed. If a neighbouring block is coded by SGPM, the derived VIPM of that block is used to fill in current MPM.

LFNST/MTS fast encoding algorithm

It is proposed to apply LFNST/MTS fast encoding algorithm as in JVET-Y0141 [3] to SGPM. The threshold is set to 1.5 for skipping the LFNST and MTS processes for SGPM.

Experimental results

The proposed method has been implemented on top of the ECM-14.0 [4]. The simulations were performed following the common test conditions [5] and summarized in the below table.

Test: Multiple SGPM improvements

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.14% | -0.04% | 100.5% | 101.1% | 100.1% | 100.0% |
| Class A2 | -0.02% | -0.02% | -0.01% | 100.4% | 101.2% | 100.8% | 100.1% |
| Class B | -0.03% | 0.05% | 0.04% | 100.7% | 100.6% | 103.0% | 100.1% |
| Class C | -0.03% | -0.06% | -0.02% | 100.4% | 101.2% | 100.0% | 100.0% |
| Class E | -0.05% | 0.06% | 0.16% | 101.1% | 101.7% | 100.0% | 100.0% |
| Overall | -0.03% | -0.01% | 0.02% | 100.6% | 101.1% | 101.0% | 100.0% |
| Class D | -0.02% | 0.06% | -0.04% | 100.6% | 101.4% | 100.0% | 100.0% |
| Class F (-3) | -0.08% | -0.18% | -0.02% | 100.3% | 101.1% | 97.9% | 100.0% |

Conclusion

It is proposed to introduce context coding of SGPM index; adaptation of virtual IPM (VIPM) derivation and fast encoding to SGPM.

It is recommended that the proposed methods be studied in the next round of EE2.

References

[1] F. Wang, Y. Yu, H. Yu, D. Wang, A.Natesan, J. N. Shingala, Jeeva Raj Arumugam, Vaibhav Valvaiker, T. Lu, F. Pu, P. Yin, S. McCarthy (Dolby), K. Naser, Y. Chen, A. Robert K. Reuzé, “EE2-1.6: Combination of spatial GPM tests”, JVET-AB0155, Oct. 2022.

[2] J.-Y. Huo, W.-H. Qiao, X. Hao, Y.-Z. Ma, F.-Z. Yang, J. Ren, M. Li, L.-H. Xu, “EE2-4.1: Modification of LFNST for MIP coded block”, JVET-AB0067, Oct. 2022.

[3] J. Chen, Y. Ye, R.-L. Liao, X. Li, X. Xiu, Y.-W. Chen, N. Yan, C.-W. Kuo, H.-J. Jhu, W. Chen, X. Wang, “EE2-4.3 related: More combined test results for sign prediction”, JVET-Y0141, Jan. 2022.

[4] ECM-14.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

[5] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AI2017, Jul. 2024.

Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).