| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 |  | Document: JVET-AJ0156-v1 |
| --- | --- | --- |

| Title: | Non-EE2 Adaptive interpolation filter for reference sample in angular mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tianyu Dong Alexey Filippov Jacek Konieczny Hongdong Qin Keqin Ding Nanshan, Shenzhen, Guangdong 518052 People’s Republic of China | Tel: Email: | tianyu.dong@tcl.com jacek.k@tcl.com hongdong.qin@tcl.com alexey.filippov@tcl.com keqin.ding@tcl.com |
| Source: | TCL |  |  |

_____________________________

- Abstract

This document reports the results of applying 8-tap interpolation filter with adaptive condition on reference sample filtering in angular mode of intra prediction. The performance is listed below.

AI{-0.050%,0.01%,0.01%,100.2%,99.0%,100.8%,100.0%}

RA{Class B,-0.01%,-0.06%,-0.12%; Class C,-0.05%,-0.20%,0.22%}

Introduction

In current angular mode prediction of ECM [1], a 6-tap interpolation filter is used in the process for reference sample. In this proposal, an 8-tap interpolation filter are implemented with conditional switch on top of 6-tap filter [2].

Proposed method

32 phases and 64 phases 8-tap interpolation filter is implemented in the function of xPredIntraAng() in  ECM14. Where several tools are affected by applying this filter, which includes DIMD, OBIC, TIMD, SGPM, and conventional Intra prediction. Two conditions are checked whether the current block will be processed by 8-tap filter.

- First condition is the angular mode index difference with horizontal and vertical mode. This difference shows how the current angle farther from the horizontal or vertical.

min( abs( intraPredAngle - (bExtIntraDir ? EXT_HOR_IDX : HOR_IDX) ), abs( intraPredAngle - (bExtIntraDir ? EXT_VER_IDX : VER_IDX) ) )

And the threshold for this difference is derived as a log2size of width and height. The threshold applied separately by bExtIntraDir. Where we use half of the threshold of extra direction mode for normal directional mode.

log2Size = ((floorLog2(width) + floorLog2(height)) >> 1) diff > ((bExtIntraDir ? log2Size : log2Size/2))

- Second condition is that, a block size check is done as follow. Where we only apply 8-tap filter on smaller blocks.

log2Size < ( bExtIntraDir ? 6 : 4) //32xN blocks, 8xN blocks

Once these conditions are satisfied, 8-tap interpolation filter will be used, otherwise existing 6-tap filter will be used. For example, for a 32x32 blocks the filter selection might looks like the figure below.

Figure 1 32x32 block with possible directions. (blue slope, 8-tap filer; red slope 6-tap filter)

Experimental results

The result shows that an average of -0.050% coding is observed in CTC AI condition [3]. A partial result of RA is also listed below. The proposed method shows similar complexity to anchor.

Table I. Experiment BDBR result.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.023% | 0.099% |  | -0.029% |  | 100.5% |  | 99.5% |  | 101.7% |  | 100.0% |  |
| Class A2 | -0.015% | -0.021% |  | -0.023% |  | 99.9% |  | 99.7% |  | 100.0% |  | 100.0% |  |
| Class B | -0.023% | 0.004% |  | 0.071% |  | 99.9% |  | 99.4% |  | 102.0% |  | 100.1% |  |
| Class C | -0.107% | 0.000% |  | -0.060% |  | 100.2% |  | 99.1% |  | 100.5% |  | 100.0% |  |
| Class E | -0.081% | -0.03% |  | 0.08% |  | 100.6% |  | 97.0% |  | 98.9% |  | 100.0% |  |
| Overall | -0.050% | 0.01% |  | 0.01% |  | 100.2% |  | 99.0% |  | 100.8% |  | 100.0% |  |
| Class D | -0.079% | -0.093% |  | -0.159% |  | 98.1% |  | 101.0% |  | 100.0% |  | 100.0% |  |
| Class F | -0.116% | -0.219% |  | -0.047% |  | 100.1% |  | 100.2% |  | 99.1% |  | 100.1% |  |
| Class TGM | 0.000% | -0.056% |  | -0.050% |  | 100.2% |  | 99.3% |  | 102.5% |  | 100.1% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |
| Class B | -0.01% |  | -0.06% |  | -0.12% |  | 100.5% |  | 100.5% |  | 99.9% |  | 99.9% |
| Class C | -0.05% |  | -0.20% |  | 0.22% |  | 100.1% |  | 97.8% |  | 99.6% |  | 100.0% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |
| Class D | -0.05% |  | 0.00% |  | 0.05% |  | 100.3% |  | 100.3% |  | 100.1% |  | 100.0% |
| Class F | 0.07% |  | 0.07% |  | 0.12% |  | 99.0% |  | 97.3% |  | 99.9% |  | 100.1% |
| Class TGM | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |

Conclusion

It is recommended to include this proposal into next round of EE2.

- ECM version 14 https://vcgit.hhi.fraunhofer.de/ecm/ECM, Sep 2024.

- Hongtao Wang, Vadim Seregin, Marta Karczewicz, “EE2-1.11: Intra prediction fusion” JVET-AB0148, Oct. 2022.

- Marta Karczewicz, Yan Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

TCL  may  have  current  or  pending  patent  rights  relating  to  the  technology  described  in  this contribution  and,  conditioned  on  reciprocity,  is  prepared  to  grant  licenses  under  reasonable  and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation ISO/IEC  International  Standard  (per  box  2  of  the  ITU-T/ITU-R/ISO/IEC  patent  statement  and licensing declaration form).