| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ00158-v2 |
| --- | --- |

| Title: | EE2-3.9: Sub-block inter mode improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yan Zhang, Han Huang, Zhi Zhang, Po-Han Lin, Chun-Chi Chen, Vadim Seregin, Marta Karczewicz | Email: | jilin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of EE2-3.9. It proposes to add CPMV search process in affine DMVR, high precision MV for affine motion compensation, and additional combined merge candidates. The proposed method was implemented on top of ECM-14.0 and the test results are as follows:

Test 3.9a, Affine CPMV search for affine DMVR

RA: -0.03% (Y), -0.03% (U), -0.08% (V), 100.1% (EncT), 100.3% (DecT), 100.0% (EncVmPeak), 99.9% (DecVmPeak)

Test 3.9b, High precision MV for affine motion compensation:

RA: -0.03% (Y), -0.03% (U), -0.01% (V), 100.7% (EncT), 100.6% (DecT), 99.9% (EncVmPeak), 100.1% (DecVmPeak)

LB: -0.09% (Y), 0.25% (U), -0.50% (V), 100.9% (EncT), 100.8% (DecT), 99.7% (EncVmPeak), 99.9% (DecVmPeak)

Test 3.9c, Additional combined merge candidates:

RA: -0.02% (Y), 0.02% (U), 0.06% (V), 100.1% (EncT), 100.4% (DecT), 100.5% (EncVmPeak), 100.3% (DecVmPeak))

Test 3.9d, combination of Test 3.9a, Test 3.9b and Test 3.9c:

RA: -0.08% (Y), -0.09% (U), -0.06% (V), 100.7% (EncT), 100.9% (DecT), 100.5% (EncVmPeak), 100.4% (DecVmPeak)

LB: -0.09% (Y), 0.25% (U), -0.50% (V), 100.9% (EncT), 100.8% (DecT), 99.7% (EncVmPeak), 99.9% (DecVmPeak)

# Introduction

In ECM-14.0, affine DMVR is applied to the affine merge mode when the DMVR enabling condition is satisfied. The affine control point motion vectors (CPMVs) have the same precision as regular motion vector, which is 1/16-pel. When performing affine motion compensation, per-pixel or subblock based motion vector is derived by the control point motion vectors and is quantized to the same 1/16-pel precision.

Subblock based temporal motion vector prediction (SbTMVP) is another subblock mode in ECM-14.0. Multiple SbTMVP candidates can be derived from different corresponding block shifted by different source MV and different reference pictures. In JVET-AH meeting, chained motion vector prediction (CMVP) was adopted. CMVP candidates are derived as the accumulation of the recursively traced MVs based on the source MVs.

# Proposal

This contribution proposes the following modifications to the subblock inter modes:

First, it is proposed to add a CPMV search process in the affine DMVR. For each control point, bilateral matching is independently performed for a block that centered by the control-points to derive the refined CPMVs. Then the refined CPMVs are used to derive the best set of CPMVs that minimize the bilateral matching cost of the current block.

Second, it is proposed to quantize the per-pixel or subblock based motion vector to 1/64-pel precision instead of 1/16-pel precision for motion compensation. The 64-phase interpolation filter has the same frequency response as the current 12-tap filter in ECM. However, the subblock motion vectors that are stored for the motion vector prediction are kept in the same 1/16-pel precision, as in ECM.

Third, additional candidates are added by combining the source MV with either TMVP or CMVP. The source MV is the MV used to derive TMVP or CMVP, which is a uni-directional MV. For example, if the source MV comes from the reference picture list 0, the list 0 MV of the TMVP or CMVP is replaced by this MV and the result is used as a new combined candidate in subblock candidate list.

# Simulation results

Test 3.9a: Affine CPMV search for affine DMVR

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.03% | -0.10% | 100.0% | 100.1% | 100.0% | 99.8% |
| Class A2 | -0.03% | -0.03% | -0.11% | 99.6% | 100.3% | 100.0% | 100.1% |
| Class B | -0.04% | -0.04% | -0.06% | 100.1% | 100.5% | 100.0% | 99.8% |
| Class C | -0.03% | -0.01% | -0.05% | 100.3% | 100.1% | 99.9% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.03% | -0.08% | 100.1% | 100.3% | 100.0% | 99.9% |
| Class D | -0.05% | -0.03% | 0.01% | 99.8% | 100.5% | 100.1% | 100.0% |
| Class F | -0.01% | -0.03% | 0.01% | 99.8% | 100.0% | 100.0% | 100.0% |
| Class TGM | -0.02% | 0.02% | -0.03% | 100.1% | 99.4% | 100.1% | 100.3% |

Test 3.9b: High precision MV for affine motion compensation

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.03% | -0.03% | 100.6% | 100.2% | 100.0% | 99.6% |
| Class A2 | 0.01% | 0.01% | -0.08% | 100.7% | 100.7% | 100.3% | 100.2% |
| Class B | -0.04% | 0.00% | -0.10% | 100.9% | 100.6% | 100.0% | 100.1% |
| Class C | -0.06% | -0.10% | 0.18% | 100.6% | 100.6% | 99.6% | 100.4% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.03% | -0.01% | 100.7% | 100.6% | 99.9% | 100.1% |
| Class D | -0.09% | -0.15% | -0.14% | 101.0% | 100.9% | 99.9% | 100.0% |
| Class F | -0.01% | -0.05% | -0.02% | 100.4% | 100.5% | 99.9% | 100.0% |
| Class TGM | -0.05% | 0.01% | -0.03% | 100.2% | 100.2% | 100.7% | 100.2% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | 0.23% | -0.35% | 100.9% | 101.0% | 100.0% | 100.0% |
| Class C | -0.08% | 0.14% | -0.16% | 100.8% | 100.5% | 99.2% | 99.7% |
| Class E | -0.10% | 0.44% | -1.20% | 101.0% | 100.9% | 99.9% | 99.8% |
| Overall | -0.09% | 0.25% | -0.50% | 100.9% | 100.8% | 99.7% | 99.9% |
| Class D | -0.08% | -0.32% | 0.25% | 101.1% | 100.8% | 100.6% | 100.0% |
| Class F | -0.23% | -0.01% | 0.09% | 100.8% | 100.4% | 100.4% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |

Test 3.9c: Additional combined merge candidates

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.07% | 0.01% | 99.5% | 100.1% | 100.2% | 99.8% |
| Class A2 | -0.01% | 0.04% | -0.04% | 99.2% | 100.3% | 100.5% | 99.9% |
| Class B | -0.03% | -0.06% | 0.05% | 100.3% | 100.3% | 100.3% | 100.3% |
| Class C | -0.02% | 0.07% | 0.19% | 100.8% | 100.6% | 100.9% | 100.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.02% | 0.06% | 100.1% | 100.4% | 100.5% | 100.3% |
| Class D | -0.07% | 0.02% | 0.00% | 100.6% | 100.4% | 101.0% | 100.8% |
| Class F | 0.05% | 0.03% | 0.06% | 100.2% | 100.8% | 100.5% | 100.5% |
| Class TGM | 0.03% | 0.05% | 0.00% | 100.6% | 100.4% | 100.5% | 100.5% |

Test 3.9d: Combination of Test 3.9a, Test 3.9b and Test 3.9c

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.23% | -0.17% | 100.6% | 100.8% | 100.3% | 99.6% |
| Class A2 | -0.06% | 0.12% | -0.05% | 100.6% | 100.7% | 100.5% | 100.4% |
| Class B | -0.10% | -0.15% | -0.05% | 100.7% | 101.0% | 100.4% | 100.6% |
| Class C | -0.10% | -0.07% | 0.01% | 100.8% | 101.0% | 100.8% | 100.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.09% | -0.06% | 100.7% | 100.9% | 100.5% | 100.4% |
| Class D | -0.12% | -0.24% | -0.02% | 100.6% | 100.9% | 101.1% | 100.9% |
| Class F | -0.06% | -0.10% | -0.09% | 100.8% | 100.8% | 100.5% | 100.6% |
| Class TGM | -0.05% | -0.01% | -0.05% | 100.7% | 100.9% | 100.4% | 100.5% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | 0.23% | -0.35% | 100.9% | 101.0% | 100.0% | 100.0% |
| Class C | -0.08% | 0.14% | -0.16% | 100.8% | 100.5% | 99.2% | 99.7% |
| Class E | -0.10% | 0.44% | -1.20% | 101.0% | 100.9% | 99.9% | 99.8% |
| Overall | -0.09% | 0.25% | -0.50% | 100.9% | 100.8% | 99.7% | 99.9% |
| Class D | -0.08% | -0.32% | 0.25% | 101.1% | 100.8% | 100.6% | 100.0% |
| Class F | -0.23% | -0.01% | 0.09% | 100.8% | 100.4% | 100.4% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |

# References

[1] ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).