| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0162-v3 |
| --- | --- |

| Title: | AHG11: Enhancing super-resolution with residual in NNVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Li, Yiqing Zhu, Wenxuan He, Qiong Liu (HUST) Zhuoyi Lv (vivo) | Email: | lx59497@gmail.com yiqingzhu@hust.edu.cn wxhe@hust.edu.cn q.liu@hust.edu.cn zhuoyi.lv@vivo.com |
| Source: | Huazhong University of Science and Technology vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, we propose 2 methods to further improve the coding performance of the super-resolution (SR) model. First, we propose two models for ×2 and ×1.5 SR, which can use side information as input based on the contribution [2]. Second, residual information is incorporated as input to the SR model.

The experimental results are reported as:

The mode of singleRatio: AI {-0.66%, 1.51%, 1.90%} and RA {-0.80%, 1.49%, 1.60%}.

The mode of multiRatio: AI {-0.84%, 2.07%, 2.12%} and RA {-0.86%, 1.27%, 1.47%}.

Introduction

In the previous paper [1], the NN tool was used to post-process the RPR image, which effectively improved the coding performance. However, due to the inability to align the resolution, the side information of the coding process (prediction image, boundary strength, block prediction information, etc.) cannot be used. Therefore, this contribution designed 2 SR models for ×2 and ×1.5 respectively, which can use the side information in the coding process and effectively improve the coding performance. In addition, this contribution also introduced residual information as the input of the model in the SR model.

Proposed method

The proposed unified SR architecture are illustrated in Figure 1 and Figure 2. The ×2 super-resolution model introduces residual and RPR images as input based on the contribution [2]. For the ×1.5 super-resolution model, the low-resolution features are downsampled by 2 using a 3×3 conv with stride=2, and the final ×1.5 upsampling is achieved using PixelShuffle (s=3) at the end of the model.

Figure 1 Scaling ratios (s=2) network structure

Figure 2 Scaling ratios (s=1.5) network structure

Training stage information

Table 1 Network Information

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 128 |
|  | Training time: | ~4 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on NNLF-LOP2 and NNIntra |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128 |
|  | Learning rate: | 4e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference stage information

Table 2 Inference information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 1 |
|  | Number of Parameters (Each Model) |  |
|  | Total Parameter Number |  |
|  | Parameter Precision (Bits) | 32(F) |
|  | Memory Parameter (kB) | 23.4k (x2) 94.2k (x1.5) |
|  | Multiply Accumulate (MAC) | 5.6 kMAC/pixel(x2) 5.5 kMAC/pixel(x1.5) |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | 256 |

Experiment results

The proposed method is tested on top of NNVC9.0, according to the common test condition [3]. This proposal adopts the adaptive strategy of JVET-AI0074[1].

Table 3 Single-scaling ratios (s=2) with RPR

|  | AI Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -1.44% | 3.31% | 4.04% | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -2.54% | 5.78% | 7.38% | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.66% | 1.51% | 1.90% | #VALUE! | #VALUE! | #VALUE! |
|  | RA Main10 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -3.16% | 0.08% | 1.34% | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -0.84% | 7.35% | 6.64% | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.80% | 1.49% | 1.60% | #VALUE! | #VALUE! | #VALUE! |

Table 4Multi-scaling ratios (s=2 && s=1.5) with RPR

|  | AI Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -1.79% | 4.14% | 4.04% | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -3.26% | 8.27% | 8.68% | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.84% | 2.07% | 2.12% | #VALUE! | #VALUE! | #VALUE! |
|  | RA Main10 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -3.21% | -0.01% | 1.26% | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | -1.09% | 6.34% | 6.11% | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.86% | 1.27% | 1.47% | #VALUE! | #VALUE! | #VALUE! |

Conclusion

This contribution proposes to design ×2 and ×1.5 super-resolution models respectively, which can use side information as the input, and also proposes to use residual information as the input of the SR model. The proposed architecture is designed to be closely aligned to the JVET-AI0093 contribution. With only a 0.3kMAC/pixel increase, the coding performance is increased by AI{-0.66%, 1.51%, 1.90%} RA{-0.80%, 1.49%, 1.60%} in singleRatio mode and by AI{-0.84%, 2.07%, 2.12%} RA{-0.86%, 1.27%, 1.47%} in multiRatio mode.		The residual will be applied in-loop filtering technology in the future. It is recommended that the proposed residual as input to be adopted into NNVC.

Reference

[1] Zhuoyi Lv, Chuan Zhou “EE1-4.3: RPR new input for super-resolution”, JVET-AI0074

[2] Chaoyi Lin, Yue Li, Junru Li, Kai Zhang, Li Zhang “EE1-4.2: On simplified super resolution model”, JVET-AI0093

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall , Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AG2016.

Patent rights declaration(s)

Huazhong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).