| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0163-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0066 (EE1-1.4: Reduced complexity input feature extraction for LOP & VLOP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Du Liu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-AJ0066 EE1-1.4, which reduces the complexity of input features for VLOP and LOP. The crosscheck includes inference crosscheck and training crosscheck.

It is reported that, for VLOP, the inference crosscheck results match to the results provided by the proponents. For training crosscheck, one model was retrained. The difference between the retrained int16 model and the int16 model from proponents is 0.25% for RA luma and 0.12% for AI luma.

For LOP, the inference crosscheck results match to the results provided by the proponents. The training crosscheck results are similar to the results provided in JVET-AJ0066 with differences less than 0.1% for both RA and AI.

# Introduction

JVET-AJ0066 proposes to remove the DCT 2x2 transform over the sparse inputs BS and IPB, and correspondingly, the input channels of these two inputs are reduced to 3 and 1, respectively.

In terms of complexity, for VLOP, the complexity reduces from 5.1619 to 5.1590 kMAC/pixel. For LOP, the complexity reduces from 16.8974 to 16.8917 kMAC/pixel.

# Results

Inference and training crosscheck is performed for EE1-1.4.1 for VLOP and LOP

## Inference crosscheck of VLOP

Inference crosscheck runs the model provided by the proponents. Inference is tested on top of NNVC-10.0. NN-intra is enabled in both anchor and test.

Encoder time and decoder time are inaccurate since they were not run on the same machine.

Inference results of the int16 model match exactly to the results provided by the proponents.

Table 1. Inference of int16 model vs. NNVC-10 int VLOP.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int VLOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.14% | 1.18% | 0.54% | 96% | 95% |
| Class A2 | -0.06% | 1.59% | 0.92% | 95% | 95% |
| Class B | -0.15% | 1.47% | 1.14% | 98% | 97% |
| Class C | -0.11% | 0.93% | 1.08% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | 1.29% | 0.96% | 97% | 96% |
| Class D | -0.12% | 0.73% | 1.46% | 120% | 128% |
| Class F | -0.19% | 0.42% | -0.02% | 96% | 94% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int VLOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.10% | 1.51% | 1.11% | 111% | 132% |
| Class A2 | -0.06% | 1.52% | 0.89% | 110% | 147% |
| Class B | -0.08% | 1.24% | 1.08% | 100% | 110% |
| Class C | -0.10% | 0.85% | 1.12% | 100% | 124% |
| Class E | -0.08% | 1.91% | 1.05% | 101% | 127% |
| Overall | -0.08% | 1.36% | 1.06% | 104% | 125% |
| Class D | -0.10% | 0.67% | 1.34% | 120% | 159% |
| Class F | -0.17% | 0.67% | 0.37% | 103% | 117% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int VLOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | 1.78% | 0.59% | 117% | 123% |
| Class C | -0.02% | 0.51% | 1.00% | 122% | 127% |
| Class E | -0.04% | -1.89% | -1.09% | 100% | 108% |
| Overall | -0.07% | 0.44% | 0.30% | 114% | 120% |
| Class D | -0.13% | 0.71% | 1.28% | 98% | 97% |
| Class F | -0.15% | -1.17% | -1.15% | 105% | 106% |

## Training crosscheck of VLOP

The training follows VLOP training configuration. Test was run with NNVC-10.0.

Quantization script was provided by the proponents. The int16 results show that the different between the retrained model and the proponents’ model is 0.25% for RA luma and 0.12% for AI luma.

It is observed that the difference for RA is a bit large for Class A1 with 0.38%. Given that the validation losses are similar between the crosschecker and the proponents, the difference in the int16 results is probably due to quantization loss.

Table 2. Retrained int16 model vs. Table 1 int16 model.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Proponents’ int16 model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.38% | 1.89% | 2.39% | 125% | 153% |
| Class A2 | 0.20% | 1.54% | 1.19% | 118% | 120% |
| Class B | 0.24% | 2.13% | 2.02% | 99% | 92% |
| Class C | 0.19% | 1.14% | 1.52% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.25% | 1.70% | 1.79% | 108% | 109% |
| Class D | 0.13% | 1.19% | 1.40% | 82% | 73% |
| Class F | 0.13% | 0.38% | 0.71% | 102% | 106% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Proponents’ int16 model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.13% | 1.23% | 2.21% | 115% | 123% |
| Class A2 | 0.13% | 1.22% | 1.29% | 100% | 89% |
| Class B | 0.11% | 1.33% | 1.85% | 100% | 111% |
| Class C | 0.09% | 0.82% | 1.30% | 97% | 93% |
| Class E | 0.14% | 0.97% | 2.50% | 100% | 108% |
| Overall | 0.12% | 1.12% | 1.80% | 101% | 104% |
| Class D | 0.06% | 0.66% | 1.22% | 80% | 74% |
| Class F | 0.03% | 0.19% | 0.76% | 102% | 111% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Proponents’ int16 model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.30% | 3.98% | 2.44% | 96% | 94% |
| Class C | 0.18% | 3.55% | 2.21% | 91% | 89% |
| Class E | 0.28% | 6.22% | 4.39% | 104% | 87% |
| Overall | 0.25% | 4.40% | 2.85% | 96% | 90% |
| Class D | 0.31% | 3.48% | 2.06% | 102% | 103% |
| Class F | 0.27% | 3.86% | 2.49% | 111% | 114% |

## Inference crosscheck of LOP

Inference crosscheck runs the model provided by the proponents. Inference is tested on top of NNVC-10.0. NN-intra is enabled in both anchor and test.

Encoder time and decoder time are inaccurate since they were not run on the same machine.

Inference results of float and int models match to the results provided by the proponents.

Table 3. Inference of int16 model vs. NNVC-10 int LOP.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.09% | 1.20% | 0.81% | 112% | 115% |
| Class A2 | 0.08% | 0.39% | 0.21% | 112% | 130% |
| Class B | 0.02% | 0.90% | 0.30% | 99% | 89% |
| Class C | -0.05% | 0.52% | 0.49% | 95% | 84% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.76% | 0.44% | 103% | 99% |
| Class D | -0.08% | 0.45% | 0.79% | 96% | 81% |
| Class F | 0.05% | 0.42% | 0.64% | 102% | 93% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.01% | 1.02% | 0.93% | 112% | 138% |
| Class A2 | 0.09% | 0.30% | 0.38% | 101% | 111% |
| Class B | -0.01% | 0.41% | 0.41% | 101% | 118% |
| Class C | 0.09% | 0.69% | 0.69% | 96% | 120% |
| Class E | 0.05% | 0.60% | 0.55% | 96% | 117% |
| Overall | 0.04% | 0.59% | 0.58% | 101% | 120% |
| Class D | 0.04% | 0.95% | 1.09% | 99% | 109% |
| Class F | 0.04% | 0.93% | 1.15% | 95% | 106% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.49% | 1.48% | 113% | 115% |
| Class C | -0.01% | 0.22% | 1.37% | 110% | 111% |
| Class E | 0.07% | 1.76% | 0.50% | 96% | 77% |
| Overall | 0.02% | 0.31% | 1.20% | 107% | 103% |
| Class D | 0.04% | -0.04% | 6.56% | 97% | 93% |
| Class F | 0.14% | 0.71% | -0.68% | 112% | 112% |

Table 4. Inference of float model vs. NNVC-10 float LOP.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.09% | -0.02% | -0.06% | 68% | 27% |
| Class A2 | 0.08% | -0.81% | 0.00% | 88% | 40% |
| Class B | 0.10% | 0.54% | 0.14% | 89% | 55% |
| Class C | 0.13% | 0.78% | 0.08% | 93% | 66% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.22% | 0.06% | 85% | 47% |
| Class D | 0.05% | -0.25% | 0.23% | 99% | 55% |
| Class F | 0.15% | -0.16% | -0.09% | 85% | 45% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.01% | -0.37% | -0.01% | 93% | 97% |
| Class A2 | 0.02% | -0.87% | -0.28% | 94% | 135% |
| Class B | -0.02% | -0.05% | -0.38% | 88% | 79% |
| Class C | -0.01% | 0.16% | 0.08% | 110% | 243% |
| Class E | -0.01% | 0.24% | -0.27% | 101% | 196% |
| Overall | -0.01% | -0.15% | -0.18% | 97% | 133% |
| Class D | -0.04% | -0.56% | -0.41% | 103% | 181% |
| Class F | -0.02% | -0.51% | -0.36% | 98% | 141% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.37% | -0.62% | 64% | 30% |
| Class C | 0.08% | 1.21% | -0.69% | 66% | 29% |
| Class E | -0.02% | 0.11% | -2.76% | 98% | 82% |
| Overall | 0.04% | 0.27% | -1.18% | 72% | 38% |
| Class D | 0.14% | 0.55% | 0.76% | 103% | 98% |
| Class F | 0.13% | 1.07% | -0.25% | 52% | 21% |

## Training crosscheck of LOP

Table 5. Inference of retrained model vs. NNVC-10 float LOP.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.06% | -0.97% | -0.79% | 116% | 97% |
| Class A2 | 0.04% | -1.42% | -0.74% | 152% | 150% |
| Class B | 0.13% | -0.15% | -0.62% | 137% | 133% |
| Class C | 0.09% | 0.45% | -0.75% | 128% | 115% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.41% | -0.71% | 133% | 123% |
| Class D | 0.18% | -0.16% | -0.54% | 125% | 93% |
| Class F | 0.06% | -0.64% | 0.05% | 129% | 101% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 float |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.02% | -0.54% | -0.57% | 146% | 306% |
| Class A2 | 0.04% | -1.07% | -0.74% | 138% | 286% |
| Class B | 0.03% | -0.63% | -0.93% | 129% | 210% |
| Class C | 0.08% | -0.48% | -0.84% | 145% | 362% |
| Class E | 0.00% | -0.87% | -1.13% | 145% | 330% |
| Overall | 0.03% | -0.70% | -0.85% | 139% | 286% |
| Class D | 0.08% | -0.98% | -0.93% | 144% | 325% |
| Class F | -0.02% | -0.33% | 0.28% | 130% | 208% |

Quantization of the float model used the LOP3 quantization script.

Table 6. Inference of retrained model vs. NNVC-10 int LOP.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.10% | -0.88% | -1.14% | 142% | 132% |
| Class A2 | 0.01% | -1.26% | -0.92% | 127% | 132% |
| Class B | 0.07% | -0.38% | -1.01% | 144% | 138% |
| Class C | 0.15% | 0.40% | -1.13% | 139% | 132% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.45% | -1.05% | 139% | 134% |
| Class D | 0.32% | -0.04% | -0.67% | 141% | 131% |
| Class F | 0.16% | -0.49% | 0.14% | 152% | 139% |

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10 int |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.36% | -0.86% | -0.91% | 130% | 165% |
| Class A2 | 0.07% | -1.14% | -0.78% | 138% | 170% |
| Class B | 0.03% | -0.70% | -0.99% | 124% | 155% |
| Class C | 0.13% | -0.57% | -0.98% | 117% | 133% |
| Class E | 0.04% | -0.88% | -1.03% | 138% | 173% |
| Overall | -0.01% | -0.80% | -0.95% | 128% | 156% |
| Class D | 0.12% | -0.80% | -0.83% | 148% | 185% |
| Class F | 0.11% | -0.38% | 0.26% | 102% | 115% |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.