| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0164-v2 |
| --- | --- |

| Title: | AhG12: On advanced SBT with position and direction inference |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Maxime Rousselot Patrice Onno | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents an Advanced SBT (ASBT) mode. In this proposed subblock residual coding mode, the position and the direction of the transform subblocks are determined based on the corresponding Inter predictor block. This ASBT mode is signalled at Transform Unit level and it can be applied on SBT transform units and other transform units as well. This ASBT mode is only applied for Inter coding blocks.

Compared to ECM-14.0, the average BDR gains and runtimes reported in this contribution are as follows:

-0.09%	-0.14%	-0.12%   %	% for the RA configuration,

-0.14%	0.04%	-0.04%   %	% for the Low Delay B configuration.

# Proposed method

The principle of the Advanced SBT is to determine the position and the direction of a subblock partition thanks to the Inter prediction block. The possible subblock partitions are the same as the ones of SBT in the VVC specifications. However, the proposed Advanced SBT infers the position and the direction of the subblock partitioning to save some signalling.

The following Figure 1 depicts this principle at encoder side. For each subblock size and direction, the encoder evaluates the best position of the transform part (grey area). Then, based on the best position, the direction for each subdivision is determined. Eventually the encoder determines the best subdivision.

The position selected is the position of the subblock which maximizes the sum of gradients obtained on the corresponding Inter predictor block. And regarding the direction (vertical or horizontal), the selected direction is the one that maximizes the sum of gradients of each best position.

Figure 1 Advanced SBT, General principle at encoder side.

Compared to the SBT mode of VVC, only the subblock subdivision is signalled if the enabled flag of the proposed method is true. The proposed Advanced SBT is signalled at TU level and it is also allowed for SBT TU with residual. The enabled flag and the subdivision flags are CABAC-coded and the context probability increment value depends on the use of SBT.

The Advanced SBT is not allowed for Intra slices and for some Inter modes as CIIP and GPM. It is also disabled for LFNST and MTS and for Screen Content sequences thanks to an SPS flag. The Advanced SBT is applied only for the Luma component.

It is important to note that the implementation of this SBT mode has no parsing issue even if the position and the direction is based on the Inter predictor block.

To reduce the encoding time, some early encoder terminations are also used to limit the number of evaluations of this proposed ASBT mode.

# Results

The proposed modifications are implemented on top of the ECM-14.0 [1] and tested according to the common test conditions (CTC) for ECM-14.0. The following table gives the results of the proposed method.

Table 12 Coding performances of ASBT.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.31% | -0.12% |  |  | 100.1% |  |
| Class A2 | -0.13% | 0.05% | -0.06% |  |  | 100.4% |  |
| Class B | -0.10% | -0.06% | -0.05% |  |  | 100.2% |  |
| Class C | -0.07% | 0.09% | 0.15% |  |  | 100.8% |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.09% | -0.05% | -0.01% |  |  | 100.3% |  |
| Class D | -0.08% | -0.14% | 0.11% |  |  | 100.4% |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.17% | -0.11% | -0.11% |  |  | 100.2% |  |
| Class C | -0.11% | -0.11% | 0.00% |  |  |  |  |
| Class E | -0.13% | -0.14% | -1.36% |  |  |  |  |
| Overall | -0.14% | -0.12% | -0.39% |  |  |  |  |
| Class D | -0.06% | 0.76% | 0.60% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

In this contribution, an Advanced SBT mode is proposed. Compared to the regular SBT mode in ECM, this subblock transform residual method infers the position and the direction of the transform part.

An average BD-rate gain of  xx% and yy%  is obtained respectively for RA and LDB configurations with a systematic gain per class.

We suggest investigating the proposed method in the next round of EE2 in the ECM-15.0.

# References

- ECM-14.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-14.0

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).