| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0165-v2 |
| --- | --- |

| Title: | [EE1-1.2: LOP with residual groups and Attention] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Samuel Eadie Thomas Ryder Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com seadie@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A unified filter architecture for the Low-performance Operation Point (LOP) was defined in JVET-AF0043. Updated training strategy and configuration were presented in JVET-AH0042. LOP3 with DCT transform input were adopted in JVET-AH0080, and complete inference results for LOP3 training are reported in JVET-AJ0014. Furthermore, in JVET-AI0221 a new LOP/VLOP architecture, featuring residual groups and spatial attention was proposed, which improves the coding gain by a big margin.

This contribution reports results of the EE1-1.2, with residual groups and the Attention module of JVET-AI0221 tested on top of the LOP3 architecture. The simulation was conducted by using the NNVC-10 reference software, training was performed by using LOP3 training strategy, and SADL int16 model was used for the inference.

Complexity of the proposed model is 16.89 (block-wise) kMAC/pixel

The proposed filter (Int16 model) with NNIntra enabled provides: {-0.4%, -0.1%, 0.4%} BD-rate change for AI test configuration, and {-0.4%, -0.0%, 0.3%} for RA test configuration, versus the LOP3 int16 (NNIntra enabled).

Compared to the VTM, the proposed filter with NNIntra enabled provides: {-8.8%, -14.4%, -13.7%} for the AI configuration, and {-7.8%, -13.6%, -11.3%} for the RA configuration.

It is suggested to adopt the proposed method for LOP.

# Introduction

A unified filter architecture for the Low-performance Operation Point (LOP) was defined in JVET-AF0043 [1]. Updated training strategy and configuration were presented in JVET-AH0042. LOP3 with DCT transform input were adopted in JVET AH0080[2], see Fig. 1 for the architecture, and complete inference results for LOP3 training are reported in JVET-AI0014.

Figure 1. Architecture of LOP3.

Furthermore, JVET-AI0221 [3] has introduced a residual group and spatial attention, which improve the coding gain significantly.

This contribution reports results of the EE1-1.2, with residual groups and the Attention module of JVET-AI0221 tested on top of the LOP3 architecture. The simulation was conducted by using the NNVC-10 reference software, training was performed by using LOP3 training strategy, and SADL int16 model was used for the inference.

# Proposed method

In this contribution, the residual groups and Attention modules are placed on top of LOP3 as shown in Figure 2. A residual backbone group consists of consecutive backbone blocks followed by a PReLU and an Attention module. The attention module is shown in Fig. 3, where the attention map is computed by using a sequence of operations, e.g., max pooling, depth-wise (DW) convolution blocks and bilinear interpolations, followed by a hard sigmoid activation. The attention is then performed with an element-wise multiplication of the input features of the Attention module to the attention map, where distant correlation of the features is captured in the down-sampled domain. The depth-wise convolution block is further shown in Fig. 4.

Figure 2 LOP3 with residual groups and Attention modules.

Figure 3 The Attention module.

Figure 4 The depth-wise convolution block (DW Conv block) for the Attention module

The number of attention channel CAttn in the Attention module is 32 and 16 for the Luma and the Chroma branch, respectively, and the number of backbone blocks in each backbone group is indicated by the parameter GB. In the current configuration, there are 5 backbone groups, and each group consists of 2 backbone blocks for the Luma branch. This can be expressed as [2, 2, 2, 2, 2] in sequence. The Chroma branch contains 2 backbone groups, in which the first group consists of 3 backbone blocks, and the second group comprises 2 backbone blocks, which is expressed as [3, 2] for the group setup.

In addition, the number of expanded channels C1 in the backbone block shown in Fig. 1 is configured to be 128 for the Luma and 80 for the Chroma to leave room for the Attention modules. With the above configuration, the complexity of the model is restricted to 16.89 kMAC/pixel.

The information for the inference and the training stage is presented in Table 1 and 2 for the proposed model, respectively.

Table1. Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 224872 |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 16.89 (13.3 frame-wise) |
|  | Total Conv. Layers |  | 145 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 0.49/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~240h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Simulation results

The method was integrated in the NNVC-10 and tested following the EE1 CTC and evaluation procedure [4]. The proposed method has been trained following the training procedure specified in JVET-AF0043. The training is being conducted with a target of 60 epochs as specified by the guidance and is identical to that of JVET-AH0080.

The proposed method was tested with NNVC-10 reference software with the SADL int16 model.

Summary of the inference results are reported below, complete results in CTC template are provided along with this document.

Comparative results to the NNVC-10 LOP3 anchor (int16) are shown in Table 3. NN Intra was enabled for both the test and the anchor. Comparison against VTM is shown in Table 4.

As observed in Tables 3, the proposed filter outperforms the LOP3 anchor with NNIntra ON by {-0.4%, -0.1%, 0.4%} for the AI configuration and {-0.44%, -0.03%, 0.34%} for the RA configuration, respectively.

Compared to VTM, shown in Table 4, the proposed method provides {-8.8%, -14.4%, -13.7%} for the AI configuration, and {-7.8%, -13.6%, -11.3%} for the RA configuration, respectively.

Table 3 Proposed method (int16) vs LOP3 anchor, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.24% | -0.76% | 0.28% | -0.15% | -0.63% | 0.36% |
| Class A2 | -0.43% | -0.96% | 0.05% | -0.40% | -0.97% | 0.25% |
| Class B | -0.31% | -0.31% | 0.01% | -0.48% | 0.08% | 0.04% |
| Class C | -0.54% | 0.79% | 1.01% | -0.63% | 1.00% | 0.78% |
| Class E | -0.40% | 0.37% | 0.54% |  |  |  |
| Overall | -0.38% | -0.14% | 0.37% | -0.44% | -0.03% | 0.34% |
| Class D | -0.53% | 0.78% | 0.61% | -0.92% | 0.52% | 0.10% |
| Class F | -0.20% | 0.93% | 0.81% | -0.09% | 0.91% | 1.32% |

Table 4 Proposed method (int16) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -9.44% | -14.81% | -14.63% | -8.89% | -12.55% | -12.36% |
| Class A2 | -8.23% | -14.70% | -11.01% | -8.03% | -13.68% | -8.31% |
| Class B | -7.98% | -14.45% | -14.42% | -7.42% | -14.73% | -12.51% |
| Class C | -7.96% | -12.82% | -12.27% | -7.18% | -12.94% | -11.40% |
| Class E | -11.22% | -15.72% | -15.89% |  |  |  |
| Overall | -8.80% | -14.40% | -13.66% | -7.78% | -13.61% | -11.34% |
| Class D | -7.76% | -11.02% | -11.48% | -7.55% | -12.23% | -10.95% |
| Class F | -5.36% | -8.72% | -8.00% | -4.00% | -7.11% | -5.19% |

# SIMD acceleration on depth-wise convolution:

# It can be noticed that this model has introduced some additional operators that are not present in LOP3, and some of these do not have a corresponding SIMD implementation in the SADL of NNVC-10. The DW convolution, shown in Fig. 4, is the one among them.

To accelerate the inference of the model with SADL, a SIMD version of the DW convolution is implemented, where all operations of the convolution, including shift and saturate, are carried out with SIMD. This results in a 10 percent decoding runtime reduction, and the CPU encoding and decoding runtime compared to that of LOP3 is shown in Table 5 and Table 6 for AI and RA, respectively.

Table 5 Proposed method (int16) with SIMD DW conv. vs LOP3 anchor, NNIntra ON, for AI

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.24% | -0.75% | 0.29% | 100% | 109% |
| Class A2 | -0.43% | -0.96% | 0.05% | 100% | 111% |
| Class B | -0.31% | -0.31% | 0.01% | 100% | 108% |
| Class C | -0.54% | 0.79% | 1.01% | 100% | 103% |
| Class E | -0.40% | 0.37% | 0.54% | 100% | 107% |
| Overall | -0.38% | -0.13% | 0.37% | 100% | 107% |
| Class D | -0.53% | 0.78% | 0.60% | 99% | 110% |
| Class F | -0.20% | 0.93% | 0.81% | 99% | 106% |

Table 6 Proposed method (int16) with SIMD DW conv. vs LOP3 anchor, NNIntra ON, for RA

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.17% | -0.66% | 0.36% | 100% | 107% |
| Class A2 | -0.39% | -0.93% | 0.20% | 99% | 104% |
| Class B | -0.47% | 0.12% | 0.11% | 100% | 106% |
| Class C | -0.66% | 1.06% | 0.72% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | 0.01% | 0.34% | 100% | 106% |
| Class D | -0.92% | 0.54% | 0.15% | 98% | 108% |
| Class F | -0.12% | 0.87% | 1.32% | 99% | 107% |

# As the CPU runtime reduction here is obtained with the acceleration of only one operator of the model among those additional operators, further runtime reduction may be achieved with a proper SIMD implementation of other operators.

# Ablation studies

- The impact of the Attention block:

# This test was conducted with a reference NN architecture of JVET-AI221 with two models, which are configured to be around 16.9kMAC/pixel as follows:

- Model with Attention: the reference architecture expanded to the specified complexity.

- Model without Attention: the reference architecture with all Attention modules removed, and the complexity budge is given to more regular backbone blocks.

# Figure 5 YUV validation loss from the training for the model with Attention and without Attention.

# It can be observed from Fig. 5 that the proposed Attention module improves the performance of the model

- The impact of the residual group

# The test was conducted with a reference NN architecture of this contribution with two models, which are configured to be around 16.9kMAC/pixel as follows:

- Model with residual group: reference architecture

- Model without residual group: residual connections for the backbone groups are removed from the reference architecture.

# Figure 6 YUV validation loss from the training for the model with residual group and without.

# It can be seemed from Fig. 6 that adding residual groups for the Attention module enhances the performance of the model.

# Conclusion

In this contribution, the results of the proposed Attention mechanism with residual groups on LOP3 with the reference training strategy are reported.

Compared to NNVC-10 LOP3 anchor (NNIntra ON), the proposed filter (int16) provides: AI: {-0.4%, -0.1%, 0.4%} and RA: {-0.44%, -0.03%, 0.34%} BD-rate change. Compared to VTM, it provides: AI: {-8.8%, -14.4%, -13.7%} and RA: {-7.8%, -13.6%, -11.3%}.

The proposed method has provided a significant coding gain compared to the existing anchor. It is suggested to adopt the proposed method.

# References

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), R. Chang, L. Wang, X. Xu, S. Liu (Tencent), AHG11: Status of the joint EE1-0 (LOP.2) unified filter training, JVET-AF0043, October 2023.

- D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson), D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), T. Shao, P. Yin, S. McCarthy (Dolby), Jay N. Shingala, Ajay Shyam, Ajat Suneja, Siddarth P. Badya (Ittiam), EE1-1.2: Joint LOP model with inputs transformed, JVET-AH0080, 2024.

- T. Ryder, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz, EE1-1.3: On low complexity operational point for in-loop filtering, JVET-AI0221, 2024.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).