| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0166-v1 |
| --- | --- |

| Title: | [EE1-2.2: Block-size invariant implementation of HOP5] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041. Based on the architecture of the HOP network, the Attention model with linear operators was derived from Transformer model in JVET-AG0162 and further refined in JVET-AI0172, which provides significant coding gain for both AI and RA configuration compared to its predecessors. However, due to the removal of the normalization layers, the inference is best performed with a fixed block size of a base granularity, since this is the block size utilized during training.

This contribution reports results of the EE1-2.2, for which an alternative method of split block with shared signalling, based on the finding from JVET-AI0175, is proposed. To perform inference for larger block, it is proposed to split the block into sub-blocks of the base granularity, and apply the same NN mode, scale/offset which is signalled for the larger block.

The method was integrated into NNVC-10, and inference testing with HOP5 int16 model under NNVC CTC was conducted. The block-size assignment of the NNVC software is unaltered.

The proposed method (int16 model) with NNIntra ON provides: {0.0 %, 0.0%, 0.0%} for AI test configuration, and {-0.03%, -0.04%, -0.04%} for RA test configuration, versus the HOP anchor (NNIntra ON), whereas the decoding runtime for RA is at 96% of the anchor.

Additionally, over -0.05% Luma gain is observed for class A for the RA configuration with a decoding runtime reduction of above 6%.

It is proposed to adopt the proposed inference method into NNVC SW under the flag of NnlfOption=5 and enable it for HOP anchor.

# Introduction

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041. The HOP4 software integration, combining with Attention module (JVET-AH0205 [1]) and wide activation of JVET-AH0189 [2] was jointly trained by the proponents and presented during AhG11/14 telco. The parameters and complexity of the HOP4 model is further adjusted with improved coding gain, and the model is adopted as HOP5 in JVET-AI0172. Complete inference results for HOP5 training are reported in JVET-AJ0014.

Based on the architecture of the HOP network, the Attention model with linear operators was derived from Transformer model in JVET-AG0162[3] and further refined in JVET-AI0172[4], which provides significant coding gain for both AI and RA configurations compared to its predecessors. However, since the removal of the normalization layers, the inference is best performed with a fixed block size of 128x128 as that of the block size during training for this model.

An Attention map scaling algorithm was proposed in JVET-AI0175[5] to improve the performance of the model, which has produced a coding gain for the RA configuration. Although this method is very straight forward to implement, in the SADL library, the method requires two extra operators, i.e. |Gather and Shift (or Division implemented with Shift operations), and may additionally need some implementation tricks.

In this document, an alternative method, based on the finding from JVET-AI0175, is proposed to utilize a split block with shared signaling mode to adapt to the existing changing block-size assignment of the NNVC software and to reduce potential signalling cost. In this test, only inference is conducted by using the NNVC-10 software with the HOP5 model, and SADL-int16 quantized model was used for the inference.

Proposed method

In the NNVC software, the in-loop filtering is performed with different modes and will generate extra side information, which is transmitted to the decoder side so that a specific filtering mode can be executed for an optimal filtering operation. The side information may include scaling factor, QP offset, slice/block level on/off signalling, etc. The scaling factor is derived on picture basis, whereas the QP offset, slice/block level control on/off may be on slice or block basis. This mode selection process is determined with a Rate-Distortion Optimization (RDO) approach at the encoder side.

For example, the current NNVC implementation has employed two block sizes for the inference of the in-loop filtering, which are 256x256 and 128x128. They are referred to as large-granularity and base-granularity block size, respectively. Additionally, small granularity of size 64x64 is implemented, but it was not selected by the current encoder configuration.  Furthermore, a padding/extension of 0 to 8 pixels can be applied on all boundaries of the block, and the padding with the actual input block is used as the final input to the filtering process.

As the model is trained with a base-granularity block size, in order to adapt to the changing block-size assignment during the inference stage, a split block with shared signalling mode is proposed in this contribution. The shared signalling mode is depicted in Fig. 1. When the input block for inference is of large granularity, the block will be split into four sub-blocks of base granularity, and the inference is conducted on each sub-block of base granularity separately. The signalling of the sub-blocks, e.g., QP offset, block on/off, will be shared with the initial input block. If the input block is of base granularity, the inference will be performed directly according to the NNVC scheme without any changes.

Figure 1 a large-granularity block is split into four base-granularity blocks for inference, and the signalling information is shared between sub-blocks.

Because the HOP network is very effective, a slice level RDO is not needed to determine if this mode is used or not. Therefore, the shared signalling mode is always enabled as long as the input is of large granularity in this test.

The information for the inference is identical to that of HOP5, shown in Table 1.

Table1. Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.4M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 466 (375 frame-wise) |
|  | Total Conv. Layers |  | 162 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.8/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

# Simulation results

The method was integrated in the NNVC-10 and tested following the EE1 CTC and evaluation procedure [6]. The proposed method has been trained following the EE1-0 training procedure (Stage 3 HOP training only) specified in JVET-AE0191 and updated for EE1 testing as specified in JVET-AG0041. The training is being conducted with a target of 20 epochs as specified by the guidance.

The proposed method was tested with NNVC-10 reference software by using the HOP5 model with the NNVC default block-size assignment in the unified filter setting.

Summary of the inference results are reported below, complete results in CTC template are provided along with this document.

Comparative results to the NNVC-10 HOP5 filter are shown in Table 2. NN Intra was enabled for both the test and the anchor.

As observed in Tables 2, the proposed filter provides coding gain versus the HOP anchor with NNIntra ON by {0.0%, 0.0%, 0.0%} for the AI configuration and {-0.03%, -0.04%, -0.04%} for the RA configuration, respectively. It is further shown that the overall decoding runtime is at the 96% of the anchor. Higher bit rate reduction of over -0.05% can be obtained for the class A of RA configuration, where the runtime reduction is above 6%.

Table 2 Proposed method (int16 model) vs NNVC10 HOP 5, NNIntra ON

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 100% | 101% |
| Class A2 | 0.00% | 0.03% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.08% | 0.07% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.03% | 0.02% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | -0.01% | 0.01% | 0.05% | 100% | 105% |

Table 3 Proposed method (int16 model) vs NNVC10 HOP5, NNIntra ON

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.05% | -0.24% | -0.01% | 98% | 91% |
| Class A2 | -0.07% | 0.13% | -0.09% | 100% | 94% |
| Class B | -0.03% | -0.06% | -0.09% | 101% | 98% |
| Class C | 0.00% | 0.00% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.04% | -0.04% | 100% | 96% |
| Class D | -0.01% | 0.07% | -0.05% | 97% | 97% |
| Class F | -0.01% | 0.06% | 0.03% | 101% | 100% |

# Conclusion

In this contribution, an alternative split block with shared signalling mode is proposed to provide a block-size invariant implementation that can adapt to the block-size assignment scheme of the NNVC software.

Compared to NNVC10 HOP anchor (NNIntra ON), the filter (int16 model) with the proposed configuration provides: AI: {0.0%, 0.0%, 0.0%} and RA: {-0.03%, -0.04%, -0.04%} BD-rate change.

Additionally, over -0.05% Luma gain is observed for class A for the RA configuration with a decoding runtime reduction of above 6%.

It is proposed to adopt the proposed inference method into NNVC SW under the flag of NnlfOption=5 and enable it for HOP anchor.

# References

- EE1-2.3 Integer implementation of HOP In-loop filter with Transformer blocks and Attention blocks, JVET-AH0205, Y. Li, D. Rusanovskyy, M. Karczewicz.

- EE1-2.2: Wide activation HOP model, JVET-0189, Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang

- EE1-1.5 Report on implementation of HOP In-loop filter with Transformer blocks, JVET-AG0162, Y. Li, D. Rusanovskyy, M. Karczewicz.

- EE1 (AhG11) On the complexity adjustment of HOP4, JVET-AI0172, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm), F. Galpin (Interdigital), Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang (Bytedance).

- EE1-Related: Block-size invariant implementation of HOP4, JVET-AI0175, Y. Li, D. Rusanovskyy, M. Karczewicz.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).