| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1-8 November 2024 | Document: JVET-AJ0168-v2 |
| --- | --- |

| Title: | AHG8: Chroma QP Offset for VCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating (Sony), Masaru Ikeda (Sony) | Email: | steve.keating@sony.com |
| Source: | Sony |  |  |

_____________________________

# Abstract

This contribution tests the use of a chroma QP offset within the VVC codec for machine analysis.  The following results are reported for an offset of -6:

- SFU−HW: -5.9% | −3.5% [RA | LD],  TVD: −6.1% | −12.9% [RA | LD]

It is proposed to include a section on chroma QP offset in the next version of JVET-AI2030.

# Introduction

At the July 2024 JVET meeting results from coding in 4:4:4 for machine use were presented, it was requested to test the effect of chroma QP offset with coding in 4:2:0 for comparison.

JVET has been working on a technical report regarding optimizations for encoders and receiving systems for machine analysis of coded video content. The latest version of this report was made available after the July 2024 JVET meeting in JVET-AI2030 [1].

It is proposed to mention the use of chroma QP offset in the next version of the report.

# Importance of chroma

Machine tasks such as object detection do not necessarily have the same chroma sensitivity as human vision so 4:2:0 sampling may not be optimal for such applications.  Previously [3], resampling from 4:2:0 to 4:4:4 prior to encoding was tested. Here the results from encoding in 4:2:0 using different chroma QP offsets are presented: chroma QP offsets of -3, -6 and -9 were tested with VTM23.3.

# Results

The results for the different chroma QP offsets are show below:

| Dataset |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ChQPoff-3 | ChQPoff-6 | ChQPoff-9 | ChQPoff-3 | ChQPoff-6 | ChQPoff-9 |
| SFU-HW | Class A | -3.1% | -3.6% | -11.1% | -7.6% | -18.6% | -23.0% |
|  | Class B | -1.5% | -8.2% | -4.5% | 0.2% | -0.8% | -0.9% |
|  | Class C | -1.6% | -3.1% | -0.2% | -2.0% | -1.3% | 2.8% |
|  | Class D | -5.9% | -7.0% | -4.1% | -2.1% | -4.6% | -2.0% |
|  | Average | -3.0% | -5.9% | -3.6% | -1.8% | -3.5% | -1.8% |
| TVD | TVD-01-1 | -22.1% | -28.9% | -37.8% | 4.1% | -32.6% | -5.9% |
|  | TVD-01-2 | -2.0% | -18.9% | -21.2% | 19.7% | -43.4% | -28.8% |
|  | TVD-01-3 | -9.2% | -10.6% | -26.6% | 11.8% | 2.7% | -27.4% |
|  | TVD-02-1 | 24.4% | 20.5% | -4.5% | 43.7% | -11.7% | 37.1% |
|  | TVD-03-1 | -7.0% | -6.1% | 3.3% | 4.9% | -9.5% | -1.5% |
|  | TVD-03-2 | -14.3% | -3.5% | 13.0% | 12.0% | 7.1% | 7.1% |
|  | TVD-03-3 | 2.1% | 4.5% | 5.9% | 9.3% | -3.1% | -1.0% |
|  | Average | -4.0% | -6.1% | -9.7% | 15.1% | -12.9% | -2.9% |

If an encoder were able to choose the best offset for each sequence the gain could be maximised as shown below:

| Dataset |  | Random access |  | Low delay |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ChQPoff |  | ChQPoff |  |
| SFU-HW | Class A | -9 | -11.1% | -9 | -23.0% |
|  | Class B | -6 | -8.2% | -9 | -0.9% |
|  | Class C | -6 | -3.1% | -3 | -2.0% |
|  | Class D | -6 | -7.0% | -6 | -4.6% |
|  | Average |  | -7.4% |  | -7.6% |
| TVD | TVD-01-1 | -9 | -37.8% | -6 | -32.6% |
|  | TVD-01-2 | -9 | -21.2% | -6 | -43.4% |
|  | TVD-01-3 | -9 | -26.6% | -9 | -27.4% |
|  | TVD-02-1 | -9 | -4.5% | -6 | -11.7% |
|  | TVD-03-1 | -3 | -7.0% | -6 | -9.5% |
|  | TVD-03-2 | -3 | -14.3% | 0 | 0.0% |
|  | TVD-03-3 | 0 | 0.0% | -6 | -3.1% |
|  | Average |  | -15.9% |  | -18.3% |

# Conclusions

The effects of using chroma QP offset when encoding for machine use have been studied. It is proposed to amend the note about chroma quality in the technical report on optimizations for encoders and receiving systems for machine analysis of coded video content to reflect these findings.

# Editorial Note for TR

The following editorial note is suggested for section 8 of the TR:

The quality of chroma vs. luma may need to be addressed when encoding in 4:2:0 as many machine tasks operate on 4:4:4 data and performance may be negatively impacted by reduced chroma quality.  The use of a negative chroma QP offset may improve performance of 4:2:0 encoding.

# Patent rights declaration

Sony may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] 	C. Hollmann, J. Chen, S. Liu. "Optimization of encoders and receiving systems for machine analysis of coded video content (draft 6)", JVET-AI2030, July 2024, Sapporo, JP

[2] 	S. Liu, C. Hollmann. "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content", JVET-AI2031, July 2024, Sapporo, JP

[3] 	S. Keating. M. Ikeda. “AHG8: 444 processing for VCM”, JVET-AI0144, July 2024, Sapporo, JP