| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0169-v1 |
| --- | --- |

| Title: | Non-EE2: Improvement on non-MPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guiqi Wang Chuan Zhou Zhuoyi Lv (vivo) | Email: | WangGuiqi@vivo.com chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, the non-MPM mode is improved by using the proposed non-MPM list construction method.

On top of ECM-14.0, simulation results of the proposed method are:

AI:  xx%, xx%, xx%, EncT: xx%, DecT: xx%.

Introduction

In current ECM-14.0, the primary MPM (PMPM) list consists of 6 entries and the secondary MPM (SMPM) list includes 16 entries. A general MPM list with 22 entries is constructed first, and then the first 6 entries in this general MPM list are included into the PMPM list, and the rest of entries form the SMPM list. The first entry in the general MPM list is the Planar mode. The remaining entries are composed of the intra modes of the left (L), above (A), below-left (BL), above-right (AR), and above-left (AL) neighbouring blocks, and DIMD modes which are sorted in ascending order of SAD cost. Up to 5 modes with the smallest SAD cost are added. The SAD cost is computed between the prediction and the reconstruction samples of the template. The sorted directional modes with added offset are added into the general MPM list, and then the default modes, until the general MPM list with 22 entries is constructed [1].

After the PMPM and SMPM lists are constructed, the remaining 45 intra modes which do not belong to either the PMPM list or SMPM list compose the non-MPM list, and are arranged in ascending order of intra mode index. The selected mode index in the non-MPM list is signalled with truncated binarization and bypass coding.

Proposed method

Firstly, the distribution of selected mode index in the non-MPM list in current ECM-14.0 was analyzed. It can be seen from the distribution histograms in Fig.1 and Fig. 2 that, the percentage of the mode index does not show a gradual downward trend, but first drop to the lowest point, then rise.

Figure 1. Distribution of Campfire

Figure 2. Distribution of other sequences

In this proposal, the non-MPM list are constructed as follows:

- The first angular mode in the SMPM list is chosen as the starting intra mode.

- From the starting intra mode, fill the non-MPM list in turns with one forward mode and then one backward with the same step, where the step increases by one in each turn. Skip the mode which has already been included in the PMPM list or the SMPM list.

- If a mode satisfies the PDP mode condition, swap it with the previous filled non-PDP mode.

By using the proposed method, a regular downward trend distribution of the selected intra mode in the non-MPM list is illustrated in Fig. 3.

Figure 3. Distribution using the proposed method

Experimental results

The proposed method has been implemented on top of ECM-14.0 [2]. Testing results of the proposed method under AI configuration are summarized in Table 1. The simulations were performed following the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1. Simulation results

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.01% | 0.04% |  |  |
| Class C | -0.01% | -0.06% | 0.00% |  |  |
| Class E | -0.03% | 0.02% | 0.07% |  |  |
| Overall |  |  |  |  |  |

References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 14 (ECM 14)”, JVET-AI2025.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0?ref_type=tags

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).