| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0170-v1 |
| --- | --- |

| Title: | AHG9: On Lens Optical Correction SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson-Allendes Sachin Deshpande | Tel: Email: | +46-721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

This input contribution relates to the Lens Optical Correction (LOC) SEI message which is included in the technologies under consideration document for future versions of VSEI. The proposals in this document are intended to broaden the applicability of the LOC SEI message and are asserted to provide clarifications of the usage of the LOC SEI message. Specifically, the following items are proposed:

- Add support for signalling vignetting parameters

- Add poly4 as a model for radial distortion

- Move the persistence flag to be the first syntax element after the cancel flag

- Add support for signalling a text string identifying the lens model

- Add clarification on the order in which corrections are expected to be performed

- Add description of when in the processing chain (to what color format) the corrections are applied

# Introduction

The idea of a Lens Optical Correction (LOC) SEI message was first introduced in JVET-AH0204 [1] and was later included in the document for technologies under consideration for future extensions of VSEI, JVET-AI2032 [2], in response to JVET-AI0158 [3]. The LOC SEI message is intended to allow for correction of lens-related distortions such as radial distortion and transverse chromatic aberration. This input contribution builds on the text of LOC SEI message in JVET-AI2032 and proposes modifications intended to generally broaden the applicability of the SEI message and to improve clarity.

# Proposals

This contribution includes six proposed items, each presented in its own subsection below.

## Support for signalling vignetting parameters

The current draft of the LOC SEI message includes support for signalling of parameters for two distortion types: radial distortion and transverse chromatic aberration. A third type of lens related distortion is vignetting. Vignetting is an optical effect that reduces the brightness of an image further away from the image center. Similar to radial distortion and transverse chromatic aberration, it is sometimes desirable to keep vignetting in order to preserve a certain look of an image or video, or even add it artificially for artistic effect as in the example in figure 1. Other times, it is desirable to be able to correct for vignetting in order to present an image or video as close as possible to the “true scene” and be able to remove the impact of the specific lens used to capture the scene.

Figure 1. Original image (left), artificial vignetting added (right)

Lensfun [4] maintains a publicly available database of lenses and their distortion parameters. Three types of distortions are available in the database: radial distortion, transverse chromatic aberration, and vignetting. At the time of writing this contribution, the database includes entries for 1 363 different lenses and 44 % of the entries include vignetting parameters. We believe that including support for vignetting would make the LOC SEI message more complete and would increase the applicability of the LOC SEI message. The proposed changes are shown below:

| … |  |
| --- | --- |
| loc_tca_parameters_present_flag | u(1) |
| if( loc_tca_parameters_present_flag ) { |  |
| loc_tca_model_id | u(8) |
| if( loc_tca_model_id = = 0 ) { |  |
| loc_tca_kr | u(25) |
| loc_tca_kb | u(25) |
| } |  |
| if( loc_tca_model_id = = 1 ) { |  |
| loc_tca_br | i(25) |
| loc_tca_cr | i(25) |
| loc_tca_vr | u(25) |
| loc_tca_bb | i(25) |
| loc_tca_cb | i(25) |
| loc_tca_vb | u(25) |
| } |  |
| } |  |
| loc_vignetting_parameters_present_flag | u(1) |
| if( loc_vignetting_parameters_present_flag ) { |  |
| loc_vignetting_model_id | u(8) |
| if( loc_vignetting_model_id = = 0 ) { |  |
| loc_vignetting_k1 | i(25) |
| loc_vignetting_k2 | i(25) |
| loc_vignetting_k3 | i(25) |
| } |  |
| loc _persistence_flag | u(1) |
| } |  |
| } |  |

loc_vignetting_parameters_present_flag equal to 1 indicates that vignetting parameters are present in the lens optical correction SEI message syntax. loc_vignetting_parameters_present_flag equal to 0 indicates that vignetting parameters are not present in the lens optical correction SEI message syntax.

loc_vignetting_model_id identifies the type of vignetting model as specified in Table 1. The value of loc_vignetting_model_id shall be equal to 0. The values greater than 0 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_vignetting_model_id greater than 0.

Table 1 – loc_vignetting_model_id

| Value | Description |
| --- | --- |
| 0 | Poly6 |

When loc_vignetting_model_id equals to 0, the polynomial 6th order (Poly6) model is specified as

cd = cs * (1 + k1 * ru * ru + k2 * ru * ru* ru* ru + k3 * ru * ru* ru* ru* ru* ru)	(X-2)

with cd referring to the distorted colour, cs referring to the undistorted colour, ru referring to the radius, and k1, k2 and k3 being the vignetting distortion coefficients.

NOTE – The same vignetting parameters are used for red, green and blue.

loc_vignetting_k1 ÷224 indicates the value of the k1 parameter from the Poly6 vignetting model from equation (X-2).

loc_vignetting_k2 ÷224 indicates the value of the k2 parameter from the Poly6 vignetting model from equation (X-2).

loc_vignetting_k3 ÷224 indicates the value of the k3 parameter from the Poly6 vignetting model from equation (X-2).

## Add poly4 as a model for radial distortion

The first version of the proposal for the LOC SEI message [1] included three models for radial distortion: Poly3, Poly5 and Ptlens. However, in the updated version of the proposal, Ptlens was removed and only Poly3 and Poly5 were kept. It appears that the reason for the removal relates to the potential difficulty of using the name Ptlens (in which the PT stands for PanoramaTools). We believe that it would be unfortunate not to be able to express radial distortion using the Ptlens model, since it is a widely used model. For the entries in the Lensfun database [4], 86% use the Ptlens model (only 14% use Poly3 and less than 1% use Poly5). It should be noted that parameters expressed with the PTlens model cannot be directly converted to Poly3 or Poly5. Therefore, it is not possible to directly use parameters from the Ptlens-based entries in the Lensfun database with the current draft of the LOC SEI message.

Our proposed solution is to introduce a slightly generalised form of the Ptlens model and call it Poly4. The Poly4 format would use 4 parameters and be defined as follows:

rd = ru * (k1 + k2 * ru + k3 * ru * ru + k4 * ru * ru* ru)

while the Ptlens format is defined as follows:

rd = ru* (ar3u + br2u + cru + 1 − a − b − c)

Thus, a lens with radial distortion information expressed using the Ptlens parameters can be signalled with Poly4 parameters by setting k1 = 1 − a − b − c, k2 = c, k3 = b, k4 = a.

It is proposed to include a NOTE to help implementers realize that the Ptlens model is supported through the Poly4 model.

The following text changes are proposed:

| … |  |
| --- | --- |
| if( loc_rd_parameters_present_flag ) { |  |
| loc_rd_model_id | u(8) |
| if( loc_rd_model_id = = 0 ) { |  |
| loc_rd_poly3_k1 | i(25) |
| } |  |
| if( loc_rd_model_id = = 1 ) { |  |
| loc_rd_poly4_k1 | i(25) |
| loc_rd_poly4_k2 | i(25) |
| loc_rd_poly4_k3 | i(25) |
| loc_rd_poly4_k4 | i(25) |
| } |  |
| if( loc_rd_model_id = = 2 ) { |  |
| loc_rd_poly5_k1 | i(25) |
| loc_rd_poly5_k2 | i(25) |
| } |  |
| } |  |
| … |  |

loc_rd_model_id identifies the type of radial distortion model as specified in Table 1. The value of loc_rd_model_id shall be in the range of 0 to 12, inclusive. The values greater than 12 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_rd_model_id greater than 12.

Table 1 – loc_rd_model_id

| Value | Description |
| --- | --- |
| 0 | Poly3 |
| 1 | Poly4 |
| 12 | Poly5 |

When loc_rd_model_id equals to 0 the polynomial 3rd order (Poly3) model is specified as

rd = ru * ( 1 − k1 + k1 * ru * ru )		(X-1)

with rd referring to the distorted radius (normalized distance to image center), ru referring to the corrected undistorted radius, and k1 being the distortion coefficient.

When loc_rd_model_id equals to 1, the polynomial 4th order (Poly4) model is specified as

rd = ru * (k1 + k2 * ru + k3 * ru * ru + k4 * ru * ru* ru)		(X-2)

with rd referring to the distorted radius (normalized distance to image center), ru referring to the corrected undistorted radius, and k1, k2, k3 and k4 being the distortion coefficients.

NOTE – Lenses with radial distortion information expressed using a three parameter formula of the form the rd = ru* (ar3u + br2u + cru + 1 − a − b − c), can be signalled with Poly4 parameters by setting k1 = 1 − a − b − c,  k2 = c,  k3 = b,  k4 = a.

When loc_rd_model_id equals to 12 the polynomial 5th (Poly5) order model is specified as

rd = ru * ( 1 − k1 * ru * ru + k2 * ru * ru* ru* ru )		(X-23)

with rd referring to the distorted radius (normalized distance to image center), ru referring to the corrected undistorted radius, and k1 and k2 being the distortion coefficients.

loc_rd_poly4_k1 ÷224 indicates the value of the k1 parameter from the Poly4 radial distortion model from equation (X-2).

loc_rd_poly4_k2 ÷224 indicates the value of the k2 parameter from the Poly4 radial distortion model from equation (X-2).

loc_rd_poly4_k3 ÷224 indicates the value of the k3 parameter from the Poly4 radial distortion model from equation (X-2).

loc_rd_poly4_k4 ÷224 indicates the value of the k4 parameter from the Poly4 radial distortion model from equation (X-2).

## Move the persistence flag to be the first syntax element after the cancel flag

In alignment with other SEI messages and in order to have persistence information easily available in the LOC SEI message, it is proposed to move it to be the first syntax element after the loc_cancel_flag.

| lens_optical_correction( payloadSize ) { | Descriptor |
| --- | --- |
| loc_cancel_flag | u(1) |
| if( !loc_cancel_flag ) { |  |
| loc_persistence_flag | u(1) |
| loc_focal_parameters_present_flag | u(1) |
| … |  |
| loc_persistence_flag | u(1) |
| } |  |
| } |  |

## Add support for signalling a text string identifying the lens model

It is proposed to add a text string syntax element to allow for signalling the lens model. This allows for receivers to use their own databases and their own distortion models for known lenses as a complement or alternative to the parameters signalled in the LOC SEI message. An additional syntax element with reserved bits is proposed to be added before the string in order to ensure that the characters are at byte aligned positions.

The following is proposed:

| lens_optical_correction( payloadSize ) { | Descriptor |
| --- | --- |
| loc_cancel_flag | u(1) |
| if( !loc_cancel_flag ) { |  |
| loc_persistence_flag | u(1) |
| loc_lens_model_present_flag | u(1) |
| if( loc_lens_model_present_flag ) |  |
| loc_reserved_zero_5_bits | u(5) |
| loc_lens_model | st(v) |
| loc_focal_parameters_present_flag | u(1) |
| if( loc_focal_parameters_present_flag ) { |  |
| loc_focal_center_x | u(16) |
| loc_focal_center_y | u(16) |
| loc_focal_length | u(16) |
| } |  |
| … |  |

loc_lens_model_present_flag equal to 1 indicates that the loc_lens_model syntax element is present in the lens optical correction SEI message syntax. loc_lens_model_present_flag equal to 0 indicates that the loc_lens_model syntax element is not present in the lens optical correction SEI message syntax.

loc_reserved_zero_5_bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for dci_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall also allow values greater than 0 for loc_reserved_zero_5_bits to appear in the bitstream and shall ignore the value of loc_reserved_zero_5_bits.

loc_lens_model specifies a string describing the applicable lens model.

NOTE X – The lens model string may be used as a complement or as an alternative to other parameters signalled in the lens optical correction SEI message.

## Clarification on the order of application

It is proposed to add text clarifying in which order the corrections are intended to be applied:

8.42.2 Lens optical correction SEI message semantics

The Lens optical correction SEI message provides the decoder with a lens distortion model to enable image correction. When parameters for multiple distortion types are available in the lens optical correction SEI message, the parameters are expressed with intention for correction in the following sequential order: first de-vignetting (if present), then anti-TCA (if present), then reversing the radial distortion (if present).

## Description of when in the processing chain the corrections are applied

It is proposed to clarify that the parameters are expressed with respect to sample values in linear-domain RGB:

8.42.2 Lens optical correction SEI message semantics

The Lens optical correction SEI message provides the decoder with a lens distortion model to enable image correction. The parameters are expressed such that the corrections can be applied to samples in linear RGB format. Pictures represented with YUV sample data (such as Y’CbCr) can be converted to linear RGB in order to apply the lens corrections as intended.

# Referemces

- JVET-AH0204-v1, AHG9: Lens Optical Correction SEI message, G. Teniou, S. Wenger, https://jvet-experts.org/doc_end_user/documents/34_Rennes/wg11/JVET-AH0204-v1.zip

- JVET-AI2032, Technologies under consideration for future extensions of VSEI (version 5), S. McCarthy, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, H. Tan, Y.-K. Wang 
https://jvet-experts.org/doc_end_user/current_document.php?id=14606

- JVET-AI0158, [AHG9] Lens Optical Correction SEI message, G. Teniou, S. Wenger (Tencent) https://jvet-experts.org/doc_end_user/current_document.php?id=14417

- Lensfun, List of supported lenses and cameras,
https://lensfun.github.io/lenslist/2999/12/31/Lenslist-master/

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).