| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0171-v2 |
| --- | --- |

| Title: | Non-EE2: Weighted OBIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Médéric Blestel, Naima Zouidi | Email: | pandrivon@ofinno.com mblestel@ofinno.com |
| Source: | Ofinno |  |  |

_____________________________

# Abstract

This contribution proposes to weigh the occurrence value of neighboring blocks coded with DIMD, TIMD and SGPM in the construction of the histogram of occurrence for improving the performance of Occurrence-based Intra Coding (OBIC) method. Weights and location dependency states are inherited from prediction information used in the construction of the DIMD, TIMD and SGPM predictors. Additionally, A distance scaling factor is considered.

On top of ECM-14.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT} results are as follows:

- AI: -0.02%, 0.04%, 0.00%  {100.1%, 100.5%}

# Introduction

Occurrence-Based Intra Coding (OBIC), proposed in JVET-AH0076 [2], derives intra prediction modes (IPM) of the current block based on the occurrence (sample-wise) of the intra prediction modes used to code adjacent and non-adjacent blocks in the neighborhood of current block. Similarly to DIMD histogram of gradients (HoG), a histogram of occurrence (HoC) is built and up to five most represented angular modes are selected. For neighboring blocks possibly using more than one IPM to generate a predictor (e.g. TIMD, DIMD, SGPM…), all the corresponding IPM are used in the HoC construction. Selected modes are blended with Planar mode to generate the OBIC predictor.

# Proposed method

In OBIC design, the contribution in the HoC of each IPM of neighboring blocks using more than one IPM (TIMD, DIMD, SGPM) is the same. For instance, in case of a neighboring block of width w and height h coded with DIMD and 5 angular IPM; both its most predominant mode M1 (i.e. the main DIMD mode with the highest weight) and the least predominant mode M5 (i.e. the DIMD mode with the lowest weight) contribute equally to the OBIC HoC although their impact in the DIMD predictor is proportional to their relative amplitude in the DIMD HoG:

HoC(M1) += w. h
HoC(M5) += w. h

This contribution proposes that OBIC reuses the weights (ki) and the location dependency state of neighboring blocks using more than one IPM.

HoC(Mi) += ki . w. h

Additionally, each contributor to the HoC is weighted by a scaling factor depending on the distance between the current block and the considered neighboring block.

# Experimental results

The tables below summarize the coding efficiency, runtime impact, and peak memory consumption of proposed method using the JVET common test conditions [2].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  | -0.02% |  | 0.04% | -0.07% |  | 98.8% |  | 100.7% |  | 100.1% |  | 100.2% |  |
| Class A2 |  | -0.01% |  | 0.02% | -0.02% |  | 101.0% |  | 100.0% |  | 99.8% |  | 100.0% |  |
| Class B |  | -0.01% |  | -0.05% | 0.06% |  | 99.9% |  | 100.5% |  | 97.7% |  | 100.2% |  |
| Class C |  | -0.01% |  | 0.04% | -0.15% |  | 100.7% |  | 100.3% |  | 99.2% |  | 100.1% |  |
| Class E |  | -0.03% |  | 0.22% | 0.19% |  | 100.2% |  | 100.9% |  | 99.3% |  | 100.1% |  |
| Overall |  | -0.02% |  | 0.04% | 0.00% |  | 100.1% |  | 100.5% |  | 99.0% |  | 100.1% |  |
| Class D |  | -0.01% |  | 0.03% | -0.09% |  | 101.2% |  | 103.0% |  | 99.9% |  | 100.0% |  |

Table 1. Performance of the proposed method on top of ECM-14.0 (AI)

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 14 (ECM 14),” JVET-AI2025, July 2024.

- M. Abdoli, R. G. Youvalari, A. Tissier, “EE2-2.2: Occurence-based intra coding (OBIC)”, JVET-AH0076, April, 2024.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AI2016, July 2024.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).