| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0172 |
| --- | --- |

| Title: | EE2-2.16: Unified reference area of IBC and IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Wenbin Yin1, Lei Zhao1, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM, a BV candidate of IntraTMP is determined to be valid if the corresponding reference block is already reconstructed inside the current picture, and a BV candidate of IBC is determined to be valid if the corresponding reference block is in the IBC reference area.

In EE2-2.16, to unify the reference area of IBC and IntraTMP, the valid IBC reference area is extended to all the reconstructed samples in the picture.

On top of ECM-14.0, simulation results of EE2-2.16 are reported as below:

AI:

Overall: { 0.00%, 0.01%, 0.00%, 99.2%, 99.6%};

Class F: { -0.29%, -0.51%, -0.48%,    , 98.2%};

Class TGM: { -1.20%, -1.26%, -1.25%, 99.1%, 95.7%};

RA:

Overall: { 0.01%, 0.00%, 0.01%, 99.6%, 99.7%};

Class F: { -0.23%, -0.33%, -0.28%,    , 99.2%};

Class TGM: { -0.73%, -0.71%, -0.78%, 99.5%, 99.4%};

LB:

Overall: { -0.02%, 0.03%, -0.26%, 99.3%, 99.5%};

Class F: { -0.48%, -1.11%, -0.49%, 99.2%, 99.6%};

Class TGM: { -0.51%, -0.38%, -0.55%, 99.6%, 95.2%}.

# Introduction

In ECM, the reference area of IBC is extended to two CTU rows above when CTU size is 128 as shown in Figure 1. When CTU size is 256, the reference area of IBC is limited to one CTU row above as shown in Figure 2.

Figure 1. Reference area of IBC when CTU size is 128. The blue block denotes the current CTU; green blocks denote the reference area; and the white blocks denote invalid reference area.

Figure 2. Reference area of IBC when CTU size is 256.The blue block denotes the current CTU; green blocks denote the reference area; and the white blocks denote invalid reference area.

On the other hand, a BV candidate of IntraTMP is determined to be valid as long as the corresponding reference block is already reconstructed inside the current picture.

# Proposed methods

In EE2-2.16, to unify the reference area of IBC and IntraTMP, the valid IBC reference area is extended to all the reconstructed samples in the picture.

# Experimental Results

The proposed method has been implemented on top of ECM-14.0 [1]. Testing results of EE2-2.16 under AI/RA/LB configurations are summarized in Table 1. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of EE2-2.16.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.03% | -0.05% | 99.1% | 99.7% | 101.2% | 104.6% |
| Class A2 | 0.00% | -0.01% | -0.05% | 99.7% | 99.6% | 101.3% | 104.4% |
| Class B | 0.00% | -0.01% | 0.04% | 99.3% | 99.7% | 101.9% | 101.8% |
| Class C | 0.00% | 0.02% | 0.00% | 98.8% | 99.4% | 101.0% | 100.2% |
| Class E | 0.00% | 0.04% | 0.01% | 99.2% | 99.4% | 100.1% | 100.7% |
| Overall | 0.00% | 0.01% | 0.00% | 99.2% | 99.6% | 101.2% | 102.1% |
| Class D | 0.00% | 0.00% | 0.00% | 98.8% | 99.6% | 100.0% | 99.9% |
| Class F | -0.29% | -0.51% | -0.48% |  | 98.2% | 100.0% | 100.8% |
| Class TGM | -1.20% | -1.26% | -1.25% | 99.1% | 95.7% | 101.0% | 101.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.12% | -0.04% | 100.0% | 99.8% | 100.6% | 102.9% |
| Class A2 | 0.01% | -0.07% | 0.03% | 100.0% | 99.4% | 100.4% | 102.8% |
| Class B | 0.01% | 0.06% | -0.02% | 99.6% | 99.8% | 100.0% | 101.3% |
| Class C | 0.00% | 0.06% | 0.07% | 99.2% | 99.9% | 100.1% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.01% | 99.6% | 99.7% | 100.2% | 101.6% |
| Class D | 0.00% | 0.00% | 0.00% | 99.2% | 99.7% | 99.9% | 99.9% |
| Class F | -0.23% | -0.33% | -0.28% |  | 99.2% | 100.1% | 100.6% |
| Class TGM | -0.73% | -0.71% | -0.78% | 99.5% | 99.4% | 100.5% | 101.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.07% | -0.40% | 99.2% | 99.9% | 100.2% | 101.4% |
| Class C | 0.00% | -0.02% | 0.04% | 99.6% | 99.4% | 100.0% | 100.2% |
| Class E | -0.05% | 0.05% | -0.42% | 99.0% | 99.1% | 98.9% | 100.4% |
| Overall | -0.02% | 0.03% | -0.26% | 99.3% | 99.5% | 99.8% | 100.8% |
| Class D | 0.00% | 0.00% | 0.00% | 99.3% | 99.9% | 99.9% | 100.0% |
| Class F | -0.48% | -1.11% | -0.49% | 99.2% | 99.6% | 100.1% | 100.4% |
| Class TGM | -0.51% | -0.38% | -0.55% | 99.6% | 95.2% | 100.8% | 101.2% |

- Conclusions

In EE2-2.16, the reference areas of IBC and IntraTMP are unified. The proposed method can bring some coding gains with coding time reduction. It is recommended to adopt EE2-2.16 into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-14.0

[2] M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, Sapporo, JP, 12–19 July 2024.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).