| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0173-v2 |
| --- | --- |

| Title: | AHG12: ALF-CCCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pekka Astola Ikram Jumakulyyev Done Bugdayci Sansli Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In this proposal a new cross-component loop filter (ALF-CCCM) is proposed for improving the quality of reconstructed chroma samples. In particular, the proposed loop filter utilizes the CCCM method for obtaining the cross-component filter coefficients at the decoder-side. The cross-component filter coefficients are derived using the SAO/CC-SAO output samples and the cross-component prediction samples are generated using the ALF luma output samples.

The impact on coding efficiency and runtimes of the proposed method over ECM-14.0 are reportedly {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

AI {0.06%, -1.56%, -1.62%, 100.7%, 109.8%, 102.8%, 102.9%}

RA {0.04%, -1.98%, -2.01%, 100.6%, 103.8%, 100.3%, 103.9%}

LDB {0.02%, -4.33%, -4.13%, 100.4%, 105.6%, 101.4%, 103.0%}

LDP {0.02%, -4.76%, -4.15%, 100.5%, 107.2%, 101.9%, 103.1%}

# Introduction

In ECM-14.0, the sample adaptive offset (SAO) and the adaptive loop filter (ALF) are used for improving the quality of the reconstructed luma and chroma samples. The CC-SAO and the CC-ALF are further used to refine the chroma reconstruction samples using luma reconstruction samples. For ALF and CC-ALF the filter coefficients are signaled. In this contribution an additional cross-component filtering scheme is proposed where the cross-component filters are derived at the decoder-side.

# Proposed Method

The proposed method aims to improve the quality of the reconstructed chroma samples. A decoder-side diagram of the proposed method is illustrated in Figure 1.

In the proposed filtering scheme, each CTU is divided into non-overlapping blocks and for each block the cross-component filter coefficients are derived using the SAO/CC-SAO outputs. The output samples of luma ALF are used as input to the ALF-CCCM filtering. To obtain the final ALF-CCCM output samples, the cross-component prediction samples are blended with the SAO/CC-SAO chroma output samples. In the blending the weights are equal to 0.5 for both the SAO/CC-SAO chroma output samples and the cross-component prediction samples.

The encoder decides the best block size for each CTU using a rate-distortion optimization loop. There are eight possible blocks sizes 2x2, 3x3, 4x4, 8x8, 16x16, 32x32, 64x64, 128x128. For filter derivation the blocks are extended by one sample on each side. For example, with 2x2 blocks the filter is derived using 4x4 blocks. The block extension is clipped against CTU boundaries.

For each CTU, the encoder’s RDO decides the best cross-component model from eight possible models. The models are listed in Table 1, where the cardinal directions indicate co-located luma sample position in the chroma grid (north up, south down).  The nonlinear and bias terms are the same as in the 7-tap CCCM model. The CCCM solver is used for deriving the filter coefficients. The 6-tap downsampling filter (used in CCCM and CCLM) is used for mapping the co-located luma into the chroma grid.

For each CTU, the choice of the block size and the choice of the cross-component model are signaled using CABAC coded flags.

For intra-coding, a CTU may inherit both the block size and the model type from the above or left CTU. This choice is signaled using a single CABAC coded flag for each CTU when ALF-CCCM is present in the left and/or above CTU. If two CTU candidates available, another CABAC coded flag is signaled to indicate the choice.

For inter-coding, a picture may inherit the block sizes and model types for all CTUs from a reference picture. The reference picture is derived from the L0 and L1 lists. Only reference pictures with ALF-CCCM present in at least one CTU are considered. The reference picture with the smallest POC distance to the current POC is selected. If activated, the picture level inheritance will skip the CTU-level signaling completely for the current picture. This choice is signaled using a single CABAC coded flag if a reference picture with at least one CTU utilizing ALF-CCCM is present.

The existing operations of SAO, CC-SAO, ALF and CC-ALF of ECM-14.0 are kept unchanged with exception that ALF chroma operates on ALF-CCCM output.

| model 0 | center | north | south | west | east | nonlinear | bias |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| model 1 | center | bias |  |  |  |  |  |  |  |  |  |
| model 2 | center | nonlinear | bias |  |  |  |  |  |  |  |  |
| model 3 | center | north | south | west | east | bias |  |  |  |  |  |
| model 4 | center | west | east | nonlinear | bias |  |  |  |  |  |  |
| model 5 | center | north | south | nonlinear | bias |  |  |  |  |  |  |
| model 6 | center | north | south | west | east | north-west | north-east | south-west | south-east | nonlinear | bias |
| model 7 | center | north | south | west | east | north-west | north-east | south-west | south-east | bias |  |

Table 1 The eight cross-component models of ALF-CCCM.

Figure 1 Decoder-side diagram of ALF-CCCM. The CCCM filters are derived using SAO/CC-SAO outputs. The CCCM filtering uses the ALF luma output samples as its input and resulting samples are blended with the SAO/CC-SAO outputs before ALF chroma processing.

# Experimental results

The performance of the proposed method was studied in ECM-14.0 and the BD-rate performance over the CTC is given in the four tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.05 % | -1.30 % | -1.72 % | 101.1% | 111.7% | 102.5% | 104.2% |
| Class A2 | 0.03 % | -1.29 % | -1.49 % | 100.9% | 111.5% | 102.5% | 103.3% |
| Class B | 0.07 % | -1.95 % | -1.75 % | 100.8% | 107.7% | 101.4% | 102.5% |
| Class C | 0.06 % | -1.62 % | -1.85 % | 100.3% | 111.6% | 103.6% | 102.4% |
| Class E | 0.08 % | -1.34 % | -1.10 % | 100.7% | 107.3% | 104.7% | 102.6% |
| Overall | 0.06 % | -1.56 % | -1.62 % | 100.7% | 109.8% | 102.8% | 102.9% |
| Class D | 0.05 % | -0.86 % | -0.94 % | 100.6% | 121.2% | 101.4% | 102.7% |
| Class F | 0.09 % | -1.30 % | -1.55 % | 99.7% | 107.9% | 102.3% | 102.3% |
| Class TGM | 0.09 % | -0.34 % | -0.39 % | 100.0% | 104.4% | 103.4% | 102.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.07 % | -1.57 % | -2.27 % | 101.1% | 104.5% | 100.8% | 106.0% |
| Class A2 | 0.03 % | -1.64 % | -2.06 % | 101.0% | 103.2% | 100.4% | 104.2% |
| Class B | 0.02 % | -2.40 % | -1.80 % | 100.4% | 103.0% | 100.8% | 103.5% |
| Class C | 0.04 % | -2.04 % | -2.03 % | 100.2% | 104.6% | 99.4% | 102.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.04 % | -1.98 % | -2.01 % | 100.6% | 103.8% | 100.3% | 103.9% |
| Class D | 0.04 % | -1.44 % | -1.26 % | 100.6% | 106.1% | 101.1% | 102.4% |
| Class F | 0.12 % | -1.31 % | -1.27 % | 100.4% | 104.5% | 97.9% | 101.7% |
| Class TGM | 0.16 % | -0.65 % | -0.94 % | 100.3% | 105.3% | 100.6% | 102.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01 % | -5.50 % | -4.18 % | 100.7% | 107.8% | 101.1% | 103.1% |
| Class C | 0.03 % | -3.99 % | -4.77 % | 100.2% | 106.2% | 102.0% | 103.0% |
| Class E | 0.02 % | -2.80 % | -3.20 % | 100.2% | 101.3% | 100.9% | 102.8% |
| Overall | 0.02 % | -4.33 % | -4.13 % | 100.4% | 105.6% | 101.4% | 103.0% |
| Class D | 0.08 % | -3.63 % | -4.15 % | 101.0% | 105.7% | 101.9% | 102.8% |
| Class F | 0.05 % | -3.31 % | -3.13 % | 100.6% | 105.8% | 101.2% | 103.2% |
| Class TGM | -0.07 % | -1.21 % | -1.34 % | 99.9% | 102.4% | 99.3% | 103.9% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08 % | -6.12 % | -4.36 % | 100.2% | 110.0% | 101.2% | 103.3% |
| Class C | 0.02 % | -3.86 % | -5.23 % | 101.0% | 106.7% | 102.0% | 102.8% |
| Class E | 0.16 % | -3.70 % | -2.38 % | 100.2% | 103.4% | 103.2% | 103.0% |
| Overall | 0.02 % | -4.76 % | -4.15 % | 100.5% | 107.2% | 101.9% | 103.1% |
| Class D | 0.05 % | -5.28 % | -4.35 % | 100.4% | 105.9% | 103.0% | 102.9% |
| Class F | 0.23 % | -3.26 % | -2.89 % | 100.2% | 106.5% | 101.3% | 102.9% |
| Class TGM | 0.09 % | -1.01 % | -1.29 % | 100.3% | 107.3% | 99.3% | 105.7% |

# Conclusion

A CCCM-based loop filter is proposed to improve the quality of chroma samples. The coding efficiency and runtime impact of the proposed method is demonstrated over ECM-14.0. It is suggested to study the proposed method in the next round of EE2.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).