| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0174-v1 |
| --- | --- |

| Title: | Non-EE2: Laplacian information for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ikram Jumakulyyev Döne Bugdayci Sansli John Funnell Jani Lainema | Email: | ikram.jumakulyyev@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In this contribution, ALF online coefficients are extended by adding new source of information, Laplacian of post-SAO buffer for additional filter coefficients derivation.

The coding efficiency and runtimes of the proposed method over ECM-14.0 are reported as below {for Y, U, V, EncT, DecT}:

RA { -0.04%, -0.03%, -0.03%, 100.6%, 100.6%}

LDB { -0.06%, 0.09%, -0.09%, 101.1%, 100.8%}

# Introduction

In ECM 14.0, ALF coefficient filter taps for online training includes spatial taps (#0 - #9), fixed filter output-based taps (#10 - #29), reconstruction buffer before DBF (#30 - #32), residual based taps (#33 - #34), Gaussian smoothed before DBF reconstruction buffer (#35).

Figure 1 – Online-trained ALF filter shapes in ECM 14.0

# Proposed Method

The proposed method introduces a new diversity of information, Laplacian of post-SAO buffer in deriving online ALF filters. Laplacian buffer is obtained by applying Laplacian operator to post-SAO buffer. At block boundaries, Gaussian pre-smoothing is applied before computing Laplacian. For deriving discrete Laplacian values we employ 2D kernel [0,1,0; 1,-4,1; 0,1,0].

New filter shape, 5x5 cross shape (taps #36 - #40) is applied to Laplacian buffer. New online-trained filter shape is shown as in Figure 2.

Figure 2 – Proposed new online-trained ALF filter shape (green)

# Experimental results

The tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 14.0 using the JVET common test conditions.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.08% | -0.09% | 101.2% | 100.1% | 101.8% | 101.3% |
| Class A2 | -0.04% | -0.01% | -0.09% | 101.2% | 100.1% | 101.4% | 101.4% |
| Class B | -0.05% | -0.05% | -0.04% | 100.5% | 100.9% | 101.1% | 100.6% |
| Class C | -0.05% | 0.01% | 0.06% | 99.9% | 100.9% | 100.6% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.03% | -0.03% | 100.6% | 100.6% | 101.2% | 100.9% |
| Class D | 0.00% | 0.02% | 0.03% | 100.5% | 101.2% | 100.4% | 100.1% |
| Class F |  |  |  |  |  |  |  |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | 0.09% | -0.09% | 101.1% | 100.8% | 106.4% | 100.5% |
| Class C | -0.02% | 0.03% | -0.20% | 100.1% | 100.4% | 100.1% | 101.2% |
| Class E | -0.12% | -0.11% | -0.63% | 102.0% | 99.0% | 101.5% | 101.1% |
| Overall | -0.06% | 0.02% | -0.26% | 101.0% | 100.2% | 103.1% | 100.9% |
| Class D | 0.00% | -0.29% | 0.59% | 101.9% | 102.7% | 100.3% | 100.1% |
| Class F |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).