| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0175-v1 |
| --- | --- |

| Title: | EE2-4.1: NSPT kernels for non-regular intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Gleb Verba, Bappaditya Ray, Patrick Garus, Muhammed Coban, Vadim Seregin | Tel: Email: | gverba@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution introduces additional sets of NSPT kernels for TIMD, DIMD, EIP, MIP, SGPM, and IntraTMP. The method was implemented on top of ECM-14.0 and experimental results following CTC are reported as follows:

AI: -0.17% (Y) -0.09% (U)	-0.01% (V)	100.4% (EncT) 99.6% (DecT)

RA: -0.03% (Y) -0.10% (U) 0.03% (V) 99.5% (EncT) 98.1% (DecT)

# Introduction

In ECM-14.0, NSPT/LFNST kernel set is chosen based on the transform block size and intra mode. NSPT is used for block shapes 4x4, 4x8, 4x16, 4x32, 8x8, 8x16, 8x32 and corresponding transposed block shapes, while LFNST is used for the rest of the block shapes. NSPT/LFNST is used for the majority of the intra prediction tools such as regular intra prediction, DIMD, TIMD, SGPM, MIP, EIP, IntraTMP etc. It is also applied to inter CUs.

In 35th JVET meeting, an additional set of NSPT kernels was proposed in JVET-AI0255 and investigated in Test 4.1 of EE2.

# Test description

This contribution proposes to replace the sets of NSPT kernels with the three new sets:

- The first set is applied for conventional intra predicted blocks.

- The second set is applied for blocks using TIMD, DIMD, EIP, MIP or SGPM.

- The third set is applied for blocks using IntraTMP and inter CUs.

The usage of the additional set depends on the prediction mode and no additional signalling is needed.

The proposed kernels require 2.2 MB of additional storage.

# Simulation results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | 0.02% | 0.01% | 99.6% | 94.9% | 101.1% | 100.3% |
| Class A2 | -0.12% | -0.11% | -0.02% | 100.7% | 99.2% | 100.0% | 100.3% |
| Class B | -0.21% | -0.10% | -0.04% | 100.0% | 99.8% | 99.9% | 100.0% |
| Class C | -0.08% | -0.15% | -0.10% | 100.9% | 101.6% | 100.7% | 100.5% |
| Class E | -0.32% | -0.09% | 0.14% | 100.6% | 101.7% | 100.2% | 100.3% |
| Overall | -0.17% | -0.09% | -0.01% | 100.4% | 99.6% | 100.3% | 100.3% |
| Class D | -0.01% | -0.01% | 0.03% | 99.1% | 101.4% | 100.4% | 100.4% |
| Class F | -0.12% | -0.03% | -0.05% | 99.3% | 100.5% | 100.1% | 99.8% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.33% | -0.15% | 100.3% | 98.2% | 100.1% | 99.7% |
| Class A2 | 0.03% | -0.12% | 0.00% | 99.7% | 97.1% | 100.1% | 99.3% |
| Class B | -0.09% | -0.02% | 0.06% | 98.2% | 98.0% | 100.1% | 99.1% |
| Class C | 0.00% | -0.02% | 0.13% | 100.2% | 98.9% | 100.2% | 99.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.10% | 0.03% | 99.5% | 98.1% | 100.1% | 99.4% |
| Class D | -0.04% | -0.02% | -0.12% | 101.3% | 99.9% | 100.5% | 100.2% |
| Class F | 0.07% | -0.37% | 0.13% | 102.0% | 114.2% | 100.1% | 100.0% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.03% | -0.34% | 98.8% | 98.7% | 100.9% | 98.4% |
| Class C | -0.03% | 0.25% | 0.28% | 99.2% | 99.7% | 100.1% | 98.8% |
| Class E | -0.04% | 0.76% | -1.19% | 97.0% | 90.2% | 100.2% | 98.8% |
| Overall | -0.03% | 0.28% | -0.34% | 98.5% | 96.9% | 100.5% | 98.6% |
| Class D | 0.05% | 0.31% | 0.39% | 99.6% | 99.0% | 100.4% | 100.2% |
| Class F | -0.37% | -0.16% | 0.56% | 110.7% | 108.7% | 99.6% | 99.4% |

# References

[1] ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).