| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0178-v1 |
| --- | --- |

| Title: | AHG8: Lightweight post-processing algorithm for machine consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Binzhe Li Shurun Wang Jie Chen Yan Ye (Alibaba Group) Shiqi Wang (City University of Hong Kong) | Tel: Email: | libinzhe.lbz, shurun.wsr, jiechen.cj, yan.ye}@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba Group, City University of Hong Kong |  |  |

_____________________________

- Abstract

In this contribution, a lightweight learning-based video post-processing algorithm is proposed to process the reconstructed pictures to improve machine task performance. Compared with the post-processing network proposed in JVET-AG0212, the enhancement neural network proposed in this contribution is simplified by reducing the number of convolutional layers and channels. The proposed lightweight algorithm has a reported complexity of 5.55 kMAC/pixel and 70.6k parameters, which is reportedly on par with NNLF’s very low complexity operating point (VLOP). According to the experimental results, it is reported that the proposed algorithm achieves 25.15% (RA), 27.97% (LD), 35.20% (AI) averaged BD-rate savings for object detection on SFU dataset, and 33.83% (RA), 40.71% (LD), 37.47% (AI) averaged BD-rate savings for object tracking task on TVD dataset.

Description of lightweight post-processing algorithm

Based on the post-processing network proposed in JVET-AG0212 [1], to reduce the computational complexity, the neural network is simplified by reducing the base channel number of convolutional layers from 32 to 16 and the number of Base Blocks from 7 to 5.

Figure 1 Architecture of lightweight post-processing network.

The structure of lightweight post-processing algorithm is shown in Figure 1, including feature extraction and feature recovery modules. The reconstructed pictures (input to the post-processing network) and processed pictures (output of the post-processing network) are noted as Ir and Ip, respectively. In order to achieve a multi-granularity post-processing, the promising U-Net architecture with parallel mirrored skip connections and multi-resolution modules is selected. In feature extraction module, the reconstructed picture Ir is fed to feature down sampling branch. Feature down sampling branch includes 3 base blocks and the outputs are {F'd1,F'd2,F'd3}. In feature recovery module, {F'd1,F'd2,F'd3} are merged with channel-wise concatenation and the final output is the processed picture Ip. Specifically, a base block is composed of 4 convolution layers, a depth-separable convolution layer, the Gaussian error linear units (GELU), layer normalization, channel attention layer and shortcuts.

Experimental results

The proposed algorithm is tested under the VCM common test conditions (CTC) specified in JVET-AI2030 [2]. The overall performance for detection and tracking is shown in Table 1.

Table 1 Overview of BD-rate results under VCM CTC

| Task | Dataset | RA (Mono-mAP) | LD (Mono-mAP) | AI (Mono-mAP) |
| --- | --- | --- | --- | --- |
| Object Detection | SFU-HW (Average) | -25.15% | -27.97% | -35.20% |
| Object Tracking | TVD (Average) | -33.83% | -40.71% | -37.47% |

The per-class performance on SFU-HW dataset for object detection is shown in Table 2 and the per-sequence performance on TVD dataset for object tracking is given in Table 3. (Please note “/” in the tables indicates the performance cannot be calculated due to the nonmonotonic machine task performance). The results show that the proposed algorithm achieves significant performance improvement under CTC.

Table 2 Performance of post-processing algorithm on SFU-HW dataset under RA, LD and AI configurations.

| SFU-HW | Metric | BD-rate (%) |  | BD-mAP | DecT |
| --- | --- | --- | --- | --- | --- |
| Config | Class Name | Mono-mAP | mAP |  |  |
| RA | Class A | -19.33% | -18.86% | 3.02 | 4967.42% |
|  | Class B | -18.51% | / | 2.56 | 10183.17% |
|  | Class C | -29.27% | -28.87% | 4.19 | 3825.30% |
|  | Class D | -29.11% | -31.42% | 2.84 | 2901.31% |
|  | Average | -25.15% | / | 3.18 | 4844.90% |
| LD | Class A | -42.85% | -42.39% | 11.31 | 7226.30% |
|  | Class B | -11.74% | / | 3.27 | 13011.65% |
|  | Class C | -32.92% | -32.63% | 5.26 | 4721.44% |
|  | Class D | -35.53% | -35.98% | 4.27 | 3309.39% |
|  | Average | -27.97% | / | 4.81 | 5974.09% |
| AI | Class A | -37.86% | -37.23% | 11.51 | 4081.18% |
|  | Class B | -28.84% | / | 6.84 | 7067.82% |
|  | Class C | -46.99% | -48.01% | 9.93 | 2311.71% |
|  | Class D | -29.10% | -30.20% | 4.41 | 1515.78% |
|  | Average | -35.20% | / | 7.40 | 2991.31% |

Table 3 Performance of post-processing algorithm on TVD dataset under RA, LD and AI configurations.

| TVD | Metric | BD-rate (%) |  | BD-MOTA | DecT |
| --- | --- | --- | --- | --- | --- |
| Config | Sequence Name | Mono-MOTA | MOTA |  |  |
| RA | TVD-01-1 | -10.78% | / | 1.15 | 12953.64% |
|  | TVD-01-2 | -7.54% | -7.31% | 0.62 | 13077.80% |
|  | TVD-01-3 | -14.37% | / | 1.61 | 13008.71% |
|  | TVD-02-1 | -26.82% | -29.68% | 2.32 | 13880.33% |
|  | TVD-03-1 | -47.97% | -49.55% | 6.58 | 12703.50% |
|  | TVD-03-2 | -64.15% | / | 5.88 | 11149.52% |
|  | TVD-03-3 | -65.19% | -58.27% | 6.26 | 9542.47% |
|  | Average | -33.83% | / | 3.49 | 12247.78% |
| LD | TVD-01-1 | -20.47% | / | 2.11 | 15926.13% |
|  | TVD-01-2 | -43.71% | -43.88% | 3.54 | 16135.44% |
|  | TVD-01-3 | -23.49% | -18.65% | 2.55 | 15676.38% |
|  | TVD-02-1 | -0.39% | / | 1.45 | 17183.72% |
|  | TVD-03-1 | -41.41% | -44.28% | 7.45 | 15205.21% |
|  | TVD-03-2 | -80.18% | -81.41% | 7.51 | 12627.62% |
|  | TVD-03-3 | -75.34% | -74.79% | 7.13 | 10169.11% |
|  | Average | -40.71% | / | 4.54 | 14506.08% |
| AI | TVD-01-1 | -16.61% | / | 2.16 | 2555.25% |
|  | TVD-01-2 | -19.49% | -18.78% | 2.06 | 2793.03% |
|  | TVD-01-3 | -15.62% | -8.74% | 1.87 | 3437.32% |
|  | TVD-02-1 | -20.99% | -20.38% | 2.22 | 6402.18% |
|  | TVD-03-1 | -59.90% | -61.49% | 11.13 | 4823.68% |
|  | TVD-03-2 | -64.39% | -67.68% | 9.56 | 3487.77% |
|  | TVD-03-3 | -65.30% | -68.07% | 10.05 | 2956.89% |
|  | Average | -37.47% | / | 5.58 | 3598.46% |

The comparison of the lightweight post-processing algorithm proposed in this contribution and the post-processing algorithm proposed in JVET-AG0212, which had been included in the AHG8 software branch, is shown in Table 4, Table 5 and Table 6, in terms of the object detection, object tracking tasks and complexity respectively. For object detection task, the proposed lightweight model achieves practically the same performance as the much more complex model in JVET-AG0212. For object tracking, the proposed lightweight model achieves slightly lower performance than the more complex model in JVET-AG0212, but still significantly outperforms the VVC anchor. Table 6 shows that the proposed lightweight model significantly reduces the post-processing model complexity, bringing it in line with the model complexity of VLOP NNLF (JVET-AI0107 [3]).

Table 4 The comparison of post-processing algorithm proposed in this contribution and that proposed in JVET-AG0212 for object detection task.

|  | Object Detection Task on SFU-HW dataset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  | LD |  | AI |  |
| Algorithms | BD-rate | DecT | BD-rate | DecT | BD-rate | DecT |
| The algorithm proposed in this contribution | -25.15% | 4844.90% | -27.97% | 5974.09% | -35.20% | 2991.31% |
| JVET-AG0212 | / | 12263.84% | -25.19% | 15119.94% | -35.40% | 7517.12% |

Table 5 The comparison of post-processing proposed in this contribution and that proposed in JVET-AG0212 for object tracking task.

|  | Object Tracking Task on TVD dataset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  | LD |  | AI |  |
| Algorithms | BD-rate | DecT | BD-rate | DecT | BD-rate | DecT |
| The algorithm proposed in this contribution | -33.83% | 12247.78% | -40.71% | 14506.08% | -37.47% | 3598.46% |
| JVET-AG0212 | -34.60% | 30114.96% | -45.48% | 35990.42% | -42.86% | 8822.93% |

Table 6 The complexity comparison of post-processing proposed in this contribution, that proposed in JVET-AG0212 and VLOP in JVET-AI0107.

| Algorithms | kMAC/pixel | Parameter Number (k) |
| --- | --- | --- |
| The algorithm proposed in this contribution | 5.55 | 70.6 |
| JVET-AG0212 | 20.8 | 275 |
| VLOP in JVET-AI0107 | 5.16 | 60.7 |

Conclusion

In this contribution, a lightweight post-processing algorithm is proposed for the purpose of machine consumption. Compared with the post-processing algorithm proposed in JVET-AG0212, the computation is reduced from 20.8kMAC/pixel to 5.55kMac/pixel and the parameter umber is reduced from 275K to 70.6K, and has similar complex as the very low complexity operation point of NNLF. The proposed lightweight model gives significant performance gain over the VVC anchor under CTC. Thus, it is suggested to include the proposed algorithm into the next draft of technical report (TR) on optimization of encoders and receiving systems for machine analysis of coded video content, and to replace the model in AHG6 software with the proposed lightweight model.

References

- B. Li, S. Wang, J. Chen, Y. Ye, S. Wang, “AHG8: A post-processing algorithm for machine consumption,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AG0212, 33rd Meeting, Online, 17–26 January 2024 (available online at https://jvet-experts.org/).

- S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AI2031, 35th Meeting, Sapporo, JP, 12–19 July 2024 (available online at https://jvet-experts.org/).

- D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson), D. Rusanovskyy, Y. Li, M. Karczewicz, “EE1-1.5: VLOP using LOP3 architecture,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AI0107, 35th Meeting, Sapporo, JP, 12–19 July 2024 (available online at https://jvet-experts.org/)

Patent rights declaration(s)

Alibaba Group and City University of Hong Kong Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).