| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0179-v1 |
| --- | --- |

| Title: | EE2-1.1: Rectangular Partitioning Restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexandre Tissier, Mohsen Abdoli, Ramin G. Youvalari, Frank Plowman | Email: | alexandret@xiaomi.com |
| Source: | Xiaomi Technology |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-1.1 exploration experiment on the rectangular partitioning restrictions. This test proposes a modification of the partitioning process and more specifically its restrictions on the maximum BT and TT size parameters. The proposed method releases the constraint of maximum BT and TT sizes in I-Slices. However, on those new unconstrained sizes, a restriction based on the number of MTT split is proposed.

The results for two EE2 tests are presented in this contribution. Test 1.1a corresponds to the normative solution and Test 1.1b corresponds to the non-normative solution.

Compared to ECM-14.0, the average BD-rate gains and runtimes reported in this contribution are as follows:

EE2 Test 1.1a:

AI { -0.06%, -0.48%, -0.45%, XX%, XX%}, RA { -0.01%, -0.03%, 0.00%, XX%, XX%}

EE2 Test 1.1b:

AI { XX%, XX%, XX%, XX%, XX%}, RA { -0.01%, -0.03%, 0.00%, XX%, XX%}

# Introduction

The partitioning process performed in ECM-14.0 [1] supports the QT split plus the MTT partitioning structure which contains BT and TT that can be both applied horizontally and vertically. Those five possible splits lead to a certain complexity that is reduced with some restrictions. One of them is the maximum BT and TT sizes which defines the maximum blocks sizes where MTT is available. For I-Slices (in AI configuration) in luma, the maximum BT and TT sizes are set to 32. For I-Slices (in AI configuration) in chroma, the maximum BT and TT sizes are set to 64 and 32, respectively. Another restriction is the maximum MTT depth that does not allow MTT split when the current MTT depth of the block is equal to 3 for I-Slices in AI configuration.

# Proposed Method

In this contribution, it is proposed to relax the maximum BT and TT sizes constraints and enable them in a limited way for those newly relaxed blocks on I-Slices. In particular, this proposal has two aspects. Firstly, it allows MTT split in some blocks (referred to as MTT restricted blocks in the rest of this contribution), where anchor disallows MTT via max BT and TT size values. Second, this contribution allows “only one” MTT split on those newly enabled MTT restricted blocks.

Figure 1 shows the current ECM configuration in black with only QT split allowed for MTT restricted blocks and the configuration proposed by the contribution that adds the MTT splits to the list of split available for MTT restricted blocks.

Figure 1: Enabling MTT splits for MTT restricted blocks

To limit the complexity increase, only one MTT split is applied on the MTT restricted block, i.e. the maximal MTT depth for those blocks are set to 1. Figure 2 depicts this restriction which allows only one MTT split on MTT restricted blocks.

Figure 2: Leaf restriction on MTT for MTT restricted blocks

Two tests are proposed for this contribution:

Test 1.1a) Proposed method is applied normatively,

Test 1.1b) Proposed method is applied non-normatively.

# Simulation Results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM-14.0 [1] using the JVET common test conditions [2].

## Test 1.1a: Normative solution

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.20% | -0.77% | -1.09% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class A2 | -0.04% | -0.56% | -0.48% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class B | -0.06% | -0.65% | -0.46% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class C | 0.02% | -0.17% | -0.15% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class E | -0.05% | -0.23% | -0.17% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Overall | -0.06% | -0.48% | -0.45% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class D | 0.00% | 0.01% | -0.10% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class F | -0.09% | -0.23% | -0.11% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class TGM | 0.05% | 0.14% | 0.13% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class A2 | 0.00% | 0.00% | 0.00% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class B | 0.00% | 0.00% | 0.00% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class C | -0.02% | -0.11% | 0.02% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | 0.00% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class D | 0.02% | -0.19% | -0.15% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class F | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class TGM | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |

## Test 1.1b: Non-normative solution

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class B | -0.03% | -0.61% | -0.43% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class C | 0.04% | -0.16% | -0.14% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class E | 0.02% | -0.17% | -0.10% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class D | 0.02% | 0.03% | -0.08% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class F | -0.05% | -0.19% | -0.07% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class TGM | 0.07% | 0.17% | 0.16% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class A2 | 0.00% | 0.00% | 0.00% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class B | 0.00% | 0.00% | 0.00% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class C | -0.02% | -0.11% | 0.02% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | 0.01% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class D | 0.02% | -0.19% | -0.15% | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class F | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |
| Class TGM | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! | #NOMBRE! | #NOMBRE! |

# Conclusion

This contribution proposes to release the constraints on the maximum BT and TT size and to restrict the maximal MTT depth to 1 on those newly released blocks. The experimental results, in AI configuration, showed that EE2 test 1.1a provides 0.06% average luma BD-Rate gain and 0.47% average chroma BD-Rate gain. It is suggested to adopt EE2 test 1.1a in the next version of ECM.

# References

- ECM-14.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).