| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0180-v1 |
| --- | --- |

| Title: | EE2-2.8: Combination of EE2-2.5 and EE2-2.7a/c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli, Ramin G. Youvalari, Frank Plowman, Alexandre Tissier, Yao-Jen Chang, Vadim Seregin, Marta Karczewicz Zheming Fan, Takeshi Chujoh, Tomohiro Ikai | Email: | mabdoli@xiaomi.com yjchang@qti.qualcomm.com fan.zheming@sharp.co.jp |
| Source: | Xiaomi Technology, Qualcomm Incorporated, Sharp Corporation |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-2.8a and EE2-2.8b which are two combinations of tests EE2-2.5 and EE2-2.7.

In the test EE2-2.5, the TIMD-merge mode is proposed, where an algorithm first creates a merge list based on the neighboring TIMD blocks. This list is then sorted based on the template cost and one candidate is finally kept in the list. A CU-level flag then indicates whether a TIMD-merge candidate is used for block prediction.

In test EE2-2.7, different configurations of removing TIMD-MRL and TIMD-ISP are proposed. Current contribution looks at combining TIMD-merge with two of those configurations, namely EE2-2.7a and EE2-2.7c, where in both MRL and ISP are normatively disabled. However, test EE2-2.7a does not fill the RD list after removal of TIMD-MRL and TIMD-ISP, while EE2-2.7c fills them by next best modes.

Here is a summary of the Test EE2-2.8a (EE2-2.5 + EE2-2.7a) results (*):

AI { -0.04%, 0.04%, 0.00%, xxx.x%, xxx.x%}, RA { x.xx%, x.xx%, x.xx%, x.xx%, x.xx%}

And here is a summary of the Test EE2-2.8b (EE2-2.5 + EE2-2.7c) results (*):

AI { -0.11%, -0.01%, -0.01%, xxx.x%, xxx.x%}, RA { x.xx%, x.xx%, x.xx%, x.xx%, x.xx%}

(*) Some missing points copied from Anchor

# Introduction

## TIMD-merge mode (Test EE2-2.5)

The TIMD merge mode, originally proposed in JVET-AG0106 [2], inherits the TIMD modes from the previously TIMD coded blocks. For this, the adjacent and non-adjacent spatial neighboring blocks are scanned and if the scanned block is coded in TIMD or TIMD merge mode, then its TIMD information (prediction modes, fusion flag, fusion weights and wide-angle conditions of TIMD modes, MTS transform types) are added to a TIMD merge list. Up to five TIMD candidates are added into the list.

### Merge list construction

Figure 1 shows the non-adjacent spatial neighboring blocks that are used in TIMD merge list generation.

Fig 1. Adjacent and non-adjacent spatial neighboring candidates for TIMD merge

Table 1: Initial TIMD merge list

| Cand | Intra modes | Fusion weights |
| --- | --- | --- |
| 1 | (IPM1,1, IPM1,2, IPM1,3) | (W1,1, W1,2, W1,3) |
| 2 | (IPM2,1, IPM2,2, IPM2,3) | (W2,1, W2,2, W2,3) |
| . . . | . . . | . . . |
| 5 | (IPM5,1, IPM5,2, IPM5,3) | (W5,1, W5,2, W5,3) |

A list of five TIMD candidates is created from the neighborhood. After forming the TIMD merge list, the merge candidates are sorted based on the SAD cost over the template of the current block. For the cost calculation, the SAD cost of the primary IPM of each TIMD candidate is used. In the case of two blocks with the same primary mode, the one with the lowest index from the neighbour map comes first.

The template size for sorting of TIMD merge candidates is decided to be one line from the immediate neighbourhood of the block.

The best candidate from the list is selected to be potentially included in encoder RDO.

The TIMD merge mode is applied to only luma blocks. Moreover, the mode is disabled for 4x4 block sizes.

In the encoder side, the SAD template costs calculation may reuse the SAD costs of the MPM list reordering in order to reduce the encoder runtime.

### Bitstream Signalling

Usage of the mode is signaled with a CABAC coded CU level flag. The TIMD merge mode is used as a sub-mode of TIMD tool.

## Disabling TIMD related combinations (Test EE2-2.7)

### Test a

Test2.7a disables the combination of TIMD and ISP (TIMD-ISP) and the combination of TIMD and MRL (TIMD-MRL) in normative way. This change decreases the number of RDOs for ineffective combinations which reduces encoding time.

### Test c

Test2.7c builds upon Test2.7a to keep the number of RDOs by replacing the reduced RDOs with other intra modes to achieve gains without increasing encoding time.

# Results

Results of Test EE2-2.8a (EE2-2.5 + EE2-2.7a):

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.05% | -0.05% |  |  |  |  |
| Class A2 | -0.01% | -0.03% | 0.02% |  |  |  |  |
| Class B | -0.03% | -0.04% | 0.00% |  |  |  |  |
| Class C | -0.05% | 0.04% | -0.09% |  |  |  |  |
| Class E | -0.06% | 0.23% | 0.15% |  |  |  |  |
| Overall | -0.04% | 0.04% | 0.00% |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |
|  | Y | U | V |  | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |

Results of Test EE2-2.8b (EE2-2.5 + EE2-2.7c)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.02% | -0.04% |  |  |  |  |
| Class A2 | -0.04% | -0.09% | -0.05% |  |  |  |  |
| Class B | -0.08% | -0.04% | 0.06% |  |  |  |  |
| Class C | -0.15% | 0.02% | -0.10% |  |  |  |  |
| Class E | -0.19% | 0.07% | 0.05% |  |  |  |  |
| Overall | -0.11% | -0.01% | -0.01% |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |
|  | Y | U | V |  | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Xiaomi Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).