| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0181-v1 |
| --- | --- |

| Title: | AHG8: On combination of pre-processing, post-processing and ROI-based adaptive QP algorithm for machine vision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang Binzhe Li Jie Chen Yan Ye (Alibaba Group) Shiqi Wang (City University of Hong Kong) | Tel: Email: | {shurun.wsr, libinzhe.lbz, jiechen.cj, yan.ye}@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba Group, City University of Hong Kong |  |  |

_____________________________

- Abstract

This contribution reports the performance of jointly enabling pre-processing, post-processing and ROI-based adaptive QP algorithm for machine vision. The post-processing enabled in this contribution is the lightweight algorithm proposed in JVET-AJ0178. According to experimental results, it is reported that when pre-processing, post-processing and ROI-based adaptive QP algorithm are all enabled, the combined method achieves 33.21% (RA), 29.52% (LD) and 42.65% (AI) averaged BD-rate savings for object detection on SFU-HW dataset, and 61.86% (RA), 39.81% (LD) and 71.41% (AI) averaged BD-rate savings for object tracking on TVD dataset, which is higher than enabling pre- and post-processing only or ROI-based adaptive QP algorithm only for machine vision.

Experiment description

In the previous meetings, JVET-AB0275 [1] proposed the ROI-based adaptive QP algorithm (referred to as ROIAQP) for encoding optimization, and JVET-AH0157 [2] combined the pre-processing and post-processing algorithm (referred to as PRE-POST), in which the foreground and background [3] of the videos were separately pre-processed before encoding and the enhancement post-filtering [4] was applied on the reconstructed videos after decoding. Due to the interesting coding performance reported, these methods were included in the technical report (TR) [5] on optimization of encoders and receiving systems for machine analysis of coded video content, and the software implementation was made available in AHG8 software branch. In JVET-AI0196 [6], the experiments of combining all three algorithms were conducted and the results showed that jointly applying PRE-POST and ROIAQP could achieve coding performance improvement compared with PRE-POST or ROIAQP only. As JVET-AJXXXX proposed a lightweight network for post-processing in this meeting, the combination experiments were re-conducted in this contribution by replacing the post-processing with the proposed lightweight network.

The experiment flowchart is shown in Figure 1. The original video is processed with foreground and background pre-processing first. And then the pre-processed video is encoded with ROIAQP optimized VVC encoder. After decoding, the decoded video is further processed by the lightweight enhancement post-filter which is proposed in JVET-AJ0178 [7]. And finally, the filtered video is fed to the machine consumption model.

Figure 1 The experiment flowchart on combined pre- and post-processing and ROI-based adaptive QP

Experimental results

The experiment is conducted under the VCM CTC specified in JVET-AI2031[8]. The overall BD rate performance for detection and tracking is shown in Table 1. Overall, better coding gain can be achieved by combining pre- and post-processing and ROI-based adaptive QP algorithm, noted as JOINT, compared with individually enabling pre- and post-processing or ROI-based adaptive QP.

Table 1 Overview of the averaged BD-rate performance in terms of mono-mAP

| Task | Dataset | Methods | RA | LD | AI |
| --- | --- | --- | --- | --- | --- |
| Detection | SFU-HW | PRE-POST | -26.91% | -27.45% | -39.38% |
|  |  | ROIAQP | -11.63% | -3.52% | -11.36% |
|  |  | JOINT | -33.21% | -29.52% | -42.65% |
| Tracking | TVD | PRE-POST | -52.06% | -32.80% | -64.66% |
|  |  | ROIAQP | -28.83% | -1.75% | -31.95% |
|  |  | JOINT | -61.86% | -39.81% | -71.41% |

The per-class performance on SFU-HW dataset for object detection is shown in Table 2 and the per-sequence performance on TVD dataset for object tracking is given in Table 3. (Please note “/” in the tables indicates the performance cannot be calculated validly, due to the nonmonotonic machine task performance or the non-overlap between rate-accuracy curves). The results show that the combined pre- and post-processing and ROI-based adaptive QP achieves significant performance improvement under CTC.

Table 2 Performance of combining pre-processing, post-processing and ROI-based adaptive QP on SFU-HW dataset under RA, LD and AI configurations.

|  |  | End-to-End BD-rate (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Object Detection | Mono-mAP | mAP | End-to-End BD-mAP | EncT | DecT |
| SFU-HW | Class A | -22.41% | / | 3.40 | 331.22% | 5347.41% |
|  | Class B | -29.88% | / | 6.12 | 362.72% | 11004.01% |
|  | Class C | -36.69% | -37.10% | 5.40 | 448.76% | 4114.90% |
|  | Class D | -35.77% | / | 4.20 | 538.35% | 3289.60% |
|  | Average | -33.21% | / | 5.10 | 434.26% | 5304.48% |
| LD | Object Detection | Mono-mAP | mAP | End-to-End BD-mAP | EncT | DecT |
| SFU-HW | Class A | -41.54% | -42.12% | 12.12 | 205.49% | 7003.66% |
|  | Class B | -19.23% | / | 3.00 | 237.91% | 12695.40% |
|  | Class C | -34.19% | -33.48% | 5.50 | 310.15% | 4366.98% |
|  | Class D | -32.13% | / | 3.89 | 350.30% | 3011.30% |
|  | Average | -29.52% | / | 4.75 | 287.51% | 5609.14% |
| AI | Object Detection | Mono-mAP | mAP | End-to-End BD-mAP | EncT | DecT |
| SFU-HW | Class A | -49.96% | -48.27% | 16.76 | 198.36% | 3801.69% |
|  | Class B | -44.29% | / | 11.10 | 229.35% | 6698.19% |
|  | Class C | -51.38% | -52.03% | 11.27 | 238.26% | 2120.88% |
|  | Class D | -30.45% | -31.00% | 4.73 | 235.30% | 1401.84% |
|  | Average | -42.65% | / | 9.63 | 231.29% | 2782.01% |

Table 3 Performance of combining pre-processing, post-processing and ROI-based adaptive QP on TVD dataset under RA, LD and AI configurations.

|  |  | End-to-End BD-Rate [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Object Tracking | Mono-MOTA | MOTA | End-to-end BD-MOTA | EncT | DecT |
| TVD | TVD-01-1 | -65.52% | / | 11.06 | 183.51% | 21326.37% |
|  | TVD-01-2 | -67.08% | -67.63% | 7.73 | 200.26% | 17865.51% |
|  | TVD-01-3 | -43.78% | / | 4.92 | 221.27% | 14248.92% |
|  | TVD-02-1 | -28.47% | -28.62% | 2.56 | 499.11% | 13647.88% |
|  | TVD-03-1 | -58.01% | -58.08% | 9.63 | 371.30% | 12603.72% |
|  | TVD-03-2 | -83.82% | / | 11.31 | 218.45% | 10970.45% |
|  | TVD-03-3 | -86.32% | -86.65% | 12.30 | 111.81% | 9349.08% |
|  | Average | -61.86% |  | 8.50 | 232.58% | 13809.62% |
| LD | Object Tracking | Mono-MOTA | MOTA | End-to-end BD-MOTA | EncT | DecT |
| TVD | TVD-01-1 | -34.89% | / | 5.22 | 164.64% | 15241.40% |
|  | TVD-01-2 | -42.02% | -44.57% | 5.22 | 171.36% | 15689.45% |
|  | TVD-01-3 | -26.47% | / | 2.87 | 189.98% | 15146.59% |
|  | TVD-02-1 | 20.81% | / | -0.76 | 286.70% | 16530.36% |
|  | TVD-03-1 | -39.29% | -42.75% | 7.37 | 241.29% | 14511.77% |
|  | TVD-03-2 | -74.90% | / | 9.37 | 149.10% | 12282.36% |
|  | TVD-03-3 | -81.91% | -82.71% | 9.51 | 93.26% | 9963.73% |
|  | Average | -39.81% | / | 5.54 | 175.64% | 14017.32% |
| AI | Object Tracking | Mono-MOTA | MOTA | End-to-end BD-MOTA | EncT | DecT |
| TVD | TVD-01-1 | -77.86% | -77.90% | 19.69 | 30.03% | 2384.52% |
|  | TVD-01-2 | -81.13% | -81.28% | 16.91 | 29.78% | 2626.86% |
|  | TVD-01-3 | -59.77% | -59.89% | 8.79 | 57.85% | 3198.40% |
|  | TVD-02-1 | -39.11% | -42.12% | 2.80 | 197.23% | 6263.66% |
|  | TVD-03-1 | -72.10% | -72.69% | 17.46 | 119.62% | 4840.55% |
|  | TVD-03-2 | -81.95% | -82.78% | 16.13 | 70.82% | 3311.16% |
|  | TVD-03-3 | -87.94% | -88.93% | 19.10 | 44.62% | 2778.98% |
|  | Average | -71.41% | -72.23% | 14.41 | 62.81% | 3430.34% |

As the lightweight post-processing network newly proposed in JVET-AJXXX is used in the experiments, we also compare the coding performance and complexity with JVET-AI0196 in which the previously proposed non-lightweight post-processing network is used. The comparison results are given in Table 4 and Table 5 where the row noted as “this contribution” shows the results of the combined test with lightweight post-processing network proposed in JVET-AJ0178 and the row noted as “JVET-AI0196” show the results of the combined test with previously proposed non-lightweight network.

Table 4 Performance comparison of this contribution and JVET-AI0196 for object detection.

|  | Object Detection Task on SFU-HW dataset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  | LD |  | AI |  |
|  | BD-rate | DecT | BD-rate | DecT | BD-rate | DecT |
| This contribution | -32.21% | 5304.48% | -29.52% | 5609.14% | -42.64% | 2782.01% |
| JVET-AI0196 | -33.77% | 13999.20% | -32.47% | 15728.72% | -44.72% | 9000.37% |

Table 5 Performance comparison of this contribution and JVET-AI0196 for object tracking.

|  | Object Tracking Task on TVD dataset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  | LD |  | AI |  |
|  | BD-rate | DecT | BD-rate | DecT | BD-rate | DecT |
| This contribution | -61.86% | 13809.62% | -39.81% | 14017.32% | -71.41% | 3430.34% |
| JVET-AI0196 | -65.00% | 33200.96% | -47.60% | 36753.39% | -72.33% | 9168.88% |

Suggestion

In this contribution, the performance of combining pre-processing, post-processing and ROI-based adaptive QP algorithm is reported. According to the experimental results, with the complexity reduction of the post-processing, the combination complexity is also significantly reduced with slight coding performance drop. Compared with individually enabling pre-processing, post-processing or ROI-based adaptive QP algorithm, jointly enabling all of them can achieve much higher coding gain. Thus, it is suggested to include the combined pre-processing, post-processing and ROI-based adaptive QP algorithm implementation into AHG8 software branch and add the following highlighted sentence to section 5.1 of TR:

“It is noted that depending on specific use cases, the technologies outlined in this report can be implemented individually or in combination to optimize machine consumption performance within the constraints of the system capabilities. For example, a better performance can be achieved by applying a post-filtering being optimized towards object detection on the decoded video that was pre-processed with the method described in Annex A.2 and encoded with the ROI-based adaptive QP method described in Annex A.1. When employing multiple technologies simultaneously, it is important to consider that certain combinations can be impractical or infeasible due to inherent methodological constraints.”

References

- C. Hollmann, J. Ström, P. Wennersten, L. Litwic, R. Sjöberg, M. Damghanian, “Information about Ericsson’s response to the CfP for Video Coding for Machines,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AB0275, 28th Meeting, Mainz, DE, 20–28 October 2022 (available online at https://jvet-experts.org/)

- S. Wang, B. Li, J. Chen, Y. Ye, “AHG8: On combined pre-processing and post-processing for machine vision,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AH0157, 34th Meeting, Rennes, FR, 17–27 April 2024 (available online at https://jvet-experts.org/).

- B. Li, S. Wang, J. Chen, Y. Ye, S. Wang, “AHG8: A preprocessing software implementation for OFM,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AD0047, 30th Meeting, Antalya, TR, 21–28 April 2023 (available online at https://jvet-experts.org/).

- B. Li, S. Wang, J. Chen, Y. Ye, S. Wang, “AHG8: A post-processing algorithm for machine consumption,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AG0212, 33rd Meeting, Online, 17–26 January 2024 (available online at https://jvet-experts.org/).

- S. Liu, J. Chen, C. Hollmann, “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 6),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AI2030, 35th Meeting, Sapporo, JP, 12–19 July 2024 (available online at https://jvet-experts.org/).

- S. Wang, B. Li, J. Chen, Y. Ye, “AHG8: On combined pre- and post-processing and ROI-based adaptive QP for machine vision,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AI0196, 35th Meeting, Sapporo, JP, 12–19 July 2024 (available online at https://jvet-experts.org/).

- B. Li, S. Wang, J. Chen, Y. Ye, S. Wang, “AHG8: Lightweight post-processing algorithm for machine consumption,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AJ0178, 36th Meeting, Kemer, TR, 1–8 November 2024 (available online at https://jvet-experts.org/).

- S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AI2031, 35th Meeting, Sapporo, JP, 12–19 July 2024 (available online at https://jvet-experts.org/).

Patent rights declaration(s)

Alibaba Group and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).