| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0183-v1 |
| --- | --- |

| Title: | AHG9: Signaling source and added pictures in EOI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Lee Hendry Tan Chul Keun Kim Junghak Nam Jaehyun Lim Seung hwan Kim | Email: | jangw.lee@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the 35th JVET meeting, contribution JVET-AI0122 proposed to include signaling information of source and added pictures in EOI SEI message when temporal upsampling during / prior to encoding is applied. Source pictures and added pictures are pictures that are used as input for and pictures that are generated from the temporal upsampling process, respectively. It was asserted that such information allows decoder to recognize added pictures and source pictures which may be useful when one or more pictures needs to be removed from the bitstream, because it is naturally preferred to remove the added pictures rather than the source pictures. The contribution was discussed and it was decided that further study was needed.

This contribution is the result of further study on the contribution JVET-AI0122 and addresses comments and suggestions given during the review and discussion in the meeting. Based on the study, the following options for modification to address the asserted problem are proposed:

- Option 1. Sublayer based signaling approach

- Option 2. Use the existing syntax with additional constraint

- Option 3. Indication of whether the current picture is a source picture

# Introduction

In the 35th JVET meeting, contribution JVET-AI0122 proposed to include signaling information of source and added pictures in EOI SEI message when temporal upsampling during / prior to encoding is applied. Source pictures and added pictures are pictures that are used as input for and pictures that are generated from the temporal upsampling process, respectively. It was asserted that such information allows decoder to recognize added pictures and source pictures which may be useful when one or more pictures needs to be removed from the bitstream because it is naturally preferred to remove the added pictures rather than the source pictures. The contribution was discussed and it was decided that further study was needed.

The comments and suggestions given from the review of JVET-AI0122 is as follows:

- It was noted that the source picture timing info SEI message has temporal sub-layer based signalling indication of synthesized pictures and suggested that it would be desirable to have consistent signalling.

- It was noted that a sub-layer design is robust to dropped pictures. With the method in the contribution, if pictures were dropped, the signalled information would not be correct.

- It was suggested that if temporal sub-layers were used in the bitstream, a decoder could choose to drop temporal sub-layers without needing signalling indicating source picture location information.

- It was noted that dropping of frames would also lead to incorrect values for eoi_num_int_pics.

- It was suggested that the source picture timing info SEI may be a more appropriate place to carry this info.

- It was suggested that the existing syntax and semantics for eoi_num_int_pics may be sufficient to accomplish the proposed functionality.

We have conducted further study on the issue and taking into account the comments above. Based on our further study, we believe the following three approaches may be able to address the asserted problem.

# Sublayer based signaling approach

## Discussion

During the discussion at JVET-AI meeting, there were comments that using sublayer approach may be able to solve the asserted problem. Those comments are as follows:

- It was noted that a sub-layer design is robust to dropped pictures. With the method in the contribution, if pictures were dropped, the signalled information would not be correct.

- It was suggested that if temporal sub-layers were used in the bitstream, a decoder could choose to drop temporal sub-layers without needing signalling indicating source picture location information.

Temporal sublayer-based approach indeed can help in the problem about dropping / removing pictures. However, such solution is applicable only when temporal scalability is used.

Another relevant comment related to the use of temporal sublayer approach suggests to use source picture timing info SEI message that has signaling of synthesized pictures of SPTI SEI message as follow:

- It was suggested that the source picture timing info SEI may be a more appropriate place to carry this info.

- It was noted that the source picture timing info SEI message has temporal sub-layer based signalling indication of synthesized pictures and suggested that it would be desirable to have consistent signalling

In our understanding, this comment meant signaling information of synthesized picture (i.e., spti_sublayer_synthesized_picture_flag[ i ]) in SPTI SEI message may accomplish the proposed functionality in JVET-AI0122. We assert that spti_sublayer_synthesized_picture_flag[ i ] is not an appropriate signaling information to accomplish the desired functionality, since this syntax element is present only when spti_source_timing_equals_output_timing_flag is equal to 0. We also assert that it is not desirerable to set spti_source_timing_equals_output_timing_flag to 1, to make spti_sublayer_synthesized_picture_flag[ i ] present, since all other signaling information in SPTI messages is present uselessly, when the timing of source pictures is the same as the timing of corresponding decoded output pictures.

Considering the above suggested approaches, we think that the desired functionality can be accomplished by the simpler solution that includes signaling information of the maximum value of TemporalId which source pictures belong to. In this way, it is natural source pictures belong to sublayer(s) which has lesser TemporalId value than sublayer(s) that added pictures belong to.

## Proposed solution (Option 1)

Syntax:

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| if( eoi_temporal_resampling_type_flag && eoi_num_int_pics > 0 ) { |  |
| eoi_max_tid_src_pic_present_flag | u(1) |
| if( eoi_max_tid_src_pic_present_flag ) |  |
| eoi_max_tid_src_pic | u(3) |
| } |  |
| } |  |
| if(EoiSpatialResamplingFlag) |  |
| eoi_spatial_resampling_type_flag | u(1) |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

Semantics:

…

eoi_max_tid_src_pic_present_flag equal to 1 indicates the syntax element eoi_max_tid_src_pic is present in the SEI message. eoi_max_tid_src_pic_present_flag equal to 0 indicates the syntax element eoi_max_tid_src_pic is not present in the SEI message. When eoi_max_tid_src_pic_present_flag equal to 1, it is a requirement of bitstream conformance that both source pictures and added pictures shall not belong to the same temporal sublayer.

eoi_max_tid_src_pic , when present, specifies the maximum TemporalId value of the temporal sublayer to which original source pictures belong to.

…

# Use the existing syntax with additional constraint

## Discussion

It was also commented that it is possible and desirable to use the existing syntax and semantics to achieve the desired functionality. The comment is as follow:

- It was suggested that the existing syntax and semantics for eoi_num_int_pics may be sufficient to accomplish the proposed functionality.

In our assertion, eoi_num_int_pics only specifies the number of added picture(s) between source pictures, however, there’s no information in the current syntax and semantics that indicates or implies whether the current picture (i.e., the picture that is in the same access unit that contains the EOI SEI message) is a source picture or not. If it can be specified that when temporal upsampling optimization is included as one of optimization types of the EOI SEI message, the current picture is a source picture, then it would be easy for decoder to differentiate source pictures and added pictures using the existing syntax element eoi_num_int_pics.

## Proposed solution (Option 2)

Semantics:

…

eoi_num_int_pics greater than 0 indicates that the count of pictures that the encoding system excluded between each pair of coded pictures in output order (when eoi_temporal_resampling_type_flag is equal to 0) or added between each pair of source pictures for encoding (when eoi_temporal_resampling_type_flag is equal to 1) within the persistence of this SEI message is constant. When eoi_temporal_resampling_type_flag is equal to 0 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system excluded between each pair of coded pictures in output order. When eoi_temporal_resampling_type_flag is equal to 1 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system added between each pair of source pictures for encoding.

eoi_num_int_pics equal to 0 indicates that the count of pictures that the encoding system excluded between each pair of coded pictures in output order (when eoi_temporal_resampling_type_flag is equal to 0) or added between each pair of source pictures for encoding (when eoi_temporal_resampling_type_flag is equal to 1) within the persistence of this SEI message is unknown or varying.

The value of eoi_num_int_pics shall be in the range of 0 to 63, inclusive.

When EoiTemporalResamplingFlag is equal to 1, eoi_temporal_resampling_type_flag is equal to 1, eoi_temporal_resampling_type_flag is equal to 1 and eoi_num_int_pics is greater than 0, it is constrained that the access unit in which the EOI SEI message is present shall contain picture that is a source picture.

NOTE X – eoi_temporal_int_pics can be used to identify the pictures that are generated from the temporal upsampling optimization. The next (eoi_temporal_int_pics)-th pictures from the current picture (i.e., the picture associated with the EOI SEI message) are pictures generated from temporal upsampling optimization.

…

# Indication of whether the current picture is a source picture

## Discussion

The proposed constraint in section 3.2 (i.e., Option 2) may restrict persistency of encoder optimizations other than temporal resampling optimization. For example, if spatial quality optimization is applied only to added pictures not to source pictures, EOI SEI message cannot indicate this case, since the EOI message cannot be present in access unit of an added picture. Although, this will not an issue if proposal in JVET-AJ0119 is adopted.

One way to overcome the concern above is to introduce a new syntax element that indicates whether the current picture is a source picture, or this information (i.e., whether the current is a source picture or not) is unknown.

## Proposed solution (Option 3)

Syntax:

…

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| if( eoi_temporal_resampling_type_flag && eoi_num_int_pics > 0 ) |  |
| eoi_src_pic_flag | u(1) |
| } |  |
| if(EoiSpatialResamplingFlag) |  |
| eoi_spatial_resampling_type_flag | u(1) |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

Semantics:

…

eoi_src_pic_flag equal to 1 specifies that the picture in the same access unit that contains the EOI SEI message is a source picture. eoi_src_pic_flag equal to 0 provides no such indication.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).