| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0184-v1 |
| --- | --- |

| Title: | AHG9: On signaling of cancellation, persistency and id in TDI SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Lee Hendry Tan Junghak Nam Chul Keun Kim Jaehyun Lim Seung hwan Kim | Email: | jangw.lee@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes modifications that are asserted to provide improvement to the signaling and semantics of syntax elements related to cancellation, persistency, and id in TDI SEI message. The proposed modifications are as follows:

- Modify the value range of txt_descr_id.

- Modify the semantics of txt_persistence_flag.

- Simplify signaling of cancellation.

# Modify the value range of txt_descr_id

## Problem statement

The current semantics of txt_descr_id specifies that the value shall be in the range of 1 to 16383 (i.e. 214 – 1). We assert that the maximum value should be changed to 8 191 (i.e. 213 – 1), to fit 13 bits allocated to txt_descr_id in the current syntax.

## Proposed modification

We propose to modify the semantics of txt_descr_id to correct its value range as follows:

txt_descr_id indicates the identifier value of this text description information SEI message. The value of txt_descr_id shall be in the range of 1 to 163838 191, inclusive. Value 0 is reserved. Text description SEI messages with different values for txt_descr_purpose use separate values spaces for txt_descr_id.

# Modify the semantics of txt_persistence_flag

## Problem statement

In our assertion, txt_persistence_flag is supposed to specify persistency of a specific text description information with a particular txt_descr_purpose and txt_descr_id, but the current semantics appears to specify that txt_persistence_flag describes persistency of any TDI SEI message.

## Proposed modification

We propose to modify the semantics of txt_persistence_flag to clarity which txt_persistence_flag specifies, by adding the yellow highlighted text as follows:

txt_persistence_flag specifies the persistence of the text information description SEI message with the current txt_descr_id and txt_descr_purpose for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information SEI message with the current txt_descr_id and txt_descr_purpose applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message with the current txt_descr_id and txt_descr_purpose applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id and txt_descr_purpose is output that follows the current picture in output order.

# Simplify signaling of cancellation

## Problem statement

The current syntax includes two cancel flags, txt_cancel_flag and txt_id_cancel_flag, that specify the cancellation of TDI SEI message with the same txt_descr_purpose and TDI SEI message with the same txt_descr_purpose and txt_descr_id, respectively. Also, there are two if statements in the syntax, one for when txt_cancel_flag is equal to 0, and one for txt_id_cancel_flag is equal to 0, respectively.

It is asserted that those two cancel flags and two if statements unnecessarily complicate the signaling information, and the signaling information can be simplified with the proposal below.

## Proposed modifications

### Option 1: combine the two cancel flags into one indication, and combine the two if checking into one

We propose to modify the syntax that combines two cancel frags (i.e. txt_cancel_flag and txt_id_cancel_flag) into one indication (i.e. txt_cancel_idc), and also combines two if statements (i.e. txt_cancel_flag is equal to 0, and txt_id_cancel_flag is equal to 0) into one (i.e. when txt_cancel_idc is not equal to 0), as follow:

Syntax:

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_purpose | u(8) |
| txt_descr_id | u(13) |
| txt_cancel_flag | u(1) |
| txt_cancel_idc | u(2) |
| if( !txt_cancel_flag ) { |  |
| if( txt_cancel_idc = = 0 ) { |  |
| txt_descr_id | u(13) |
| txt_id_cancel_flag | u(1) |
| if( !txt_id_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics:

The text description information SEI message provides text description about one or more pictures.

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx. The value of text_descr_purpose shall be in the range of 0 to 5, inclusive. Values in the range of 6 to 255, inclusive, for text_descr_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of text_descr_purpose in the range of 0 to 255, inclusive.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US and Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6..255 | Reserved |

txt_descr_id indicates the identifier value of this text description information SEI message. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved. Text description SEI messages with different values for txt_descr_purpose use separate values spaces for txt_descr_id.

txt_cancel_idc equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_purpose in output order that applies to the current layer. txt_cancel_idc equal to 2 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_purpose and the same txt_descr_id in output order that applies to the current layer. txt_cancel_idc equal to 0 indicates that text description information follows. Value of 3, for txt_cancel_idc is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

txt_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_purpose in output order that applies to the current layer. txt_cancel_flag equal to 0 indicates that text description information follows.

txt_descr_id indicates the identifier value of this text description information SEI message. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved. Text description SEI messages with different values for txt_descr_purpose use separate values spaces for txt_descr_id.

txt_id_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id and txt_descr_purpose values as those in the current SEI in output order that applies to the current layer. txt_id_cancel_flag equal to 0 indicates that text description information syntax elements (txt_persistence_flag, txt_num_strings_minus1, txt_descr_string_lang[ i ] , txt_descr_string[ i ] ) follow.

txt_persistence_flag specifies the persistence of the text information description SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

…

### Option 2: combine two cancel flags into one indication

Alternatively, we propose to modify the syntax that combines two cancel frags (i.e. txt_cancel_flag and txt_id_cancel_flag) into one (i.e. txt_cancel_idc), and includes two if statements without modification, as follow:

Syntax:

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_purpose | u(8) |
| txt_cancel_flagidc | u(12) |
| if( !txt_cancel_flagidc ! = 1 ) { |  |
| txt_descr_id | u(13) |
| txt_id_cancel_flag | u(1) |
| if( !txt_id_cancel_flagtxt_cancel_idc = = 0 ) { |  |
| txt_persistence_flag | u(1) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics:

The text description information SEI message provides text description about one or more pictures.

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx. The value of text_descr_purpose shall be in the range of 0 to 5, inclusive. Values in the range of 6 to 255, inclusive, for text_descr_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of text_descr_purpose in the range of 0 to 255, inclusive.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US and Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6..255 | Reserved |

txt_cancel_idc equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_purpose in output order that applies to the current layer. txt_cancel_idc equal to 2 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_purpose and the same txt_descr_id in output order that applies to the current layer. txt_cancel_idc equal to 0 indicates that text description information follows. Value of 3, for txt_cancel_idc is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

txt_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_purpose in output order that applies to the current layer. txt_cancel_flag equal to 0 indicates that text description information follows.

txt_descr_id indicates the identifier value of this text description information SEI message. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved. Text description SEI messages with different values for txt_descr_purpose use separate values spaces for txt_descr_id.

txt_id_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id and txt_descr_purpose values as those in the current SEI in output order that applies to the current layer. txt_id_cancel_flag equal to 0 indicates that text description information syntax elements (txt_persistence_flag, txt_num_strings_minus1, txt_descr_string_lang[ i ] , txt_descr_string[ i ] ) follow.

txt_persistence_flag specifies the persistence of the text information description SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).