| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0185-v1 |
| --- | --- |

| Title: | AHG9: Editorial updates for NNPFC, SPO and TDI SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Lee Hendry Tan Junghak Nam Chul Keun Kim Jaehyun Lim Seung hwan Kim | Email: | jangw.lee@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes several editorial modifications for VSEI v4 draft. The proposed modifications are as follows:

(Related to NNPFC SEI message)

- Modify sentence in the semantics of nnpfc_num_metadata_extension_bits.

(Related to SPO SEI message)

- Modify the semantics of po_reserved_zero_4bits.

(Related to TDI SEI message)

- Modify wording the semantics of txt_persistence_flag.

- Change syntax element name from txt_persistence_flag to txt_id_persistence_flag.

- Change structure name from text_description() to text_description_info().

- Change prefix of syntax element name from txt_ to tdi_.

# Modify the semantics of nnpfc_num_metadata_extension_bits

## Problem statement

It is asserted that the cyan highlighted text in the semantics of nnpfc_num_metadata_extension_bits is written differently from the syntax, as below:

nnpfc_num_metadata_extension_bits equal to 0 specifies that nnpfc_reserved_metadata_extension is not present. When nnpfc_num_metadata_extension_bits is greater than 0, let the variable numSpecifiedMetadataExtensionBits be the number of bits representing all syntax elements between nnpfc_num_metadata_extension_bits and nnpfc_reserved_metadata_extension. nnpfc_num_metadata_extension_bits greater than 0 specifies the sum of numSpecifiedMetadataExtensionBits and the length, in bits, of nnpfc_reserved_metadata_extension.

The value of nnpfc_num_metadata_extension_bits shall be in the range of numSpecifiedMetadataExtensionBits to 2 048, inclusive. Values in the range of numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive, for nnpfc_num_metadata_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of nnpfc_num_metadata_extension_bits in the range of 0 to numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| nnpfc_metadata_alignment_zero_bit | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_for_human_viewing_idc | u(2) |
| nnpfc_for_machine_analysis_idc | u(2) |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

When nnpfc_num_metadata_extension_bits is equal to 0 not only nnpfc_reserved_metadata_extension is not present, there are several other syntax elements are not present as well. Furthermore, nnpfc_reserved_metadata_extension is already constrained to be not present in the bitstream confirming to this version of the specification, regardless the value of nnpfc_num_metadata_extension_bits is equal to 0 or greater than 0. Therefore, it is be good to update this semantics to remove the confusion. One possibility is to update it by also include the text about the presence of other syntax element within that if condition.

## Proposed modification

nnpfc_num_metadata_extension_bits equal to 0 specifies that nnpfc_application_purpose_tag_uri_present_flag, nnpfc_metadata_alignment_zero_bit, nnpfc_application_purpose_tag_uri, nnpfc_scan_type_idc, nnpfc_for_human_viewing_idc, nnpfc_for_machine_analysis_idc and nnpfc_reserved_metadata_extension isare not present. When nnpfc_num_metadata_extension_bits is greater than 0, let the variable numSpecifiedMetadataExtensionBits be the number of bits representing all syntax elements between nnpfc_num_metadata_extension_bits and nnpfc_reserved_metadata_extension. nnpfc_num_metadata_extension_bits greater than 0 specifies the sum of numSpecifiedMetadataExtensionBits and the length, in bits, of nnpfc_reserved_metadata_extension.

The value of nnpfc_num_metadata_extension_bits shall be in the range of numSpecifiedMetadataExtensionBits to 2 048, inclusive. Values in the range of numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive, for nnpfc_num_metadata_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of nnpfc_num_metadata_extension_bits in the range of 0 to numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive.

# Modify the semantics of po_reserved_zero_4bits

## Problem statement

The current semantics of po_reserved_zero_4bits is as follows:

po_reserved_zero_4bits shall be equal to 0. Values greater than 0 for po_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of po_reserved_zero_4bits in the range of 0 to 15, inclusive.

In our assertion, the cyan highlighted sentence is inconsistent with other semantics of reserved bits in VSEI  document, even thought the statement is not wrong. Most semantics of reserved bits in VSEI document is written as follows:

erp_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this edition of this document. Other values for erp_reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of erp_reserved_zero_2bits.

We assert that this inconsistent description may be confusing to readers as there are different ways for decoders to handle each reserved bit.

## Proposed modification

We propose to change the semantics of po_reserved_zero_4bits to be consistent with the existing semantics as follows:

po_reserved_zero_4bits shall be equal to 0 in bitstreams conforming to this edition of this document. Values greater than 0 for po_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of po_reserved_zero_4bits in the range of 0 to 15, inclusive. Decoders shall ignore the value of erp_reserved_zero_2bits.

# Modify wording the semantics of txt_persistence_flag

## Proposed modification

txt_persistence_flag specifies the persistence of the text information description information SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

# Change syntax element name from txt_persistence_flag to txt_id_persistence_flag

## Problem statement

In our understanding, txt_persistence_flag specifies the persistence of a specific TDI SEI message with the given identifier (i.e., txt_descr_id), not just any TDI SEI message, while a persistence flag of other SEI messages specifies (e.g., eoi_persitence_flag, spti_persistence_flag) the persistence of any of that SEI message. It is asserted that it is needed the differentiated name that fits the meaning of txt_persistence_flag different from a persistence flag of other SEI messages.

## Proposed modification

It is proposed to change syntax element name from txt_persistence_flag to txt_id_persistence_flag in the syntax and semantics of the TDI SEI message.

# Change structure name from text_description() to text_description_info()

## Problem statement

It is asserted that the structure name of text description information SEI message is not consistent with other SEI messages in VSEI v4 as follow:

- encoder optimization information SEI message : encoder_optimization_info()

- source picture timing information SEI message : source_picture_timing_info()

- object mask information SEI message : object_mask_info()

- modality information SEI message : modality_info()

## Proposed modification

It is proposed to change the structure name of TDI SEI message from text_description() to text_description_info() as follow:

| text_description_info( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_purpose | u(8) |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_descr_id | u(13) |
| txt_id_cancel_flag | u(1) |
| if( !txt_id_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Change prefix of syntax element name from txt_ to tdi_

It is asserted that the prefix (i.e., txt_) of text description information SEI message is not consistent with other SEI messages in VSEI v4 as follow:

- encoder optimization information SEI message : eoi_

- source picture timing information SEI message : spti_

- object mask information SEI message : omi_

- modality information SEI message : mi_

## Proposed modification

It is proposed to change the prefix of syntax element name from txt_ to tdi_ in the syntax and semantics of the TDI SEI message.

# Patent rights declaration(s)

LG Electronics Inc. does not have any current or pending patent rights relating to the technology described in this contribution.