| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0188-v0 |
| --- | --- |

| Title: | EE2-5.4: Coding Information based Classification for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Na Zhang1, Lei Zhao1, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This contribution reports the test results of EE2-5.4 which proposes the coding information based classification for Adaptive Loop Filter (ALF). There are 3 subtests (5.4a, 5.4b and 5.4c) that are included inside EE2-5.4. In EE2-5.4a, the partitioning information is introduced to associate the classification of online filters of ALF. In EE2-5.4b, the partitioning information and residual information are introduced into the classification of offline filters of ALF. The EE2-5.4c combines the first 2 subtests.

On top of ECM-14.0, simulation results of the proposed method are reported as below:

Test 5.4a:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

Test 5.4b:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

Test 5.4c:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

- Introduction

In the online filters for ALF in ECM-14.0, there are 3 classifiers known as texture-based, band-based, and residual-based classifiers. The coding information such as partitioning information is not utilized to generate the classification results of online filters. On the other hand, in the offline filters, there is only one texture-based classifier which uses the reconstruction samples before ALF to produce the classification results of offline filters. The coding information such as partitioning information and residual information are not considered to determine the filtering offset of offline filters.

- Proposed Method

In EE2-5.4, a method of coding information based classification for ALF is proposed, towards both online filters and offline filters.

For the online filters, each 2×2 unit is classified into 2 noise levels according to the partitioning information. For a 2×2 unit, it is classified into noise level-1 if it is located at a CU or TU boundary, and into noise level-0 if it is not. The class number of existing classifiers is reduced from 25 to 12. These 12 classes are further combined with the proposed 2 noise levels to generate final 12×2 = 24 classes in total. The number of classifiers for online filters is kept as 3 and no additional encoder selection is introduced.

For the offline filters, each 2×2 unit is classified into 2 noise levels in the same way as the classification of online filters. Besides, each 2×2 unit is further classified into 2 residual levels based on a predefined threshold also in the same way as the classifier of online filters. Furthermore, the generated offset of offline filters is adjusted based on the boundary level and residual level accordingly, where a stronger offset is applied on the positions at boundaries or with high residuals.

The proposed method in EE2-5.4 is always applied.

- Simulation Results

The proposed method was implemented on top of the ECM-14.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets.

Table 1 Performance of the EE2-5.4a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.01% | -0.01% |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  |  |
| Class F | 0.01% | -0.04% | -0.04% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.14% | -0.04% |  |  |  |  |
| Class C | -0.14% | -0.08% | 0.18% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.06% | 0.02% | 0.14% |  |  |  |  |
| Class F | 0.07% | 0.01% | 0.16% |  |  |  |  |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.22% | -0.01% | -0.11% |  |  |  |  |
| Class E | 0.12% | 0.56% | -0.39% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.07% | 1.37% | 0.29% |  |  |  |  |
| Class F | -0.26% | -0.75% | -0.41% |  |  |  |  |

Table 2 Performance of the EE2-5.4b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.00% | 0.00% |  |  |  |  |
| Class C | 0.01% | 0.00% | 0.00% |  |  |  |  |
| Class E | -0.01% | 0.01% | 0.01% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.04% | 0.04% |  |  |  |  |
| Class F | 0.02% | 0.00% | 0.00% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.08% | 0.00% |  |  |  |  |
| Class C | -0.08% | -0.01% | 0.28% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.12% | -0.06% | 0.08% |  |  |  |  |
| Class F | 0.00% | 0.00% | -0.02% |  |  |  |  |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.01% | 0.03% | -0.15% |  |  |  |  |
| Class E | 0.07% | -0.01% | -0.58% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.11% | 0.39% | 0.75% |  |  |  |  |
| Class F | -0.24% | -0.59% | 0.14% |  |  |  |  |

Table 3 Performance of the EE2-5.4c

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% |  |  |  |  |
| Class C | 0.01% | 0.00% | 0.00% |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.04% | 0.04% |  |  |  |  |
| Class F | 0.03% | -0.05% | -0.05% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.08% | 0.09% |  |  |  |  |
| Class C | -0.18% | -0.02% | 0.16% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.14% | -0.12% | 0.10% |  |  |  |  |
| Class F | -0.02% | 0.17% | 0.02% |  |  |  |  |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.20% | 0.38% | -0.31% |  |  |  |  |
| Class E | -0.17% | 0.60% | -0.42% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.13% | 0.00% | -0.41% |  |  |  |  |
| Class F | -0.06% | -0.72% | 0.03% |  |  |  |  |

- Conclusions

In this contribution, the test results of EE2-5.4 are reported. In the proposed method, the coding information is introduced into the classification process for both online filters and offline filters of ALF. The proposed method can achieve promising coding performance with negligible coding time changes. It is recommended to include the proposed method into the next version of ECM.

- Reference

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, JP, July 2024.

- Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).