| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0191-v1 |
| --- | --- |

| Title: | EE2-2.3/EE2-2.4: TIMD with enhanced block vectors deployment and BV merge list improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Miloš Radosavljević Ya Chen Fabrice Le Léannec Kevin Reuzé (InterDigital) Jiaye Fu Jiaqi Zhang Siwei Ma (PKU) Cheng Huang (ZTE) | Tel: Email: | karam.naser@InterDigital.com jiaye.fu@ucdconnect.ie |
| Source: | InterDigital, PKU, ZTE |  |  |

_____________________________

# Abstract

This contribution reports the results of tests EE2-2.3 and EE2-2.4. This proposal studies the use of block vectors (BVs) in TIMD mode with the extension of BV generation through the auto-relocated block vector prediction (ARBVP) process. It also enhances BV merge list generation for IntraTMP, DIMD, and SGPM mode. Clustering and refinement are used in EE2-2.4 test for TIMD and DIMD.

The following test results {Y, Cb, Cr, Enc time, Dec time, Enc mem, Dec mem} are reported on top of ECM-14 anchor for EE2-2.3:

AI: -0,06% , 0,01% , 0,00% , 100,4% , 101,4%, 101,0%, 100,9%

RA: -0,02% , -0,13% , 0,00% , 100,0% , 99,7%, 100,1%	100,2%

The following test results are reported on top of ECM-14 anchor for EE2-2.4:

AI: -0,06% , -0,04% , -0,02% , 103,0% , 111,9%, 101,4%, 101,0%

RA: #VALUE! , #VALUE! , #VALUE! , #NUM! , #NUM!

# Introduction

In addition to IBC and IntraTMP modes, the BV-based prediction in ECM is used in combination with directional prediction. Specifically, it is used with SGPM and with DIMD. In both modes, the merge candidates (adjacent and non-adjacent) are evaluated with the template cost to select the best block vector predictor to be combined with the directional modes.

It is asserted that TIMD can further benefit from the block vector prediction in the same manner as SGPM and DIMD modes. Specifically, any of the three fusion modes of TIMD can be replaced by a BV-based prediction from the merge candidates based on the template cost. Merge candidates list of IntraTMP can be expanded with the new process of auto-relocated block vector prediction. However, auto-relocated block vector prediction is not yet used for DIMD or TIMD.

Currently, merge BVs are collected from local and non-local PUs coded in IBC or IntraTMP mode. However, it may be beneficial to generate the BV merge list from all the spatial merge candidates that contain BVs (BV-based prediction is currently used in several modes in ECM software).

Also, it is noted that DIMD, TIMD, and SGPM may benefit from refinement process in the same way as IntraTMP.

# Proposed Method

EE2-2.3 test comprises the following:

- Allow TIMD with block vectors. When deriving the intra modes of TIMD (2 angular and 1 non-angular), the block vectors candidates are also tested. If the template cost is smaller than any of the other modes, that mode is replaced by the block vector.

- Use ARBVP candidates in TIMD. The additional block vector candidates generated by ARBVP process are considered for TIMD.

- Include IntraTMP-LIC best-by-cost candidate when constructing ARBVP candidates and merge candidates.

- Use adjacent candidates as in OBIC.

- Include BVs from DIMD, TIMD, and SGPM when constructing merge candidates.

- Include BVs from DIMD, TIMD, and SGPM when constructing ARBVP candidates.

- Include BVs from SGPM blocks in SGPM candidates.

- Refinement of ARBVP candidates in IntraTMP uses 5x5 window size while merge candidates use 11x11 window size.

In addition, following aspects are used to limit complexity:

- Up to 30 local and non-local merge candidates are used in SGPM and in DIMD.

- IntraTMP candidates are sorted based on the position relative to the IntraTMP search region and 30 candidates are kept. Candidates outside the IntraTMP search region are prioritized.

- In ARBVP process for TIMD, the number of ARBVP candidates added to the list is dynamically adjusted according the TIMD merge list size under construction.

- In ARBVP process for IntraTMP, the number of ARBVP candidates added to the list is dynamically adjusted according the IntraTMP merge list under construction.

- In IntraTMP, if overlap is detected, clustering is not done if merge candidate's TM cost is too high.

EE2-2.4 test adds additional elements on top of EE2-2.3 test. They include clustering and refinement of DIMD and TIMD BVs. A refinement window is defined around each BV from the merge list, and for all coinciding BVs, the same refinement window is used. In other words, multiple BVs are coinciding in one refinement area. All overlapping BVs are fused together, and a single refinement region is used.

# Experimental Results

## EE2-2.3

The method is implemented on top of ECM-14.0. The following results are obtained.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,03% | 0,12% | -0,04% | 100,1% | 101,4% | 98,9% | 102,3% |
| Class A2 | -0,08% | -0,14% | -0,03% | 100,4% | 101,5% | 101,3% | 102,5% |
| Class B | -0,06% | -0,04% | 0,01% | 100,5% | 101,3% | 101,7% | 100,2% |
| Class C | -0,05% | -0,11% | -0,09% | 100,4% | 101,4% | 100,4% | 100,1% |
| Class E | -0,11% | 0,28% | 0,17% | 100,5% | 101,4% | 102,3% | 100,2% |
| Overall | -0,06% | 0,01% | 0,00% | 100,4% | 101,4% | 101,0% | 100,9% |
| Class D | -0,03% | -0,10% | -0,09% | 100,5% | 101,0% | 100,0% | 100,0% |
| Class F | -0,19% | -0,30% | -0,24% | 100,3% | 101,9% | 100,3% | 100,1% |
| Class TGM | -0,40% | -0,43% | -0,40% | 100,4% | 101,8% | 100,6% | 100,4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,01% | -0,09% | 0,02% | 100,0% | 99,1% | 100,0% | 100,5% |
| Class A2 | -0,01% | -0,19% | -0,09% | 100,1% | 99,6% | 100,3% | 100,0% |
| Class B | -0,04% | -0,07% | -0,02% | 99,9% | 99,9% | 100,0% | 100,1% |
| Class C | -0,02% | -0,18% | 0,07% | 100,0% | 100,1% | 100,0% | 100,2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,02% | -0,13% | 0,00% | 100,0% | 99,7% | 100,1% | 100,2% |
| Class D | 0,01% | 0,10% | -0,06% | 100,0% | 100,5% | 100,1% | 100,0% |
| Class F | -0,08% | -0,07% | -0,01% | 99,9% | 100,3% | 100,0% | 100,0% |
| Class TGM | -0,25% | -0,14% | -0,19% | 100,0% | 101,1% | 99,5% | 99,8% |

## EE2-2.4

The method is implemented on top of ECM-14.0. It includes all aspects of EE2-2.3 with additional elements described in the previous section. The following results are obtained.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,03% | -0,08% | -0,07% | 101,7% | 106,1% | 99,0% | 102,1% |
| Class A2 | -0,09% | -0,11% | -0,05% | 103,1% | 112,1% | 101,5% | 103,4% |
| Class B | -0,06% | -0,05% | 0,07% | 103,3% | 112,2% | 102,2% | 100,0% |
| Class C | -0,04% | -0,03% | -0,05% | 103,1% | 112,7% | 101,7% | 100,1% |
| Class E | -0,10% | 0,09% | -0,04% | 103,9% | 116,3% | 101,6% | 100,1% |
| Overall | -0,06% | -0,04% | -0,02% | 103,0% | 111,9% | 101,4% | 101,0% |
| Class D | 0,00% | 0,23% | -0,11% | 102,9% | 114,4% | 99,8% | 100,1% |
| Class F | -0,19% | -0,23% | -0,23% | 103,8% | 116,0% | 98,7% | 100,1% |
| Class TGM | -0,41% | -0,43% | -0,43% | 104,8% | 109,9% | 101,8% | 100,5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0,05% | -0,10% | 0,17% | 100,5% | 100,6% | 100,4% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0,00% | -0,26% | -0,53% | 100,4% | 100,8% | 100,1% | 100,0% |
| Class F | -0,03% | -0,16% | -0,07% | 100,8% | 101,1% | 99,9% | 100,1% |
| Class TGM | -0,28% | -0,26% | -0,21% | 101,2% | 101,6% | 100,2% | 100,1% |

As the EE2-2.3 test gives the same gain as EE2-2.4 test with the smaller added complexity, we propose to adopt EE2-2.3 test.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).