| J oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0193 |
| --- | --- |

| Title: | AHG14: Update of Guidelines for NNVC software development |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin E. François | Tel: Email: | franck.galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution proposes to update the guidelines of the NNVC software development, to ensure a smoother integration of new adopted features. The update consists in updating the reference of the core guidelines used for VTM and ECM, as well as adding details related to the SIMD implementation.

It is proposed to update the guidelines of the NNVC software development. Several updates are proposed:

- Clarification of some guidelines

- Update of guidelines since the introduction of new tools

The following sections are taken from JVET-AI2007 [5] and changes are shown in color.

# Guidelines for NNVC software development

The use of the following rules is mandatory for all software implementation work. Changes that don’t comply with these rules will not be accepted by the software coordinators.

The software contains legacy code using different styles. When working on sections using old styles, that are different from the rules in this document, it is encouraged to update the related sections to the new style.

## Reference software guidelines

Except if specified otherwise, the guidelines for the C++ part of NNVC (contained in the source directory), should follow the same guidelines as the ones specified in the document [1].

## Specific software guidelines

### Dumper in NNVC

In order to train the model, one might need to dump data internal to the codec. A set of dumpable data, controllable by a set of macros, is already available in the file TypeDef.h (prefixed by NNVC_USE_xx).

The option parameter “DumpBasename” at the decoder then allows to dump the data.

The data are dumped into binary files for each data type to dump and a global description is dumped into a json file.

If a new data is added, it should follow the same policy as the one in place:

- Add a macro to specify the data to dump

- Add the data description in the json file

- Dump the data in a consolidated file per sequence

- Add the corresponding data loading process in the python data_loader module

Note: a set of script to create a consolidated dataset are available in the training/tools directory. An example of usage is given for the HOP model in the corresponding readme file.

### SADL

#### Overview

The inference of the models is done using the SADL library [2]. The SADL library is available as a submodule in the git repository of NNVC.

Currently SADL is a header only library. In order to avoid conflict of build options during the build of the NNVC software, it is advised to keep the SADL inclusion inside the compilation unit (not in a header file) and set the build option in the CMakeList.txt (see the NNInference.cpp file for an example).

#### New features

In case new features are needed to the SADL library, a merge request should be provided to the SADL repository ( https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl/-/merge_requests ).

When adding a new feature, the following components are required:

- Adapt the converter script to convert the ONNX feature into a SADL feature, carrying over the necessary parameters used by the layer.

- Add the feature in the SADL library, in order to be merged in the master branch, the new feature should have both float and integer implementation.

- Add relevant unit test to test the feature and add the tests to the testing script. Do not add features which are not tested/used. The unit test consists in implementing the feature in ONNX (which can itself be derived from a pytorch or tensorflow implementation for example), do the inference in ONNX and compare the result to the inference in SADL. The inference results should match up to some threshold defined from floating point accuracy.

- When proposing integer implementation (which is not necessarily available in all frameworks), integer implementation should be discussed before integration as part of a contribution. The integer implementation should be parametrizable by a quantization scale when relevant.

It is encouraged to propose a MR at the same time the EE code is released.

# Guidelines for training software development

## Software structure

The training software is placed in the training/training_scripts directory.

When adding a new script in training/training_scripts directory, the same copyright disclaimer reproduced in [1] should be added in the beginning of the file.

The python scripts should follow PEP8 convention [3]. The rules to follow are set in the. flake8 at the root of the repository. These rules are enforced during Merge Request. The software flake8 on your scripts allows to test the compliance before submitting a Merge Request.

The python software should use publicly available modules in order to ease the cross-check/repeatability of the training stage.

## Source modification

The data loading should be based on the provided data loader. The data loader allows data loading of an offline-created database to speedup the loading (see the example and tools directories). If a feature is missing, it should be added in the data loader module. Duplication of existing code is strongly discouraged, especially, missing features in the training scripts should be added in the common source, as opposed to model specific modification.

In order to ease the cross-checking, a single configuration file in json format should be provided, containing all the directory paths which need to be adapted to run on a particular environment should be provided. See the config.json file in the HOP model training directory for an example of such file.

For training, a guideline and a script or a set of scripts should be provided allowing the training of the model from scratch. The script should be as automatic as possible, after setting the correct parameters in the json file.

## Example of software structure

NN_Filtering_HOP

- config.json

- model/

  - model.py

  - model.json

- training/

  - trainer.py

  - cfg/training.json

- Readme.md

NN_Filtering_HOP

- config.json

- model/

  - model.py

  - model.json

- training/

  - trainer.py

  - cfg/training.json

- Readme.md

An example of software structure is given in the HOP model directory:

# Guidelines for models

## Models location

The models should be put in the models directory. The int16 version of the model is mandatory, while the float version is optional. The quantization parameters or method to convert a float model into an int16 model should be provided in the documentation of the training script.

The models or data files should be added to the git repository using the git LFS feature. Documentation can be found here [4]. By default, files with sadl, nnr, index and pb extension will be added as LFS files.

A parameter to control the model’s filename should be added to the codec to control the models to use. The default parameters should load the default models, assuming the runtime directory contains the models directory.

## Models architecture

### Layers design

In order to be able to perform inference on a wide range of AI accelerators, it is highly recommended to produce a SADL model which is fully compatible with the ONNX framework, and hence the tensorflow or pytorch inference for example. No custom layers or behavior should be performed on the SADL side since SADL is not aimed at being the final inference platform.

In particular the integer implementation of layer should be derived in a straightforward way from the float operation when possible. Badly defined operations such as intrinsic functions should be avoided. Complex operations such as intrinsic functions should be avoided when possible.

Custom operations not compatible with current AI framework should be done outside SADL for compatibility reasons.

Use of SIMD intrinsics is acceptable where it provides significant run time reduction. There shall be a plain (i.e., not processor-specific) fallback implementation for every function. This scalar implementation should be implemented in a straightforward manner as it is used for reference for more advanced implementation. Use of provided accessors to data structure (e.g. tensor) is encouraged since it is used for boundaries check in testing mode (pointer arithmetic should therefore be avoided). When multiple implementations (e.g., using different SIMD extensions) of a function exist, their behaviour shall be identical, i.e., they should produce identical results.

### Dynamic models

Models containing scripts (for example python scripts, torch scripting etc.) are discouraged since no AI accelerators would be able to run the inference and memory back and forth with the CPU would be needed. In framework running inference in eager mode, special care should be given to the code in order to control where the inference is performed (on scripts side or framework side). A sanity check is to save the model in ONNX and perform the inference in onnx without any scripting.

When parts of the model are conditional, it is recommended to either use multiple models, when possible, or to use frameworks condition layers (where, compare, equal etc.).

When part of the model is dynamic on the input sizes, a call to the init function in SADL before the inference is needed. If the input sizes have also an impact on layers parameters, either use an additional input to set the parameters or use the appropriate dynamic input option to save the ONNX model and use the shape node to propagate the sizes.

# Appendix

We present some examples of issues arising with parameters making the model unnecessarily dynamic.

## Dynamic model – Constant folding issue

h = 3

w = 4

d = 5

s = (h, w, d)

conv01 = nn.Conv2d(d, d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

…

def forward(self, inputs):

x, = inputs

y = conv01(x)

y = y.reshape((1,h*w*d))

y = y.reshape(s)

return y

model = Model()

…

torch.onnx.export(model, inputs_torch, "./test1.onnx", opset_version=10)

h = 3

w = 4

d = 5

s = (h, w, d)

conv01 = nn.Conv2d(d, d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

…

def forward(self, inputs):

x, = inputs

y = conv01(x)

y = y.reshape((1,h*w*d))

y = y.reshape(s)

return y

model = Model()

…

torch.onnx.export(model, inputs_torch, "./test1.onnx", opset_version=10)

Folded constants

The following python code is given:

Which results in the above graph.

We notice that the reshape size is folded to a constant, which makes the model use only a fixed size inputs, whereas in the python side, the variable h, w and d would allow to the resizing of the inputs. The model is not exportable.

### Parametrized layer

h = 3

w = 4

d = 5

s = (h, w, d)

conv01 = nn.Conv2d(d, d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

…

def forward(self, inputs):

x, = inputs

y = conv01(x)

y = y.reshape((1,-1))

y = y.reshape(s)

return y

model = Model()

…

torch.onnx.export(model, inputs_torch, "./test1.onnx", opset_version=10)

h = 3

w = 4

d = 5

s = (h, w, d)

conv01 = nn.Conv2d(d, d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

…

def forward(self, inputs):

x, = inputs

y = conv01(x)

y = y.reshape((1,-1))

y = y.reshape(s)

return y

model = Model()

…

torch.onnx.export(model, inputs_torch, "./test1.onnx", opset_version=10)

A solution to solve the above issue is to use a feature of the reshape layer where the size is deduced from input:

Which results in the above graph. Unfortunately, the second reshape layer is still using folded constants because the shape is on python side. The model is not exportable.

Dynamic inputs

h = 3

w = 4

d = 5

conv01 = nn.Conv2d(d, d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

…

def forward(self, inputs):

x, = inputs

y = conv01(x)

sh = y.shape

y = y.reshape((1,-1))

y = y.reshape(sh)

return y

model = Model()

…

torch.onnx.export(model, inputs_torch, "./test1.onnx",

opset_version=10, input_names=["in"], dynamic_axes={ 'in': { 2: 'h' ,3: 'w'}} )

h = 3

w = 4

d = 5

conv01 = nn.Conv2d(d, d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

…

def forward(self, inputs):

x, = inputs

y = conv01(x)

sh = y.shape

y = y.reshape((1,-1))

y = y.reshape(sh)

return y

model = Model()

…

torch.onnx.export(model, inputs_torch, "./test1.onnx",

opset_version=10, input_names=["in"], dynamic_axes={ 'in': { 2: 'h' ,3: 'w'}} )

Note that for some versions of ONNX, the dynamic axes option is needed to inform the exporter to avoid folding some variables.

The above graph is then produced for some version of ONNX, where the “shape” node is used to make the graph adapted to inputs size change.

Depending on the ONNX version and pytorch version, the graph can differ. For example, with opset 10, we found the following exported graph.

It makes the graph unnecessarily complex. In practice, the above pattern is caught by the converter to be simplified back to the previous version.

h = 3

w = 4

d = 5

s = (h,w,d)

conv01 = nn.Conv2d(d,d, kernel_size=(3, 3), padding=1, bias=True)

conv02 = nn.Conv2d(d,d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

...

def forward(self, inputs):

x, = inputs

if h>3:

y = self.conv01(x)

else:

y = self.conv02(x)

return y

torch.onnx.export(model, inputs_torch, "./test4.onnx", opset_version=13,

input_names=["in"], dynamic_axes={ 'in': { 2: 'h' ,3: 'w'}} )

h = 3

w = 4

d = 5

s = (h,w,d)

conv01 = nn.Conv2d(d,d, kernel_size=(3, 3), padding=1, bias=True)

conv02 = nn.Conv2d(d,d, kernel_size=(3, 3), padding=1, bias=True)

class Model(nn.Module):

...

def forward(self, inputs):

x, = inputs

if h>3:

y = self.conv01(x)

else:

y = self.conv02(x)

return y

torch.onnx.export(model, inputs_torch, "./test4.onnx", opset_version=13,

input_names=["in"], dynamic_axes={ 'in': { 2: 'h' ,3: 'w'}} )

Dynamic model – non exportable pattern

In this example, we can see even with dynamic inputs, the graph is folded to one variant only. The model is not exportable.

Several workarounds are possible:

- Prepare 2 models, 1 for each case.

- Create a conditional graph where both branches are executed, and one is selected using conditional layer.

# References

[1] JVET-AJ0187, “AHG3: Guidelines for VVC reference software development”, F. Bossen, X. Li, K. Sühring

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl

[3] https://peps.python.org/pep-0008/

[4] https://git-lfs.com/

[5] JVET-AI2007, “Guidelines for NNVC software development”, F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li